3GPP TSG-SA2 Meeting #166	S2-2411532

# 18 - 22 November, 2024, Orlando, USA                        rev of S2-2411179 S2-2410860 S2-2410274

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5748 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HSBO_roaming traffic offloading via session breakout in HPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, SK Telecom, [China Telecom] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_HSBO |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR addresses WT1 in WID (SP-240491) for TEI19_HSBO, “Roaming traffic offloading via session breakout in HPLMN”. It is proposed how to select a proper H-UPF deployed in the visited region of the serving PLMN for HR PDU session. It can prevent some target traffic, which can be served at the local server in the visited region, from remotely traversing to the H-UPF in the home region. To address this, it is proposed that Serving Network ID can be considered for UPF selection when the PDU Session is Home Routed and the H-SMF is aware of such specific deployment H-UPF, i.e., H-UPF is available at the visited region for a given VPLMN. Additionally, this CR also provides a proposal for WT2 SP-240491 on how to handle the user plane path failure between V-UPF and H-UPF of the HR session when the HR session is established over H-UPF deployed in the visited region for the purpose of roaming traffic offloading via session breakout in HPLMN. When the user plane path failure occurs for the paths between V-UPF and H-UPF of the HR session, it is required to switch the user plane path to the conventional home routed path (i.e., via H-UPF located in the home region) rather than waiting the ordinary restoration procedure for the H-UPF in the visited region. In Rev 4, further detailed operation for PDU Session release and re-establishment is clarified. Additionially, it is proposed how to provisioning DNS server information available at VPLMN. For successful roaming traffic offloading, the use of DNS server nearby VPLMN region, is needed to return the address of application servers nearby VPLMN region instead of servers in HPLMN region. However, in the HSBO deployment scenario, H-SMF and H-UPF are physically located in different places and would have different network subnet. This implies that it is difficult for H-SMF to manage DNS servers nearby VPLMN region. Furthermore, H-SMF would not be aware of IP topology of the visted region. To address this, it is proposed that H-UPF deployed nearby VPLMN region is configured with DNS server information and able to provision it to H-SMF via N4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | H-UPF selection considering serving network ID Change of the H-UPF when UP path failure between H-UPF and V-UPF occurs Provisioning DNS server information available at VPLMN region H-SMF initiated PDU Session Release and re-establishment operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Roaming traffic offloading at H-UPF deployed in the visited region is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4, 6.3.3.3, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/////// First Change //////

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

<snip>

[192]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[193]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[194]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[195]	3GPP TS 38.414: "NG-RAN; NG data transport".

[196]	3GPP TS 38.799: "Study on Additional Topological Enhancements for NR".

[X] 	3GPP TS 23.527: "5G System; Restoration Procedures".

/////// Next Change //////

4.2.4	Roaming reference architectures

Figure 4.2.4-1 depicts the 5G System roaming architecture with local breakout with service-based interfaces within the Control Plane.

Figure 4.2.4-1: Roaming 5G System architecture- local breakout scenario in service-based interface representation

NOTE 1:	In the LBO architecture. the PCF in the VPLMN may interact with the AF in order to generate PCC Rules for services delivered via the VPLMN, the PCF in the VPLMN uses locally configured policies according to the roaming agreement with the HPLMN operator as input for PCC Rule generation, the PCF in VPLMN has no access to subscriber policy information from the HPLMN.

NOTE 2:	An SCP can be used for indirect communication between NFs and NF services within the VPLMN, within the HPLMN, or in within both VPLMN and HPLMN. For simplicity, the SCP is not shown in the roaming architecture.

NOTE 3:	For clarity, the NWDAF(s) with roaming exchange capability (RE-NWDAF) and their connections with other NFs, are not depicted in the service-based architecture diagram. For more information on network data analytics architecture refer to TS 23.288 [86].

NOTE 4:	Depending on the architecture deployed, the Primary or Centralized NSACF at the VPLMN can fetch the maximum number of registered UEs or the maximum number of LBO PDU sessions to be enforced from the HPLMN Primary or Centralized NSACF as described in clause 5.15.11.3.1.

Figure 4.2.4-2: Void

Figure 4.2.4-3 depicts the 5G System roaming architecture in the case of home routed scenario with service-based interfaces within the Control Plane.

Figure 4.2.4-3: Roaming 5G System architecture - home routed scenario in service-based interface representation

NOTE 4:	An SCP can be used for indirect communication between NFs and NF services within the VPLMN, within the HPLMN, or in within both VPLMN and HPLMN. For simplicity, the SCP is not shown in the roaming architecture.

NOTE 5:	UPFs in the home routed scenario can be used also to support the IPUPS functionality (see clause 5.8.2.14).

NOTE 6:	For clarity, the NWDAF(s) with roaming exchange capability (RE-NWDAF) and their connections with other NFs, are not depicted in the service-based architecture diagram. For more information on network data analytics architecture refer to TS 23.288 [86].

NOTE X: In the home routed scenario, the H-UPF operated by HPLMN operator can be deployed nearby VPLMN. (see Annex X).

Figure 4.2.4-4 depicts 5G System roaming architecture in the case of local break out scenario using the reference point representation.

Figure 4.2.4-4: Roaming 5G System architecture - local breakout scenario in reference point representation

NOTE 7:	The NRF is not depicted in reference point architecture figures. Refer to Figure 4.2.4-7 for details on NRF and NF interfaces.

NOTE 8:	For the sake of clarity, SEPPs are not depicted in the roaming reference point architecture figures.

NOTE 9:	For clarity, the NWDAF(s) with roaming exchange capability (RE-NWDAF) and their connections with other NFs, are not depicted in the reference point architecture figure. For more information on network data analytics architecture refer to TS 23.288 [86].

The following figure 4.2.4-6 depicts the 5G System roaming architecture in the case of home routed scenario using the reference point representation.

Figure 4.2.4-6: Roaming 5G System architecture - Home routed scenario in reference point representation

The N38 references point can be between V-SMFs in the same VPLMN, or between V-SMFs in different VPLMNs (to enable inter-PLMN mobility).

NOTE 10:	For clarity, the NWDAF(s) with roaming exchange capability (RE-NWDAF) and their connections with other NFs, are not depicted in the reference point architecture figure. For more information on network data analytics architecture refer to TS 23.288 [86].

For the roaming scenarios described above each PLMN implements proxy functionality to secure interconnection and hide topology on the inter-PLMN interfaces.

Subscription-based routing to a particular core network as specified in clause 6.44 of TS 22.261 [2] enables forwarding of the signalling and user traffic of certain UEs to a target PLMN that may be neither the serving PLMN nor the HPLMN of the UE. This is achieved by selecting NFs residing in the target PLMN. The NRF of the HPLMN, with optional support of the NRF in that target PLMN as specified in clause 4.17.5 of TS 23.502 [3], is responsible to provide proper network function instance information during network function discovery and selection.

Figure 4.2.4-7: NRF Roaming architecture in reference point representation

NOTE 11:	For the sake of clarity, SEPPs on both sides of PLMN borders are not depicted in figure 4.2.4-7.

Figure 4.2.4-8: Void

Operators can deploy UPFs supporting the Inter PLMN UP Security (IPUPS) functionality at the border of their network to protect their network from invalid inter PLMN N9 traffic in home routed roaming scenarios. The UPFs supporting the IPUPS functionality in VPLMN and HPLMN are controlled by the V-SMF and the H-SMF of that PDU Session respectively. A UPF supporting the IPUPS functionality terminates GTP-U N9 tunnels. The SMF can activate the IPUPS functionality together with other UP functionality in the same UPF, or insert a separate UPF for the IPUPS functionality in the UP path (which e.g. may be dedicated to be used for IPUPS functionality). Figure 4.2.4-9 depicts the home routed roaming architecture where a UPF is inserted in the UP path for the IPUPS functionality. Figure 4.2.4-3 depicts the home routed roaming architecture where the two UPFs perform the IPUPS functionality and other UP functionality for the PDU Session.

NOTE 12:	Operators are not prohibited from deploying the IPUPS functionality as a separate Network Function from the UPF, acting as a transparent proxy which can transparently read N4 and N9 interfaces. However, such deployment option is not specified and needs to take at least into account very long lasting PDU Sessions with infrequent traffic and Inter-PLMN handover.

The IPUPS functionality is specified in clause 5.8.2.14 and TS 33.501 [29].

Figure 4.2.4-9: Roaming 5G System architecture - home routed roaming scenario in service-based interface representation employing UPF dedicated to IPUPS

/////// Next Change (all new text)  ///////

Annex X (informative):
Support of traffic offloading at the H-UPF deployed nearby VPLMN for HR PDU Session

X.1	General

The Home PLMN operator can deploy H-UPFs on sites located physically close to the VPLMN (e.g., visited region) for the support of the local traffic routing at the H-UPF, i.e., home session breakout performed at the H-UPF controlled by H-SMF.

Figure x.2-1: Deployment scenario for traffic offloading at the H-UPF deployed nearby VPLMN region for HR PDU Session

Figure x.2-1 depicts the deployment scenario of H-UPF nearby VPLMN for HR PDU Session for traffic offloading at the H-UPF. For simplicity, other network functions are not shown.

X.2	PDU Session establishment for supporting traffic offloading at the H-UPF

This clause describes the HR PDU Session establishment for the roaming traffic offloading at the H-UPF deployed nearby VPLMN.

The following are some considerations:

1.	The H-SMF decides to support roaming traffic offloading at the H-UPF for HR PDU Session based on the configured operator policy

2.	When performing H-UPF selection as described in clause 6.3.3.3, UE location information (e.g. MNC and MCC of the VPLMN) can be considered in order to select a H-UPF nearby VPLMN for a given HR PDU Session, based on the HPLMN operator policy.

2.	Based on the operator policy, the selected H-UPF deployed nearby VPLMN can provide the H-SMF with the DNS server information, if it is locally configured during the HR PDU Session Establishment as described in clasue 4.3.2.2.2 of TS 23.502 [3].

X.3	Re-establishment of PDU Session due to UP Path failure

For the deployment scenario where the H-UPF of HR PDU Session is deployed nearby VPLMN, if the H-SMF receives from H-UPF the UP Path failure report for the path between the H-UPF (PSA) and V-UPF, the H-SMF acts as described in TS 23.527 [x]. The H-SMF can trigger release of the HR PDU Session and include a cause value indicating that a PDU Session re-establishment is requested as described in clause 4.3.4.3 of TS 23.502 [3]. During the new HR PDU Session re-establishment, an H-UPF in home region can be selected based on operator policy.

/////// End of Change ///////