3GPP TSG-WG2 Meeting #166	S2-2411556

18 - 22 November, 2024, Orlando, USA	(revision of S2-2410683)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1246 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 – Update of VFL training and inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson, Apple, ETRI, InterDigital], vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solving multiple Editor’s Notes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes in 5.4: 1) Added that Vertical Federated learning is a machine learning techinique working without exchanging/sharing of local data set nor ML models 2) Editor's note: Support for AF acting as VFL server selecting multiple NWDAFs is FFS. Is solved by introducing: - An AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s)) to participate in a VFL procedure. 3) Editor's note: Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS. Is solved in procedure procedure for preparation phase Editor's note: For an NWDAF impacts of the split into AnLF and MTLF are FFS. are solved in procedure clauses, where NWDAFs being part of inference have earlier been part of training. 4) Adding where the procedures are described Changes in 6.2F.2: Clarifying that some parameters are needed for VFL. Changes in 6.2H Added Preparation procedure Alignments including Sample and feature alignment is done in this phase |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL not implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2, 6.2H.2.0, 6.2H.2.1, 6.2H.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 6.2G.2 and underlying sub clauses (changed text in this clause and underlying clauses, new sub clauses and moved sub clauses of 6.2H into this clause). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | [The following contents are added in rev3] This CR is proposed to focus on the VFL Preparation procedure as agreed in the last meeting, the clauses about terminology alignment and service description are postponed for the next meeting. Editor´s Note: The preparation procedure when an untrustrd AF is part of the procedure is FFS. Addressing the above EN by adding the description about the NEF. Editor´s note: Clarifications which parameters are regarded as requirements and can be modified by the VFL client is required Addressing the above EN by clarifying that the samples could be modified by the VFL Clinets, as it will be more efficient with flexible samples in preparation stage. Editor´s note: It is ffs whether the VFL client can only reduce the requirements (e.g. samples) or also add new requirements. It is also ffs whether step 1 needs to be repeated if requirements have been reduced by the VFL client. Addressing the above EN by clarifying that VFL client provides sample IDs selected by itself, and step 1 should not be repeated when the requirements is changed, as the VFL Server could determine the final samples based on the responses from VFL Clients. Editor´s note: It is ffs whether. the VFL server sends a separate message for each VFL client towards the NEF. Usage of temporary NWDAF IDs needs to be added. Addressing the above EN by clarifying that VFL Server should send one message to NEF with temporary NWDAF IDs. |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2H.2.2	Preparation procedure for Vertical Federated Learning

6.2H.2.2.0	General

Preparation procedure is used to check if the VFL Client(s) can meet the ML Model training requirement. The procedure includes the negotiation, between server and client(s) to enable interoperability, sample alignment and optionally feature alignment.

NOTE 1: 	Features can be non-specified, privacy protected and in this case feature alignment is a simple alignment using info registered in NRF, e.g. data sources info or proprietary Feature ID).

Editor´s note: Possible refinement of the selection of features during the preparation phase is FFS.

NOTE 2:	Vertical Federated Learning preparation procedure can be skipped if the VFL Server can decide which VFL Client(s) support the VFL procedure to be performed, e.g. based on local configuration.

6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Editor's note:	Whether the VFL Interoperability information (e.g. type of supported VFL training method(s)) needs to be part of the NF profile registered in NRF is FFS.

Editor´s Note: Whether the sample IDs are included as part of the MLModelTraining or a separate Nnwdaf service is defined is FFS.

Editor´s note:	For UEs as samples, additional discussion is needed on whether UE needs to be registerd or not.

Editor’s Note: whether the case of trusted AF as VFL Server is included in the clause of NWDAF as VFL server or in the clause of untrusted AF as VFL server is FFS.

1.	The NWDAF as VFL Server sends a Federated Learning preparation request to each of the VFL Client(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag. The Server may add a list of sample IDs, Analytics ID and optionally Feature ID. When the VFL Client is an untrusted AF, the VFL Server could send the FL preparation request to the NEF, and the NEF forwards the FL preparation request to the untrusted AF as VFL Client.

2.	Each VFL Client checks if it can meet the ML Model training requirement. If it cannot meet the requirements for any reason, it may decide which requirements it can agree to.

3.	Each VFL Client invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the VFL Server whether it accepts the requirements, whether it requests new requirements or whether it will not join. The client includes the requirements it can accept or may add a reason if it cannot join the VFL process. If the VFL client determines to join the VFL process, it should feedback sample IDs selected by itself. When the VFL Client is an untrusted AF, the VFL Client could send the response to the NEF, and the NEF forwards the response to the VFL Server.

4.	The VFL Server NWDAF determines the VFL Client(s) to be involved in the FL procedures based on the information received in step 6 in Figure 6.2H.2.1-1 (if performed) and other information received in step 3 (if available).

6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when the AF as the VFL server

This clause specifies the preparation (including sample alignment) procedure for AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL Server

Editor´s Note: Whether the sample IDs are included as part of the MLModelTraining or a separate Nnwdaf service is defined is FFS.

  - The untrusted AF as VFL server sends VFL preparation request to the candidate VFL client(s) with the ML Preparation Flag to check if the VFL client(s) can meet the ML Model training requirement, which may include Analytics ID, Sample alignment requirement, VFL Availability time requirement (time span needed for the VFL process), etc.. Sample alignment requirement includes a list of sample IDs targeted for VFL training as defined by the AF. The temporary NWDAF IDs obtained in the discovery procedure (see Clause 6.2H.2.1.1) is included to indicate the target NWDAFs.

  - The NEF maps the external UE IDs (e.g. GPSIs) to the internal UE IDs (e.g. SUPIs). The NEF may send VFL preparation request to each candidate VFL client based on the temporary NWDAF IDs using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service when multiple candidate VFL clients are involved with the same information as provided in step 1 in 6.2H.2.2.1.

Editor’s Note:	Whether to extend the existing ML Preparation Flag for reuse or introduce a new flag dedicated for VFL preparation is FFS.

Editor's note:	Whether to use ML Model interoperability information or introduce new identifiers for interoperability among NWDAF and AFs is FFS.

Editor’s Note:	The specific service name for the untrusted AF case is FFS.

  - Same as step 1 in 6.2H.2.2-1.

  - Same as step 2 in 6.2H.2.2-1.

  - Same as step 3 in 6.2H.2.2-1.

  - Based on the response from the NWDAF(s) as VFL Client(s) and optional sample alignment requirement from the AF as VFL Server, the NEF obtains the intersection of samples and maps internal UE ID(s) to external UE ID(s). When multiple candidate VFL clients are involved, the NEF may aggregate the response from each candidate VFL client. And the NEF sends a VFL Preparation response message to the untrusted AF with the intersection of samples.

* * * End of Changes * * *