3GPP TSG-WG SA2 Meeting #166	S2-2411580

# 18-22 Nov, 2024, Orlando, US		(revision of S2-2410978, 0670)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0574 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF enhancements for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, [Huawei], [HiSilicon], [NTT DOCOMO, [Nokia] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Regarding the following EN in clause 5.18, we introcude a new procedure of Location Measurement data collection from LMF for NWDAF side AI/ML positioning ML model training. Editor's note: The procedure of NWDAF containing MTLF collects input data for LMF-based AI/ML Positioning ML model training is FFS. Clarify that the LMF can request/subscribe to ML model from NWDAF based on the conclusions for KI#1 in TR 23.700-84. When the NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning, NWDAF needs to get data from LMF. However, currently, the LMF does not have services for exposing the positioning measurement data. Therefore, we propose to define new Nlmf_DataExposure_Subscribe, Nlmf_DataExposure_Notify, and Nlmf_DataExposure_UnSubscribe services for the data collection for NWDAF side AI/ML positioning ML model training. To remove the EN by adapt the procedure to ML model performance monitoring, by updating 6.X.4 title and figure generally as data collection by NWDAF, and add new clause 5.18.X to describe the ML training by LMF, NWDAF. Further clarify the input data for model training. It also proposes to use “input data” instead of “Data samples” which is not confusiong. Editor's note: It’s FFS how to adapt this procedure to ML model performance monitoring. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new procedure of Location Measurement data collection from LMF for NWDAF side AI/ML positioning ML model training. Define new LMF services for the data collection for NWDAF side AI/ML positioning ML model training. Other enhancements based on conclusions for KI#1 in TR 23.700-84. Corrections: LMF-based AI/ML positioning instead of AI/ML positioning. Update NL8 to support LMF to receive the performance monitoring result from NWDAF Update 6.X.4 title and figure to generally describe data collection by NWDAF. Add standalone clause 5.18.X to describe the AI/ML positioning model training by NWDAF and LMF. And clarify input data as ground truth data and UE measurements data. Replace “data samples” by “input data” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The exposure of LMF Location Measurement data and other enhancements based on KI#1 conclusions in TR 23.700-84 are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.X (new), 4.4.13, 6.X.4 (new), 8.3.1, 8.3.X (new); 5.18.X(new); |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Regarding the clause 6.X.4, it should be treated together with TS 23.273 CR0584 and CR0597. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (new) * * * *

4.3.X	Network Data Analytics Function, NWDAF

LMF may provide input data to NWDAF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

LMF may obtain ML model from NWDAF for performing LMF-based AI/ML Positioning.

# * * * * Next change * * * *

4.4.13	NL8 Reference Point

The NL8 reference point supports LMF to receive location related analytics from NWDAF, provide input data for model training or model performance monitoring to NWDAF, or receive AI/ML positioning model or model performance monitoring result from NWDAF, as defined in TS 23.288 [37].

# * * * * Next change * * * *

5.18.X ML model training for LMF-based AI/ML Positioning

NWDAF containing MTLF supports to train the ML model for LMF-based AI/ML positioning. When NWDAF containing MTLF determines to train the ML model based on the request from LMF or internal triggers, the NWDAF containing MTLF collects the input data for ML model training by invoking the procedure of Data collection by NWDAF for LMF-based AI/ML positioning as specified in clause 6.X.4.

LMF may also support to train the ML model for LMF-based AI/ML positioning in LMF. When LMF determines to train the ML model, LMF collects the input data for ML model training by invoking the proceudres of Data collection from NG-RAN for LMF-based AI/ML positioning described in clause 6.X.3. or/and Data collection from UE for LMF-based AI/ML positioning described in clause 6.X.2.

Input data of each UE/PRU for ML model training collected by LMF/NWDAF includes:

- measurements data from UE/gNB

- ground truth data of UE/PRU

# * * * * Next change (new) * * * *

6.X.4	Data collection by NWDAF for LMF-based AI/ML positioning

The NWDAF containing MTLF may request or subscribe to the input data (e.g., location measurements, and/or ground truth data) from LMF for AI/ML model training or AI/ML model performance monitoring for LMF-based AI/ML Positioning.

Editor's note: 	It’s FFS how to adapt this procedure to ML model performance monitoring.

Figure 6.X.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to collect the input data based on the request from LMF or internal trigger, e.g., to train an AI/ML model or to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters. If the NWDAF wants to collect the input data of PRUs, the NWDAF may also include a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

Editor's note: 	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes an AoI and a notification target address to request the input data from LMF. The NWDAF may also include requested number for data samples and time window for data collection.

Editor's note:	Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS.

4.	The LMF performs steps 1 to 4 in clause 6.X.2, or steps 1 to 7 in clause 6.X.3 to collect the input data.

5.	The LMF sends the collected input data to the NWDAF by invoking Nlmf_DataExposure_Notify service operation, then the NWDAF trains the ML model or performs ML model performance monitoring based on the input data.

# * * * * Next change * * * *

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | MeasurementData | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPSubcribe | Subscribe/Notify | AMF |
|  | UPNotify |  | AMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |
| Nlmf_DataExposure | Subscribe | Subscribe/Notify | NWDAF |
|  | Notify |  | NWDAF |
|  | UnSubscribe |  | NWDAF |

# * * * * Next change * * * *

8.3.X	Nlmf_DataExposure service

8.3.X.1	General

Service description: This service enables an NF service consumer to subscribe to input data for ML model training or ML model performance monitoring. See Clause 6.X.4 for examples of usage of this service.

8.3.X.2	Nlmf_DataExposure_Subscribe service operation

Service operation name: Nlmf_DataExposure_Subscribe

Description: Allow the consumer NF to subscribe the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Target Address, Notification Correlation ID, AoI.

Input, Optional: time window when to collect data, number of samples, Subscription Correlation ID (in the case of modification of the subscription).

Editor's note:	Whether the consumer NF (i.e. NWDAF) needs to indicate the data source type (i.e., UE location measurements or NG RAN location measurements) and/or data type(s) is FFS, which needs to be coordinated with RAN WGs. A PRU indication and/or a data sample precision threshold is also FFS.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

When the requested number of samples has been provided or the time window for data collection has elapsed, the subscription is automatically terminated.

8.3.X.3	Nlmf_DataExposure_Notify service operation

Service operation name: Nlmf_DataExposure_Notify

Description: Allow the consumer NF to get notification about the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Correlation ID.

Input, Optional: input data for ML Model traning/performance monitoring, Subscription Correlation ID (this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation ID by the LMF).

Editor's note:		The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

Output, Required: Operation execution result indication.

Output, Optional: None.

8.3.X.4	Nlmf_DataExposure_UnSubscribe service operation

Service operation name: Nlmf_DataExposure_UnSubscribe

Description: Allow the consumer NF to cancel subscription to input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

# * * * * End of changes * * * *