SA WG2 Meeting #166	S2-2411611

18 - 22 November, 2024, Orlando , US

Title:	[draft] Reply LS on request for IMS Data Channel related clarifications

Response to:	LS on request for IMS Data Channel related clarifications (S2-2411347/S3i240707)

Release:	Rel-18

Work Item:	NG_RTC

Source:	China Mobile (will be SA2)

To:	SA3-LI

Cc:	SA4, CT1, CT4, SA3

Contact person:	Yi Jiang

yijiang at Chinamobile dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA2 thanks SA WG3 for the LS on request for IMS Data Channel related clarifications.

SA2 clarifies that from IMS architectural perspective the DCSF and MF are required to be present in the IMS network that supports and provides IMS data channel services.

SA2 provides answers to the questions in the LS as below:

- The IMS-AS in the remote UE’s side of the IMS session always notifies the DCSF about the IMS Data Channel related event notifications, irrespective which side initiates the IMS Data Channel setup (originating party or the terminating party).

- SA2 answer: Yes, if the remote UE has subscription of IMS data channel.

- The statement 1 is true even if the local UE and the remote UE are in the same CSP domain.

- Sa2 answer: Yes. The IMS AS notifies events to the DCSF separately for local UE and remote UE.

- In addition to notifying the DCSF, the Remote UE’s network would also have an MF on the IMS Data Channel media.

- SA2 answer: The IMS data channel media may not be anchored on an MF, if the data channel is a P2P application data channel and has no requirement to be anchored for media processing.

- In the SessionEvenNotifictions, how are the Calling Identity and Called Identity populated by the IMS AS (i.e. which SIP headers from the initial INVITE, re-INVITE are used)?

- SA2 answer: Not in the scope of SA2.

- If call forwarding occurs at the terminating side (clause 10.123, TS 24.186), the two users present on the established IMS session (i.e. after the successful redirection) are:

  - Original calling party.

  - Redirected-to party.

5-1: 	Assuming Calling Identity is the original calling party, what would the Called Identity be that IMS AS sends in the session Event Notification to the DCSF, i.e.redirecting party identity or the redirected-to party identity?

SA2 answer: Not in the scope of SA2.

5-2:	If it is the former, what would be purpose including the Called Identity in the Session Event Notification, specially, when that user is not using the IMS Data Channel?

SA2 answer: Not in the scope of SA2.

5-3:	If it is the later, does the IMS-AS in the originating network know about the redirected-to party identity?

SA2 answer: Not in the scope of SA2.

5-4: 	Do the MF in the Local UE side and Remote UE side have same role – e.g., MF in the UDP Proxy or HTTP Proxy?

SA2 answer: SA2 clarifies the role of the MF is in media flow granularity and is decided by the DCSF. So it is possible for a MF to have different roles for different media flows in a IMS session.

2	Actions

To SA3-LI

ACTION: 	SA2 kindly asks SA3-LI to taken the answers above into account.

3	Dates of next TSG SA WG 2 meetings

SA2#166-Ad Hoc-e                 20th  – 24th January 2025                  Online

SA2#167                                 17th  – 21th February 2025                Greece