3GPP TSG-SA WG2 Meeting #166	S2-2411639

18th – 22nd November 2024	(revision of S2-24xxxx)

Title:	[Draft] LS Reply on Usage of E2E Data Volume Transfer Time Analytics

Response to:	LS S6-244627 on Usage of E2E Data Volume Transfer Time Analytics

Release:	Rel-19

Work Item:	FS_XRApp, eNA_Ph3

Source:	3GPP TSG SA WG2

To:	SA6

Cc:	-

Contact person:	Belen Pancorbo

belen.pancorbo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

SA2 would like to thank SA6 on the LS on usage of E2E Data Volume Transfer Time Analytics (S2-2411286 /S6-244627).

SA2 has discussed the questions and would like to provide the following answer.

“It is described in TS 23.288 clause 6.18.1: The data volume may be the expected or observed data volume from UE to AF or from AF to UE. The E2E data volume transfer time analytics may be used to assist an AF or NEF with AI/ML-based services, e.g. for member UE selection of federated learning. The usage of E2E Data Volume Transfer Time Analytics for member UE selection of federated learning can be seen as support for best-effort traffic type.

Question 1:	Whether the E2E Data Volume Transfer Time Analytics from NWDAF can be used for GBR and Delay-critical GBR traffic types?”

Response:	AI/ML-based services is one of the usage scenarios of the E2E Data Volume Transfer Time Analytics. The analytics can also be used in other scenarios, not limited to AI/ML-based services and best-effort traffic type. The E2E Data Volume Transfer Time Analytics is also applicable for support the scenarios with GBR and Delay-critical GBR traffic types. Considering that "XR traffic is characterized by multiple service flows and a wide variation in data sizes", as described in TS 23.288 clause 6.18.1, the data volume may be the expected or observed data volume from UE to AF or from AF to UE. That is the data volume as input parameter is a dedicated value, and this value could be expected or observed, and not necessary to be a fixed value in the real application scenarios.

“In Table 6.18.2-3 of TS 23.288, transmitted UL/DL data volume is one of the inputs from AF to NWDAF for E2E Data Volume Transfer Time Analytics.

Question 2:	Whether the analytics subscription can be updated with updated data volume, e.g. when the actual data volume changed after the XR traffic is started?”

Response:	The analytics subscription can be modified by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation with updated inputs. Thus, data volume as input to the E2E Data Volume Transfer Time Analytics subscription, can be modified when the actual data volume changed.

# 2	Actions

To SA6

ACTION: 	SA2 kindly asks SA6 to take the above answers into account.

# 3	Dates of next TSG SA WG2 meetings

SA2#166 Adhoc-e                 20th January – 24th January 2025 			Online

SA2#167                               17th February – 21st February 2025 			Athens, GR