3GPP TSG-WG2 Meeting #166	S2-2411651

# Orlando, FL, USA, November 18 – 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5140 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of mismatching capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In case the UE and network does not support a common set of steering functionalitities (e.g. if the UE only supports MPTCP and ATSSS-LL with Active Stanby and the network only supports MPQUIC-IP), the network may reject the MA PDU Session Establishment Request, or accept the new PDU Session as a single-access PDU Session. The UE can currently not influence the decision by the network. In some cases the UE may prefer that the PDU Session is reject in case the PDU Session cannot be established as a MA PDU Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the handling when there is no common steering functionality in the UE and the network. Allow the UE to indicate whether it prefers a rejection or a downgrade to a SA PDU Session in this case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing description. Unclear handling in the UE and the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.2.1, 4.22.2.2.1, 4.22.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.22.2	UE Requested MA PDU Session Establishment

#### 4.22.2.1	Non-roaming and Roaming with Local Breakout

The signalling flow for a MA PDU Session establishment when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is based on the signalling flow in Figure 4.3.2.2.1-1 with the following differences and clarifications:

-	The PDU Session Establishment Request message may be sent over the 3GPP access or over the non-3GPP access. In the steps below, it is assumed that it is sent over the 3GPP access, unless otherwise specified.

-	In step 1, the UE provides Request Type as "MA PDU Request" in UL NAS Transport message and its ATSSS Capabilities as defined in clause 5.32.2 of TS 23.501 [2] in PDU Session Establishment Request message. The UE may also include a "Downgrade to SA PDU Session not allowed" indication in the PDU Session Establishment Request message.

The "MA PDU Request" Request Type in the UL NAS Transport message indicates to the network that this PDU Session Establishment Request is to establish a new MA PDU Session and to apply one or more steering functionalities (defined in TS 23.501 [2], clause 5.32.6) for steering the traffic of this MA PDU session over multiple accesses.

The "Downgrade to SA PDU Session not allowed" indication indicates that the requested MA PDU Session should not be converted to a single-access PDU Session. If this indication is set, the SMF should reject the PDU Session Establishment if there is no steering functionality that can be used by both the UE and the network.

If the UE requests an S-NSSAI and the UE is registered over both accesses, it shall request an S-NSSAI that is allowed on both accesses.

The UE indicates to AMF whether it supports non-3GPP access path switching, i.e. whether the UE can transfer the non-3GPP access path of the MA PDU Session from a source non-3GPP access (N3IWF/TNGF) to a target non-3GPP access (a different N3IWF/TNGF).

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects an SMF, which supports MA PDU sessions. If the AMF supports non-3GPP access path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF selects a SMF that supports non-3GPP access path switching, if such an SMF is available.

-	In step 3, the AMF informs the SMF that the request is for a MA PDU Session by including "MA PDU Request" indication and in addition, it indicates to SMF whether the UE is registered over both accesses. If the AMF determines that the UE is registered via both accesses, but the requested S-NSSAI is not allowed on both accesses, then the AMF shall reject the MA PDU session establishment. If the AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the selected SMF supports non-3GPP path switching and UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF indicates whether the UE supports non-3GPP path switching to the SMF.

The AMF shall reject the PDU Session Establishment request if the request is for a LADN.

-	In step 4, the SMF retrieves, via Session Management subscription data, the information whether the MA PDU session is allowed or not.

-	In step 4, if there is no steering functionality that can be used by both the UE and the network, and the "Downgrade to SA PDU Session not allowed" indication is not provided in step 1, the SMF may decide to accept the requested PDU Session as a single-access PDU Session. The SMF may take this decision based on local operator policy, ATSSS Capabilities from the UE and DNN configuration in SMF, which are not specified in the present document. If the "Downgrade to SA PDU Session not allowed" indication is provided in step 1, the SMF should reject the PDU Session Establishment request if there is no steering functionality that can be used by both the UE and the network.

If the SMF decided to downgrade to a single-access PDU Session, the remaining steps in this clause are skipped, and instead the steps after step 6 in Figure 4.3.2.2.1-1, clause 4.3.2.2.1, are used. Otherwise following steps are executed.

-	In step 7, if dynamic PCC is to be used for the MA PDU Session, the SMF sends an "MA PDU Request" indication to the PCF in the SM Policy Control Create message and the ATSSS Capabilities of the MA PDU session. The SMF provides the currently used Access Type(s) and RAT Type(s) to the PCF. The PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

The PCF provides PCC rules that include MA PDU session control information, as specified in TS 23.503 [20]. From the received PCC rules, the SMF derives (a) ATSSS rules, which will be sent to UE for controlling the traffic steering, switching and splitting in the uplink direction and (b) N4 rules, which will be sent to UPF for controlling the traffic steering, switching and splitting in the downlink direction. If the UE indicates the support of "ATSSS-LL Capability", the SMF may derive the Measurement Assistance Information.

If the SMF receives a UP Security Policy for the PDU Session with Integrity Protection set to "Required" and the MA PDU session is being established over non-3GPP access, the SMF does not verify whether the access can satisfy the UP Security Policy.

-	In the remaining steps of Figure 4.3.2.2.1-1, the SMF establishes the user-plane resources over the 3GPP access, i.e. over the access where the PDU Session Establishment Request was sent on:

-	In step 10, the N4 rules derived by SMF for the MA PDU session are sent to UPF and two N3 UL CN tunnels info are allocated by the UPF. If the ATSSS LL functionality is supported for MA PDU Session, the SMF may instruct the UPF to initiate performance measurement for this MA PDU Session. If the MPTCP functionality and/or the MPQUIC functionality is supported for the MA PDU Session, the SMF may instruct the UPF to activate the MPTCP functionality and/or the MPQUIC functionality for this MA PDU Session. In step 10a, the UPF allocates addressing information for the Performance Measurement Function (PMF) in the UPF. If the UPF receives from the SMF a list of QoS flows over which access performance measurements may be performed, the UPF allocates different UDP ports or different MAC addresses per QoS flow per access. In step 10b, the UPF sends the addressing information for the PMF in the UPF to the SMF. If UDP ports or MAC addresses are allocated per QoS flow and per access, the UPF sends the PMF IP address information and UDP ports with the related QFI to the SMF in the case of IP PDU sessions and sends the MAC addresses with the related QFI to the SMF in the case of Ethernet PDU sessions.

In step 10a, if the message from the SMF instructs the UPF to activate MPTCP functionality, the UPF allocates the UE "MPTCP link-specific multipath" addresses/prefixes. In step 10b, the UPF sends the "MPTCP link-specific multipath" addresses/prefixes and MPTCP proxy information to the SMF. If the message from the SMF instructs the UPF to activate MPQUIC functionality, the UPF allocates the UE "MPQUIC link-specific multipath" addresses/prefixes. In step 10b, the UPF sends the "MPQUIC link-specific multipath" addresses/prefixes and MPQUIC proxy information to the SMF. The "MPTCP link-specific multipath" addresses/prefixes and the "MPQUIC link-specific multipath" addresses/prefixes may be the same.

-	In step 11, for the MA PDU session, the SMF includes an "MA PDU session Accepted" indication in the Namf_Communication_N1N2MessageTransfer message to the AMF and indicates to AMF that the N2 SM Information included in this message should be sent over 3GPP access. The AMF marks this PDU session as MA PDU session based on the received "MA PDU session Accepted" indication. If the AMF indicated in step 3 that non-3GPP path switching while maintaining two N2 connections for non-3GPP access is supported, the SMF indicates support of non-3GPP path switching in the PDU Session Establishment Accept message.

-	In step 13, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by SMF. If the ATSSS -LL functionality is supported for the PDU Session, the SMF may include the addressing information of PMF in the UPF into the Measurement Assistance Information. If the MPTCP functionality is supported for the MA PDU Session, the SMF shall include the "MPTCP link-specific multipath" addresses/prefixes of the UE and the MPTCP proxy information. If the MPQUIC functionality is supported for the MA PDU Session, the SMF shall include the "MPQUIC link-specific multipath" addresses/prefixes of the UE and the MPQUIC proxy information.

-	After step 18 in Figure 4.3.2.2.1-1, if the SMF was informed in step 2 that the UE is registered over both accesses, then the SMF initiates the establishment of user-plane resources over non-3GPP access too. The SMF sends an Namf_Communication_N1N2MessageTransfer to the AMF including N2 SM Information and indicates to AMF that the N2 SM Information should be sent over non-3GPP access. Namf_Communication_N1N2MessageTransfer does not include an N1 SM Container for the UE because this was sent to UE in step 13. After this step, the two N3 tunnels between the PSA and RAN/AN are established.

The last step above is not executed when the UE is registered over one access only, in which case the MA PDU Session is established with user-plane resources over one access only. How user-plane resources can be added over an access of the MA PDU Session is specified in clause 4.22.7.

**** Next Change ****

#### 4.22.2.2	Home-routed Roaming

##### 4.22.2.2.1	Home-routed Roaming - UE registered to the same PLMN

When the UE is registered to the same VPLMN over 3GPP access and non-3GPP access, the MA PDU Session is established as specified in Figure 4.3.2.2.2-1 ("UE-requested PDU Session Establishment for home-routed roaming scenarios") with the differences and clarifications:

-	The PDU Session Establishment Request message may be sent over the 3GPP access or over the non-3GPP access.

-	In step 1, the UE provides Request Type as "MA PDU Request" in UL NAS Transport message and its ATSSS Capabilities, as defined in clause 5.32.2 of TS 23.501 [2] in PDU Session Establishment Request message. The UE indicates to AMF whether it supports non-3GPP access path switching. The UE may also include a "Downgrade to SA PDU Session not allowed" indication in the PDU Session Establishment Request message.

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects a V-SMF and an H-SMF, which supports MA PDU sessions. The V-SMF serves the UE over both accesses. If the AMF supports non-3GPP access path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF selects a V-SMF and H-SMF supporting non-3GPP access path switching, if such a V-SMF and H-SMF are available.

-	In step 3, the AMF informs the SMF that the request is for a MA PDU Session by including an "MA PDU Request" indication and in addition, the AMF indicates to V-SMF that the UE is registered over both accesses. If the AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the selected SMFs supports non-3GPP path switching and UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF indicates whether the UE supports non-3GPP path switching to the V-SMF.

-	In step 5, two DL N9 tunnel CN info and two UL N3 tunnel CN info are allocated by the V-SMF or by the V-UPF.

-	In step 6, the V-SMF informs the H-SMF that the request is for a MA PDU Session by including an "MA PDU Request" indication and indicates to H-SMF that the UE is registered over both accesses. The V-SMF indicates to H-SMF the relationship between the DL N9 tunnel CN info and the access type. If the single CN Tunnel is established by the H-SMF, the DL N9 tunnel info binding to the access over which the NAS message is received is to be used. The V-SMF also provides the "Downgrade to SA PDU Session not allowed" indication, if received in the PDU Session Establishment Request message.

-	In step 7, the H-SMF retrieves, via Session Management subscription data, the information whether the MA PDU session is allowed or not.

-	In step 7, if the "Downgrade to SA PDU Session not allowed" indication was not received by H-SMF, the H-SMF may decide to convert the MA PDU Session requested by the UE into a single-access PDU Session. The H-SMF may take this decision based on local operator policy, ATSSS Capabilities from the UE and DNN configuration in H-SMF, which are not specified in the present document.

If the SMF decided to downgrade to a single-access PDU Session, the remaining steps in this clause are skipped, and instead the steps after step 7 in Figure 4.3.2.2.2-1, clause 4.3.2.2.2, are used. Otherwise, following steps are executed.

-	In step 9, if dynamic PCC is to be used for the MA PDU Session, the H-SMF sends an "MA PDU Request" indication to H-PCF in the SM Policy Control Create message and the ATSSS Capabilities of the MA PDU session. The H-SMF provides the currently used list of Access Type(s) and RAT Type(s) for the MA-PDU session to the H-PCF. The H-PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

The H-PCF provides the PCC rules containing MA PDU session control information and the H-SMF derives the ATSSS rules for the UE and the N4 rules for the H-UPF.

-	In step 12, two UL N9 tunnel CN info are allocated by the H-SMF or by the H-UPF. After this step, the two N9 tunnels between the H-UPF and V-UPF are established.

-	In step 13, the H-SMF sends "MA PDU session Accepted" indication to V-SMF in the Nsmf_PDUSession_Create Response message. The H-SMF indicates to V-SMF the relationship between the UL N9 tunnel CN info and the access type.

-	In step 14, the V-SMF sends the "MA PDU session Accepted" indication in the Namf_Communication_N1N2MessageTransfer message to the AMF and indicates the AMF to send the N2 SM Information included in this message over the access that the UE sent the PDU Session Establishment Request. The AMF marks this PDU session as MA PDU session based on the received "MA PDU session Accepted" indication.

The V-SMF indicates support of non-3GPP path switching in the PDU Session Establishment Accept message.

If the V-SMF received two UL N9 tunnel CN info from the H-SMF, the V-SMF also initiates the establishment of user-plane resources over the other access. The V-SMF sends an N1N2 Message Transfer to AMF including N2 SM Information and the other access type to indicate to AMF that the N2 SM Information should be sent over the other access. The N1N2 Message Transfer does not include an N1 SM Container for the UE which was sent to UE over the access that the UE sent the PDU Session Establishment Request.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by H-SMF and may include Measurement Assistance Information.

-	After step 20, if the V-SMF was informed in step 3 that the UE is registered over both accesses, then the V-SMF initiates the establishment of user-plane resources over the other access too. The V-SMF sends an N1N2 Message Transfer to the AMF including N2 SM Information and indicates to the AMF over which access the N2 SM Information should be sent. The N1N2 Message Transfer does not include an N1 SM Container for the UE because this was sent to the UE in step 14. After this step, two N9 tunnels between the H-UPF and the V-UPF as well as two N3 tunnels between the V-UPF and RAN/AN are established, or, if the H-UPF is connected to two different V-UPFs, the H-UPF has one N9 tunnel with each V-UPF.

**** Next Change ****

##### 4.22.2.2.2	Home-routed Roaming - UE registered to different PLMNs

When the UE is registered to different PLMNs over 3GPP access and non-3GPP access, the MA PDU Session is established first over one access as specified in Figure 4.3.2.2.2-1 ("UE-requested PDU Session Establishment for home-routed roaming scenarios") and then over the other access with the following differences and clarifications:

-	In step 1, the UE provides Request Type as "MA PDU Request" in UL NAS Transport message and its ATSSS Capabilities, as defined in clause 5.32.2 of TS 23.501 [2]. The UE indicates to AMF whether it supports non-3GPP access path switching. The UE also includes the PDU Session ID of the already established MA PDU Session. The UE may also include a "Downgrade to SA PDU Session not allowed" indication in the PDU Session Establishment Request message.

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects a V-SMF, which supports MA PDU sessions. If the AMF supports non-3GPP access path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF may select a V-SMF and H-SMF supporting non-3GPP access path switching. If the UE provides Request Type "MA PDU Request" in step 1 and the UE context in SMF data from UDM includes SMF identity information for that PDU Session ID, the AMF selects the H-SMF indicated by UDM.

-	In step 3, the AMF informs the V-SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication). If the AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the selected SMFs supports non-3GPP path switching and UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF indicates whether the UE supports non-3GPP path switching to the V-SMF.

-	In step 6, the V-SMF informs the H-SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication). The V-SMF also provides the "Downgrade to SA PDU Session not allowed" indication, if received in the PDU Session Establishment Request message.

-	In step 7, the H-SMF retrieves, via Session Management subscription data, the information whether the MA PDU session is allowed or not.

-	In step 7, if the "Downgrade to SA PDU Session not allowed" indication was not received by H-SMF, the H-SMF may decide to convert the MA PDU Session requested by the UE into a single-access PDU Session. The H-SMF may take this decision based on local operator policy, ATSSS Capabilities from the UE and DNN configuration in H-SMF, which are not specified in the present document.

If the SMF decided to downgrade to a single-access PDU Session, the remaining steps in this clause are skipped, and instead the steps after step 7 in Figure 4.3.2.2.2-1, clause 4.3.2.2.2, are used. Otherwise, following steps are executed.

-	In step 9, if dynamic PCC is to be used for the MA PDU Session, the H-SMF sends an "MA PDU Request" indication to PCF in the SM Policy Control Create message and the ATSSS Capabilities of the MA PDU session. The H-SMF provides the currently used Access Type(s) and RAT Type(s) for the MA-PDU session to the PCF. The PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

-	In step 14, the V-SMF indicates support of non-3GPP path switching in the PDU Session Establishment Accept message.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by H-SMF and may include Measurement Assistance Information.

-	After the MA PDU Session is successfully established on the first access, the UE shall initiate again the MA PDU Session establishment procedure in Figure 4.3.2.2.2-1 over the other access with the following differences and clarifications:

-	In step 1, the UE shall send another PDU Session Establishment Request over the other access containing the same PDU Session ID that was provided over the first access. The UE also provides Request Type as "MA PDU Request" in UL NAS Transport message. The UE indicates to the AMF whether it supports non-3GPP access path switching.

-	In step 2, if the AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the may select a V-SMF that support non-3GPP access path switching.

-	In step 3, if the AMF supports non-3GPP path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF indicates whether the UE supports non-3GPP path switching to the V-SMF.

-	In step 12, new UL N9 tunnel CN info is allocated by the H-SMF or by the H-UPF.

-	In step 14, the V-SMF indicates support of non-3GPP path switching in the PDU Session Establishment Accept message.

-	In step 16, the UE receives another PDU Session Establishment Accept message, which may contain updated ATSSS rules for the MA PDU session.

-	After step 20, two N9 tunnels between the H-UPF and two different V-UPFs as well as two N3 tunnels between different V-UPF and RAN/AN are established, or two N3 tunnels, one is between V-UPF and RAN/AN over 3GPP access and the other is between H-UPF and RAN/AN over non 3GPP access, as well as one N9 tunnel between H-UPF and V-UPF are established.

**** End of Changes ****