3GPP TSG-SA2 Meeting #165	S2-2411696

# Orlando, USA 18 – 22nd November, 2024		          (revision of S2-240)

Source: 	Nokia

Title: 		Support of Multiplexed Media Identification Information as part of Media Related Information for end-to-end ciphered traffic (feature interaction between KI4 and KI2)

Document for: 	Approval

Agenda Item: 	19.3

Work Item / Release:	XRM_Ph2 / Rel-19

Abstract of the contribution: support of flow multiplexing for ciphered traffic (feature interaction between KI4 and KI2)

# 1	Discussion

- KI4 aims to specify the possibility of using Multiplexed Media Identification information such as RTP SSRC and PT headers to demultiplex (S)RTP/UDP media flows multiplexed over a single IP 5-tuple when e.g. these flows require distinct QoS handling.

- However, the multiplexed media flows could also be ciphered and/or carried by other protocols than (S)RTP/UDP, for example when running on top of QUIC protocol. Actually, a growing portion of the XR traffic sent over the Internet is carried in this way.

- Therefore, the specifications related to KI4 should consider also the case of demultiplexing ciphered non-(S)RTP/UDP based media flows.

- KI 2 related specifications define Media Related Information (including PDU set information) which is transported from AS proxy to UPF along with the end-to-end (UDP) payload using an in-band signaling protocol (e.g., connect-UDP or QUIC-aware proxying). This Media Related Information may contain Multiplexed Media Identification.

  - The description in this discussion document does not address usage of MoQ but it is not the intention of the authors to exclude MoQ

- Specifications cannot be complete when they don’t consider Feature interactions between features defined in a study/work Item

# 2 Proposal

It is proposed to endorse the following

R19 XRM-Ph2 related work will specify that

- It shall be possible that the Media Related Information sent by the AS using and In-Band Signaling protocol (connect-UDP, QUIC-aware proxying or UDP options) to the UPF includes Multiplexed Media Identification Information similar to (S)RTP SSRC and PT defined as part of KI4; this Multiplexed Media Identification  may be  used by UPF to demultiplex (S)RTP media flows. The Multiplexed Media Identification Information shall support both RTP and non-RTP based media streams multiplexing.

- Support of XRM media multiplexing for ciphered XRM traffic may be defined for UL traffic but beyond 5GC sending related QoS rules, whether and how to support these QoS rules is left for UE implementation.

NOTE: 	Multiplexed Media Identification Information can only be included if HTTP datagrams [udp-Connect] or QUIC packet [QUIC aware proxying] only carry the content of a single media stream. This should be ensured by the AS (proxy) implementation.