3GPP TSG-SA WG2#166	S2-2411745

Orlando, FL,USA, 18-22 November 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	Discussion on non-3GPP Device Connection Information

Document for:	Discussion/Approval

Agenda Item:	19.8.2

Work Item / Release:	UIA_ARC

Abstract of the contribution: This contribution analysed the requirement of parameters contained in non-3GPP Device Connection Information for PDU Session Establishment and proposed a way forward.

# 1.	Discussion

There is no agreement in SA2#165 on what parameters are provided by the UE/5G-RG in PDU Session Establishment Request.

For Ethernet PDU Session, the UE includes Device Identifier, MAC address and optional VLAN tag of the non-3GPP device in PDU Session Establishment request or PDU session modification request.

The discussion mainly focused on the scenarios of IP type of PDU Session. The UE IP address/prefix is only allocated to the UE by the network after PDU Session Establishment. As a result, the UE cannot include the IP address/prefix in the non-3GPP Device Connection Information in the PDU Session Establishment Request.

Given this, the following two options can be distinguished:

-	Option 1: Conclude that the UE cannot send non-3GPP Device Connection Information in PDU Session Establishment Request of IP type PDU Session. This means that after successful establishment of PDU Session, the UE triggers PDU Session Modification procedure to send non-3GPP Device Connection Information, including Device Identifier, IP address/prefix and port ranges for NAT.

-	Option 2: In some scenarios the UE may already have at least a subset of the non-3GPP Device Connection Information available (e.g., Device Identifier, reserved port ranges for NAT). In these scenarios the UE could be allowed to send the available information in non-3GPP Device Connection Information already in the PDU Session Establishment Request of IP type PDU Session. After UE IP address allocation, the SMF provides the UE IP address/prefix to PCF which complements the non-3GPP Device Connection Information sent during PDU Connection Establishment for QoS differentiation of non-3GPP Device traffic.

Observations:

Option 1 requires the UE to always also trigger a PDU Session Modification procedure for the QoS differentiation of non-3GPP device traffic even if the UE already has a subset of the non-3GPP Device Connection Information available (Device Identifier, reserved port ranges for NAT) already at the time of PDU Session Establishment.

Option 2 avoids the need to trigger PDU Session Modification procedure for UEs that already have a subset of the non-3GPP Device Connection Information available (Device Identifier, reserved port ranges for NAT) at the time of PDU Session Establishment.

# 2.	Proposal

Option 2 is more efficient in signalling exchange.

It is proposed to agree the way forward of IP PDU Session Establishment for non-3GPP Device with Option 2.