SA WG2 Meeting #166	S2-2411791

18 - 22 November, 2024, Orlando, USA

# Title:	Discussion paper on the relocation reporting indication

# Source:	Huawei, HiSilicon

# Document for:	Approval

Agenda Item:	19.11.2

Abstract:	This contribution analyses why the relocation reporting indication is needed for reporting the remaining data from source UPF to consumer when UPF relocation occurs. A related CR is in S2-2411790.

Discussion

According to Clause 6.4.2 of TS 23.288, the NDWFA may collect QoS monitoring results (i.e., QoS flow Bit Rate, QoS flow Packet Delay, Packet transmission and Packet retransmission) from UPF for Service Experience analytics.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| Finer granularity location (1...max) | GMLC | UE positions. |
| ....>UE location |  | GAD shape or location coordinates (see TS 23.032 [34]). |
| ....>Timestamp |  | A time stamp when the location was measured. |
| ....>LCS QoS |  | The accuracy of the measurement. |
| UE ID | AMF | List of SUPIs. If UE IDs are not provided as Target of Analytics Reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| DNAI | SMF | Identifies the access to DN to which the PDN session connects. |
| PDU Session type | SMF | Type of the PDU Session. |
| SSC Mode | SMF | SSC Mode selected for the PDU Session. |
| Access Type | SMF | List of Access Types used for the PDU Session. |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Packet transmission | UPF | The observed number of packet transmission. |
| Packet retransmission | UPF or AF | The observed number of packet retransmission. |

According to Clause 6.5.2 of TS 23.288, the NDWFA may collect Traffic usage report from UPF for UPF load analytics.

Table 6.5.2-2: Data collected by NWDAF for UPF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Traffic usage report | UPF | Report of user plane traffic in the UPF for the accumulated usage of network resources (see TS 29.244 [17]) |

According to Clause 5.1 and clause 6.2.9 of TS 23.288, the data collected by the NWDAF is used for model training or data analytics (i.e., model inference).

5.1	General

The NWDAF provides analytics to 5GC NFs and OAM as defined in clause 7. An NWDAF may contain the following logical functions:

-	Analytics logical function (AnLF): A logical function in NWDAF, which performs inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and exposes analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo.

-	Model Training logical function (MTLF): A logical function in NWDAF, which trains Machine Learning (ML) models and exposes new training services (e.g. providing trained ML Model) as defined in clause 7.5 and clause 7.6.

NOTE 1:	NWDAF can contain an MTLF or an AnLF or both logical functions.

NOTE 2:	Pre-trained ML Model storage and provisioning to NWDAF is out of the scope of 3GPP.

6.2.9	User consent for analytics

Depending on local policy or regulations, to protect the privacy of user data, the data collection, ML Model training and analytics generation for a SUPI or GPSI, Internal or External_Group_Id or "any UE" may be subject to user consent bound to a purpose, such as analytics or ML Model training. The user consent is subscription information stored in the UDM, which includes:

a)	whether the user authorizes the collection and usage of its data for a particular purpose;

b)	the purpose for data collection, e.g. analytics or model training.

Observation 1: The NWDAF may need to collect QoS monitoring results or Traffic usage report from the UPF. The data from UPF is used by the NWDAF for model training or data analytics.

The collected data is used by NWDAF to extract features, which are the input of ML models. The features may be, for example, average Bit Rate over 5 minutes or maximum Packet Delay over 10 minutes. In order to be able to generate the appropriate features, the NWDAF may indicate the data source (e.g., UPF) to report data according to certain requirements. For example, to generate the average Bit Rate over 5 minutes, the NWDAF can indicate the UPF to report QoS monitoring results every 5 minutes (i.e., periodic reporting, and period is equal to 5 minutes). In this scenario, collecting data less than 5 minutes is not useful to the NWDAF, since it cannot be used to generate valid features. Therefore, when the UPF changes, although the UPF has collected some data and has not reported it to the NWDAF because the reporting period has not reached, the NWDAF can indicate the UPF not to report the data if the data is not useful to the NWDAF.

In addition, according to Clause 5.2.2.2 of TS 29.244, the consumer can indicate some reporting thresholds to the UPF, and the UPF only needs to report the data when the corresponding thresholds are reached. Therefore, when the UPF changes, the NWDAF can indicate the UPF not to report the data if the corresponding thresholds are not reached.

5.2.2.2.1	General

When provisioning a URR, the CP function shall provide the reporting trigger(s) in the Reporting Triggers IE of the URR which shall cause the UP function to generate and send a Usage Report for this URR to the CP function. When adding or removing reporting trigger(s) to or from the URR, the CP function shall provide the new complete list of applicable reporting triggers in the Reporting Triggers IE in the PFCP Session Modification Request message.

For the volume-based measurement method, the CP function may provision:

-	the Volume Threshold IE, to request the UP function to generate a usage report when the measured traffic reaches the threshold;

-	the Volume Quota IE, to request the UP function to stop forwarding packets (or only allow forwarding of some limited user plane traffic, based on operator policy in the UP function) and, if no Volume Threshold is provisioned, to also generate a usage report, when the measured traffic reaches the quota;

-	the Dropped DL Traffic Threshold IE, to request the UP function to generate a usage report when the downlink traffic that is being dropped reaches the threshold; and/or

Observation 2: When UPF changes, the data that has been collected by source UPF but not sent to the consumer may be not useful to the consumer in some scenarios. Therefore, the consumer can indicate the UPF whether to report the remaining data when UPF changes.

Proposal 1: The UPF event consumer can include a relocation reporting indication in the subscription message to indicate whether the collected data needs to be sent to the consumer when the UPF relocation occurs.

Summary of Proposals

Proposal 1: The UPF event consumer can include a relocation reporting indication in the subscription message to indicate whether the collected data needs to be sent to the consumer when the UPF relocation occurs.