3GPP TSG-WG SA2 Meeting #166	S2-2411805

18 - 22 November, 2024, Orlando, USA	(revision of S2-24xxxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Local offloading traffic handling at I-SMF

Document for:	Approval

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_ph3 / Rel-19

Abstract: This contribution discusses about the how to do the local offloading at the I-SMF.

1. Introduction

In previous two SA2 meeting for local offloading handling at the I-SMF, some progress has been made. However there are still several open issues to be resolved. In this paper we analyze those issue and give the related proposal.

2. Discussion

The open issue can be listed as: 1) How the local offloading policy is transferred to I-SMF; 2) How to configure the ULCL/L-PSA UPF controlled by the I-SMF; 3) Clarification the usage of the AF traffic influence information at the I-SMF.

Issue 1: Local offloading policy transferred to I-SMF

The related EN is as following:

- Editor’s Note: It is FFS whether the local offloading policy received from PCF is sent to the I-SMF via SMF transparently or not.

- Editor’s Note: It is FFS whether SMF transfers transparently Local Offloading Policy to I-SMF.

The above EN is related to how SMF transfers the local offloading policy to I-SMF.

The local offloading policy is used to indicate which IP range(s) or FQDN(s) can (or cannot) be offload. If the local offloading policy received from PCF is associated with DNAI, i.e. some IP range(s) or FQDN(s) can only be offloaded via some dedicated DNAI(s), it is beneficial that the SMF filters the received policy and transfers only the policy I-SMF interested.

During the offline discussion, it seems that the local offloading policy is preferred to be defined to not have any DNAI level granularity, then the SMF cannot do any filtering and just transparently transfer the received offloading policy to the I-SMF.

Proposal 1: Depending on whether the local offloading policy have DNAI level granularity, SMF forwards the local offloading policy received from PCF to the I-SMF transparently or not.

Issue 2: How to configured UL-CL/L-PSA

The related EN in the revised paper is as following:

- Editor’s note: Whether I-SMF or SMF generates N4 rules is FFS.

The above EN is related to how to configure the ULCL/L-PSA UPF controlled by the I-SMF.

There are two options on how to resolve this issue: a) Reusing the ETSUN mechanism and N4 rule information is generated by the SMF and sent to the I-SMF; b) N4 rule information is determined by the I-SMF locally.

Regardless whether the N4 rule information is generated by SMF or I-SMF, as the DNS query is handled by I-SMF directly, only the I-SMF has knowledge on the traffic to be offloaded, i.e. the related traffic descriptor. The difference between these two mechanisms is on whether the I-SMF sends the offloaded traffic information to SMF or use that information to configure the UL-CL/L-PSA directly.

N4 rule generated by the SMF

In the existing ETSUN procedure, when the I-SMF requests the SMF to generate the N4 rule, it uses DNAI to indicate the traffic to be offloaded. In the local offloading case, the traffic to be offloaded is the result of the DNS query, which is not aware by the SMF. Using the DNAI information to identify the traffic to be offloaded is impossible, i.e. existing ETSUN procedure need be updated if the N4 rule is still generated by SMF.

Observation 1: if the N4 rule (for UL-CL/L-PSA controlled by the I-SMF) is generated by SMF, using the DNAI as existing ETSUN mechanism to identify the traffic to be offloaded is impossible. The I-SMF need describe the offloaded traffic and reports it to the SMF, i.e. the existing ETSUN procedure need be updated.

N4 rule generated by the I-SMF

From I-SMF view it is possible to use the traffic information to configure the UL-CL/L-PSA directly. At S2#165 meeting the question related to this approach is that no interface between SM-PCF and I-SMF. Whether Policy and Charging may cause some issue:

- Policy issue: The traffic to be offloaded is the traffic which AF have SLA agreement with 5GC network. In most case the offload traffic does not need dedicated policy, i.e. default policy is enough. If there are some traffic which need be treated specially, it can be solve by implementation option, e.g. as part of the EDI information or preconfigured at the I-SMF. Hence Policy issue can be solved via the implementation mechanism.

- Charging issue: there are two solutions to solve this issue. 1) similar as the HR-SBO mechanism, the SMF can notify the charging information via the RoamingChargingProfile as defined in TS 29.502; Per that information, the I-SMF can do the charging similar as the V-SMF. 2) in the existing network deployment flat charging package have been widely adopted, in that case the offloaded information maybe not need be counted; Hence charging issue depends on the operator’s policy.

- Existing HR-SBO: above Policy/Charging issue also happens in the HR-SBO scenario. All the mechanism considering in the HR-SBO case can also be used here.

As the last resort operator can also choose that some dedicated traffic are not allowed to be offloaded, e.g. some traffic exceeds the volume threshold configured at the I-SMF. This can be part of the local offloading policy.

Observation 2: if the N4 rule (for ULCL/L-PSA controlled by the I-SMF) is generated by I-SMF, depending on the operator’s policy the Policy and Charging issue can be solved by implementation mechanism. Operator can also choose some dedicated traffic not offloaded if needed.

The intention of the KI#1 is to reduce the burden of the central SMF on EC handling. It seems natural that the ULCL/L-PSA UPF should also be configured by the I-SMF to avoid that each time after successful handling of DNS query, the I-SMF need report the related offloading traffic to the SMF, then SMF generates the N4 information and return it back again to the I-SMF. In case the UE moves within the I-SMF service area and traffic need be offloaded from via one DNAI(L-PSA) to via another DNAI(L-PSA), the I-SMF should also be able to manage it by itself. Hence we see the benefit that the I-SMF need be able to configure the N4 rule for UPF controlled by it. This is the target of KI#1.

Based on above analyse we see the advantage and potential drawback on configure the N4 rule by I-SMF directly. As similar mechanism has been adopted in the HR-SBO case, we do not see problem due to introduction of the N4 rule configured by the I-SMF directly.

Proposal 2: For the UL-CL/L-PSA controlled by the I-SMF, the N4 rule is generated by I-SMF directly and not need the assistance from SMF. This can be triggered due to the result of DNS query or for preparation of DNS query.

Issue 3: Usage of the AF traffic influence information at the I-SMF

In the TR conclusion it is agreed that the AF traffic influence information is sent to the I-SMF if it is available. We do see some necessity to transfer this information to the I-SMF.

It is I-SMF determines how to select UL-CL/L-PSA per the received information, e.g. EDI/local offloading policy. The AF traffic influence information is also part of the information to assist the UL-CL/L-PSA selection. It is more suitable to have all information for ULCL/L-PSA selection in one place to have a consistence decision and avoid any potential conflict decision.

In the existing ETSUN procedure, the DNAI of interest is sent to I-SMF per the AF traffic influence information and the DNAI supported by I-SMF. This is used for I-SMF to determine ULCL/L-PSA UPF selection. If the AF traffic influence information has already sent to I-SMF, the I-SMF can directly determine ULCL/L-PSA need be inserted and in which place based on that information. Hence the DNAI of interest sent by SMF is not needed.

Proposal 3: For the handling of the AF traffic influence information:

- AF traffic influence information is sent to I-SMF for UL-CL/L-PSA selection.

- The SMF does not need to return the interest of DNAI to the I-SMF.

3. Conclusion and proposal(s)

It is proposed to discuss and agreed above Proposal. The related accompanied paper (S2-2411806/11807/11808) are also proposed on the concrete change.