3GPP TSG-WG SA2 Meeting #166	S2-2411912

# Orlando, USA, 18th Nov – 22nd Nov, 2024		(revision of S2-2410483)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1418 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of mixed traffic handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to support the traffic detection and QoS Flow mapping for mixed traffic multiplexed within one transport connection, it is concluded in FS_XRM_Ph2 to introduce the media flow media flow description information within the IP packet filter set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds support of mixed traffic handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI cannot be completed without support of mixed traffic handling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5783 TS 23.502 CR 5103 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.1.3.27.X		Support of Mixed Traffic Handling

IP Packet Filters with (S)RTP Multiplexed Media Information may be provided by the AF to request different QoS handling for multiplexed media flow(s) as described in clause 5.7.6.2 and 5.37.X of TS 23.501 [2]. The PCF may, based on local configuration and/or the input from the AF as well as the UE/UPF capability information on support of (S)RTP Multiplexed Media Information, generate the PCC rules as following:

-	If both the UE and UPF capability information indicate the support of (S)RTP Multiplexed Media Information, the PCF generates PCC rules which contain IP Packet Filter Set with the (S)RTP Multiplexed Media Information together with QoS Parameters for the multiplexed media flow(s).

-	If the UE capability information indicates no support of (S)RTP Multiplexed Media Information while UPF supports, the PCF generates the PCC Rule with the IP Packet Filter without (S)RTP Multiplexed Media Information and corresponding QoS Parameters for the uplink traffic flow. For the downlink direction, the PCF generates the PCC Rules which contain the IP Packet Filter Set with the (S)RTP Multiplexed Media Information for the multiplexed media flows.

-	If the UE capability infomormation indicates support of (S)RTP Multiplexed Media Information while UPF doesn’t support, the PCF generates the PCC Rule with the IP Packet Filter without (S)RTP Multiplexed Media Information and corresponding QoS Parameters for the downlink traffic flow. For the uplink direction, the PCF generates the PCC Rules which contain the IP Packet Filter Set with (S)RTP Multiplexed Media Information for the multiplexed media flows.

-	If neither UE nor UPF supports the (S)RTP Multiplexed Media Information, the PCF generates the PCC Rule(s) with the IP Packet Filter Set without (S)RTP Multiplexed Media Information and corresponding QoS Parameters for the traffic flow.

The PCF further provides the generated PCC Rule(s) to the SMF.

In case the UE/UPF cannot support the (S)RTP Multiplexed Media Information, depending on how the PCF local policies are set, the PCF may need to notify the AF that the requested QoS requirements cannot be fulfilled due to the lack of support from UE/UPF. The AF may re-provision the QoS requirements for the traffic flow for the corresponding direction in which the UE/UPF cannot support the (S)RTP Multiplexed Media Information.

Alternatively, the AF may provide QoS requirements for the traffic flow in addition to the QoS requirements for the media flows. Then the PCF may, based on the UE/UPF capability information on support of (S)RTP Multiplexed Media Information, generate the PCC Rules as described above.

# * * * * End of changes * * * *