3GPP TSG-SA WG2 Meeting #166	S2-2411989

# Orlando, USA, November 18-22, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1523 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility procedure for UE-Satellite-UE communication in IMS – continuation of optimized media routing under a single satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, this CR adds description on Mobility procedure of UE-Satellite-UE communication in IMS focusing on continuation of optimized media routing when both UEs are and will be served under a single satellite. This CR is written based on the following principle as well as the above-mentioned conclusion. - Use of the N5 interface between IMS and 5GC is assumed. - IMS AS, requested by P-CSCF via SIP MESSAGE, sends SIP re-INVITE to complete IMS AGW relocation. (P-CSCF does not need to act as SIP B2B UA.) - No UE impact. - No new SDP attribute is introduced. (A new SIP header is introduced for conveying satellite ID.) - Subscription data is not considered. This CR is prepared since the procedure of continuation of optimized media routing causes the SIP glare issue (i.e., the issue of both networks sending SIP re-INVITE to the other network simultaneously) when both UEs are and will be served under a single satellite. This issue needs to be resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Mobility procedure of UE-Satellite-UE communication in IMS focusing on continuation of optimized media routing when both UEs are and will be served under a single satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.5.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after TS 23.228 CR 1428 and CR 1521. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

AX.5.2.3 Continued optimized media routing procedure when both UEs are served by a single satellite

Figure AX.5.2.3-1 depicts a signalling flow diagram for continuation of optimized media routing after the originating UE and the terminating UE served by a single satellite becomes served by a different single satellite and change of satellites happens simultaneously for both UEs.

NOTE 1:	IMS entities not relevant for the procedure are omitted below for brevity of the description.

Use of the N5 interface between IMS and 5GC is assumed.

Based on the procedure described in clause AX.5.1, P-CSCF is expected to subscribe from 5GC for the early and the late notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic by using a scheme defined in TS 23.501 [93] and TS 23.502 [94].

Figure AX.5.2.3-1: Continued optimized media routing procedure when both UEs are served by a single satellite

The steps in the call flow are as follows:

1.	Step 1 of clause AX.5.2.1 applies both as it is and as the originating network and the terminating network are exchanged to each other.

2.	P-CSCFs in the originating network and in the terminating network have recognized that UEs in the originating network and in the terminating network are served by a single satellite and recognize that both UEs become served by a different single satellite that has the satellite ID received in step 1. Each P-CSCF determines to continue activating optimized media routing.

NOTE 2:	How P-CSCF judges that both UEs become served by a different single satellite is not specified.

3-6.	Steps 3-6 of clause AX.5.2.1 except for configuration of the IP address allocated in the remote network in step 4 apply both as it is and as the originating network and the terminating network are exchanged to each other.

7.	Step 7 of clause AX.5.2.1 applies. P-CSCF in the terminating network waits for interaction from the originating network.

8-15.	Steps 8-13 of clause AX.5.2.1 apply. In addition, in step 12, P-CSCF in the terminating network sets an implementation-specific timer for releasing IMS AGW on the source satellite in the terminating network that expires sometime after UE in the originating network receives SIP re-INVITE in step 17.

16. P-CSCF requests IMS AGW on the target satellite to configure the IP address received in step 15 to be used by the IMS AGW on the target satellite as the destination of media traffic towards the terminating network. This step 16 is performed according to clause 8.4 of TS 23.334 [74].

17-21.	Steps 14-18 of clause AX.5.2.1 apply. In addition, in step 21, P-CSCF in the terminating network releases IMS AGW on the source satellite in the terminating network after the timer being set in step 12 expires.

## ---End of the Change---