3GPP TSG-WG SA2 Meeting #166	S2-2412005

# Orlando, USA, 18-22 Nov, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3838 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of MME obtaining mapped Cell ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_SAT_ARCH_EPS |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | From the SA2 pespective, in the case of satellite access for NB-IoT, if an eNB can return a mapped Cell ID with higher accuarcy to the MME based on the UE Coarse Location Information, it is useful for services that require accurate cell identification (e.g. lawful intercept, charging). Based on LS R3-245819 from RAN3, RAN3 already supports MME transfers UE Coarse Location Information to RAN: • The eNB may request the MME to provide the coarse UE location of an NB-IoT NTN UE for e.g. load balancing among different beams. • Upon this request, the MME may signal the coarse UE location to eNB, if available. Based on R3-245817 from RAN3: The eNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. the wording ‘received from the UE’ after ‘UE location information’ has been removed, which means the UE Coarse Location Information from MME can also be used. SA2 can align with RAN3 to clarify the MME can transfer UE Coarse Location Information to eNB to further obtain a mapped cell ID from eNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify MME can transfer UE Coarse Location Information to the eNB, and the eNB can return a mapped Cell ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MME cannot obtain a mapped Cell ID with sufficient accuarcy for services that require accurate cell identification (e.g., lawful intercept, charging). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.4a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.13.4a Support of transferring UE Coarse Location Information to RAN

In the case of NB-IoT, if an eNB has requested the coarse UE location information from an MME using S1-AP INITIAL UE MESSAGE, the MME shall send the coarse UE location information to RAN using S1-AP INITIAL CONTEXT SETUP, DOWNLINK NAS TRANSPORT or CONNECTION ESTABLISHMENT INDICATION messages after obtaining Coarse Location Information from the UE.

NOTE:  	As described in TS 36.300 [5], the eNB is responsible for constructing mapped Cell ID based on the UE location information, in the case of NB-IoT, the eNB can return a mapped Cell ID with higher accuarcy to MME based on the UE Coarse Location Information from MME, it is useful for services that require accurate cell identification (e.g., lawful intercept, charging).

# * * * * End of changes * * * *