3GPP TSG-WG SA2 Meeting #165	S2-2412013

# Orlando, USA, 18-22 Nov, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1493 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS-AGW relocation for UE-Sat-UE communication at change of satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on 5GSAT_Ph3_ARCH WID work task, UE-satellite-UE communication via IMS-AGW deployed on satellite should be supported. When satellite changes, the IMS-AGW on satellite may be relocated to a target satellite or on the ground. This can be triggerd by a re-INVITE message. The IMS-AS, based on trigger from the P-CSCF, sends a re-INVITE message to UE via the P-CSCF at change of satellite to trigger media re- negotiation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an IMS-AGW relocation procedure for UE-Sat-UE communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompleted specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.5.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Implementation notes: The referenced TS23.501 clause 5.4.x.2.3 is added by TS23.501 CR5770 and this CR should be implemented after TS23.228 CR1492. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AX.5.2	At change of satellite

Based on the call setup procedure described in clause AX.5.1, both originating and terminating P-CSCF should subscribe to the SMF for the Notification of User Plane Management Events (i.e., UP path change) associated with UE-Satellite-UE communication media traffic, with Early and Late notifications type based on clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94].

At change of satellite, the SMF serving the originating or terminating UE may report a target satellite identifier and a candidate DNAI associated with UPF onboard the target satellite (if the target satellite is connected with source satellite) or associated with UPF on ground network (if the target satellite is not connected with source satellite). The procedure in this clause describes how media re-negotiation and IMS-AGW relocation are performed after receiving the report, in which the P-CSCF A or B can be any side of the originating or terminating P-CSCF. The P-CSCF performs different actions based on the candidate DNAI reported from SMF:

-	If the SMF reports a target satellite identifier and a candidate DNAI associated the UPF onboard the target satellite, the P-CSCF shall determine whether UE-satellite-UE communication can continue, e.g., by checking whether the target satellite is connected with the peer side satellite (if they are not the same).

-	If the SMF reports a candidate DNAI associated with the PSA UPF on ground network, the P-CSCF shall perform media re-negotiation as normal after selecting a ground IMS-AGW.

Clause AX.5.2.1 describes how the IMS-AGW relocation and media re-negotiation procedure is performed, clause AX.5.2.1.1 descibes the case of ground DNAI reported from SMF and AX.5.2.1.2 descibes the case of DNAI and target satellite identifier reported from SMF.

### AX.5.2.1 IMS-AGW relocation and media re-negotiation procedure

#### AX.5.2.1.1 In case of ground DNAI reported from 5GC

Figure AX.3.2.1.1-1: In case of ground DNAI reported from 5GC

1.	P-CSCF A receives a Late Notification from the SMF which contains a DNAI associated with a PSA UPF on ground network.

NOTE X: When UE handovers to a target satellite, the UL CL/BP and L-PSA is not inserted on the target satellite by SMF in this case, PSA UPF on ground is used to route media traffic between UE and source IMS-AGW A on satellite for service continuity.

2. 	As the reported DNAI is associated with UPF on the ground network, P-CSCF A selects an IMS-AGW A' on ground and allocates transport resources with step 3a and 3b.

3.	P-CSCF A sends a re-INVITE Request Indication to IMS-AS A via a SIP MESSAGE to trigger it to sends a re-INVITE message to UE A.

4.	After receiving the re-INVITE message, P-CSCF A generates an SDP Offer and sends it to UE A using the re-INVITE message.

5. 	After receiving the re-INVITE message with SDP offer, the UE sends a 200 OK message to the P-CSCF A. P-CSCF A allocates transport resources with step 5a and 5b.

6.	P-CSCF A sends 200 OK with SDP offer to the IMS-AS A.

7. 	IMS-AS A sends a re-INVITE message carrying the SDP offer to P-CSCF B via IMS-AS B.

8.	As the re-IVITE message in step 7 does not contain a satellite identifier, P-CSCF B re-selects an IMS-AGW B' on ground and request to allocation transport resource.

9.	P-CSCF B sends a re-INVITE message to UE B.

10.	After receiving 200 OK message from UE, P-CSCF B request to allocation transport resource.

11.	P-CSCF B sends the 200 OK message to IMS-AS A via IMS-AS B.

12.	The IMS-AS A sends an ACK to P-CSCF B via IMS-AS B, and the P-CSCF B forward the ACK to UE B.

13.	After sending ACK to UE B, P-CSCF B releases IMS-AGW B on satellite and P-CSCF B may instruct the SMF via PCF to remove UL CL/BP and L-PSA UPF from source satellite by providing an explicit notification that media traffic from/to this UE to/from IMS-AGW B on satellite ceases to exist (see NOTE 7 of clause 4.3.5.7 in TS 23.502 [94]).

14.	IMS-AS A sends an ACK with SDP answer to P-CSCF A, the P-CSCF A request IMS-AGW A' to allocation transport resource.

15.	The P-CSCF A send an ACK with SDP answer to UE A.

16. P-CSCF A releases IMS-AGW A on source satellite.

#### AX.5.2.1.2 In case of DNAI and target satellite identifier reported from 5GC

Figure AX.3.2.1.1-2: In case of target satellite identifer and DNAI reported from 5GC

1.	P-CSCF A receives an Early Notification from SMF which contains a target satellite identifier and a DNAI associated with a UPF onboard the target satellite.

NOTE Y: The simultaneous change of Branching Point or UL CL and additional PSA in TS 23.502 [94] is perfomed by SMF for service continuity in this case.

2. 	P-CSCF A determines whether UE-Satellite-UE communication can continue or not using target satellite identifier and identifier of satellite serving UE B, and select IMS-AGW A on target satellite if UE-Satellite-UE communication can continue, otherwise, P-CSCF A selects IMS-AGW A' on ground. P-CSCF A also request to allocate transport resouces in step 2a and 2b.

NOTE Z:	How the P-CSCF uses the satellite identifications to derive whether the two satellites are connected with ISL(s) is based on implementation or operators’ policy e.g., by querying transport network system, or based on node level information reported by the IMS-AGW deployed on the satellites which presents that the two satellite are connected or not.

3. 	P-CSCF A sends a re-INVITE Request Indication to IMS-AS A via SIP MESSAGE to trigger it to sends a re-INVITE message to UE A.

4.	IMS-AS send a re-INVITE message to UE via P-CSCF A. After receving the re-INVITE message, P-CSCF A generates an SDP Offer and sends it to UE A using the re-INVITE message.

5.	After receiving the re-INVITE message with SDP offer, the UE sends a 200 OK message with SDP answer to the P-CSCF A. After receiving the SDP answer, P-CSCF A requests to allocate transport resources in step 5a and 5b.

6.  P-CSCF A sends 200 OK with SDP offer to the IMS-AS A, if UE-Satellite-UE communication can continue, target satellite identifier is also carried by the 200 OK message.

7.	IMS-AS A sends a re-INVITE message with SDP offer and target satellite identifier if available to P-CSCF B via IMS-AS B.

8.	P-CSCF B decides whether the UE-Satellite-UE communication can continue based on whether there is satellite identifier in the received re-INVITE message, if there is no satellite identifier received, the UE-Satellite-UE communication cannot continue, the P-CSCF B re-selects an IMS-AGW B' on ground and request to allocate transport resource. If satellite identifier is received, the UE-Satellite-UE communication can contiunue, the P-CSCF B update the IMS-AGW B on satellite using the address information of the target IMS-AWG A on satellite in SDP offer from P-CSCF A.

9.	P-CSCF B sends a re-INVITE message to UE B.

10.	After receiving 200 OK message from UE, P-CSCF B request to allocate transport resource.

11.	P-CSCF B sends the 200 OK message with SDP Answer IMS-AS A via IMS-AS B.

12.	The IMS-AS A sends an ACK to P-CSCF B via IMS-AS B, and the P-CSCF B forward the ACK to UE B.

13. After sending ACK to UE B, if IMS-AGW B' on ground is selected, P-CSCF B releases IMS-AGW B on satellite and P-CSCF B may instructs SMF via PCF to removes UL CL/BP and L-PSA UPF from satellite by providing an explicit notification that media traffic between the UE and IMS-AGW B on satellite ceases to exist.

14.	The IMS-AS A sends an ACK with SDP answer to P-CSCF A. P-CSCF A requests target IMS-AGW A on satellite or IMS-AGW A' on ground to allocate transport resource in step 14a and 14b.

15. If target IMS-AGW A on satellite is selected, P-CSCF A instructs SMF via PCF to update UL CL/BP and L-PSA forwarding rules by providing following paramaters:

-	A flow description information which contains the IP address of the target IMS-AGW A;

-	A DNAI associated with the IMS-AGW A on-board the target satellite and corresponding N6 traffic routing information.

16.	P-CSCF A forwards the ACK message with SDP offer to UE.

17. If target IMS-AGW A' on ground is selected, after sending ACK to UE A, P-CSCF A may instruct SMF via PCF to removes UL CL/BP and L-PSA UPF from source satellite and target satellite by providing an explicit notification that media traffic between the UE and source IMS-AGW A on satellite ceases to exist (see NOTE 7 of clause 4.3.5.7 in TS 23.502 [94]).

18. P-CSCF A releases source IMS-AGW A on satellite.

# * * * * End of changes * * * *