3GPP TSG-WG SA2 Meeting #166	S2-2412034

# 18-22 Nov, 2024, Orlando, US		(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0627 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the input data based on RAN LS feedback. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In reply LS (S2-2409568/R1-2405578), there are agreements in RAN1 on the input data for training AI/ML model: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Propose: Update the collected input data with specific parameters based on RAN1 LS, to specify that input data for ML model training/performance monitoring may include channel measurement, quality indicator of channel measurement, time stamp of channel measurement, or/and include ground truth label (or its approximation), quality indicator of label, time stamp of label. |  |  |  |  |  |  |  |  |
| Summary of change: |  | update input data for ML model training/performance monitoring to include channel measurement, quality indicator of channel measurement, time stamp of channel measurement, or/and include ground truth label (or its approximation), quality indicator of label, time stamp of label. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIML_CN feature is not completed in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (new) * * * *

5.18	Support for UE positioning based on a ML Model at the LMF

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 5.18.1.

Editor's note:	The procedure of LMF collects input data from NG-RAN for ML model training is FFS.

LMF may also request a ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF. LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37]. LMF requests ML model training for LMF-based AI/ML Positioning with ML Model Filter Information, which may indicate the area of interest. NWDAF containing MTLF collects input data to perform the ML model training and model provision to LMF as described in clause 6.2A of TS 23.288 [37].

Editor's note:	The procedure of NWDA containing MTLF collects input data for LMF-based AI/ML Positioning ML model training is FFS.

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

Editor's note:	The Performance monitoring procedures are FFS.

The collected input data for ML model training or ML model performance monitoring may include channel measurement, quality indicator of channel measurement, time stamp of channel measurement, or/and include ground truth label (or its approximation), quality indicator of label, time stamp of label.

# * * * * End of changes * * * *