SA WG2 Meeting #166	S2-2412072

# 18 - 22 November 2024, Orlando, USA                 (revision of S2-2408214)

Source: 	Xiaomi

Title: 	Discussion for max number restriction of non-3GPP devices behind UE

Document for: 	information

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This paper is to discuss the un-necessity of max number restriction of non-3GPP devices behind UE.

1	Discussion

The restriction on max number of simultaneously active Device Identifier per UE/5G-RG was concluded in SA2#163 meeting, but need further discussion to determine how to handle it.

-	The optional restriction within the 5GS on max number of simultaneously active Device Identifiers per UE/5G-RG for device whose traffic need to be differentiated will be handled at the normative stage.

Observation1: there is not necessary to set max number of simultaneously active Device Identifier per UE/5G-RG restriction on network side, it is the owner of the UE or the capacity of the UE to determine how many non-3GPP devices it can support. There is no value for network to make such restriction, if the UE has the capabilities to support more numbers of non-3GPP devices to access to network, and also the owner of the UE/non-3GPP devices wants to, why to make such restriction, what benefit it can bring?

More importantly, it brings negative user experience. For example, one user subscribes to the UIA service with the max number (e.g., max:10) one UE can support, when user decides to add more non-3GPP devices (e.g., max:20), or reduce the number of non-3GPP devices (e.g., Max:5), the user has to update the subscription in network/operator, even if the UE has the ability to support more numbers.

Observation 2: Currently, 3GPP devices or non-3GPP devices can access to the Internet via another UE’s portable hotspot, in this case, it is implementation for UE to control the number how many 3GPP/non-3GPP devices can simultaneously access to this UE.

Proposal: It is implementation to perform restriction on the max number of simultaneously active Device Identifiers per UE/5G-RG.

2	Proposal

It is proposed to agree the following statement for the restriction on max number in S2-2412070:

It is implementation to perform restriction on the max number of simultaneously active Device Identifiers per UE/5G-RG.