3GPP TSG-WG SA2 Meeting #166	S2-2412087

# Orlando, USA, 18th Nov – 22nd Nov, 2024		(Revision of S2-2411083)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1310 | rev | 11 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCC enhancements for support of identified non-3GPP devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, InterDigital Inc., NEC, Nokia, OPPO, ZTE, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in SP-240971 - Once a UE/5G-RG binds the Device Identifier to a non-3GPP device, QoS differentiation can be performed within a PDU Session (using different QoS Flows) or between PDU Sessions. In the case of non-3GPP devices sharing a PDU Session, QoS Flows are provisioned in the UE/5G-RG using PDU Session Modification procedures. - UE/5G-RG may send the Device Identifier and user plane address of the non-3GPP device to the SMF in a NAS-SM message, and the SMF will forward the Device Identifier and user plane address to the PCF. The PCF will take it into account for policy decisions based on the Device Identifier information retrieved from the UDR. Device Identifier is used to retrieve the information from the UDR. The user plane address for a non-3GPP device can be the UE IP Address and/ Port ranges in case of IPv4 or an IPv6 address for IP PDU session type and the MAC address and/or the VLAN tag ID that is associated with the non-3GPP device’s traffic for Ethernet PDU Session type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add session management related policy control for identified non-3GPP devices connecting behind a UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Session management related policy control for identified non-3GPP devices connecting behind a UE is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | ‘Clause 5.xx’ in this CR’s clause 6.1.3.x refers to the clause 5.x in CR 5750 of TS 23.501 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

#### 6.1.3.x	Policy control for non-3GPP devices connecting behind a UE

The support of QoS differentiation for non-3GPP devices connecting behind a UE using a Non-3GPP Device Identifier is specified in clause 5.xx of TS 23.501 [2].

The SMF may receive from the UE the Non-3GPP Device Identifier and, optionally, the corresponding user plane address as specified in TS 23.502 [3].  When a Policy Control Request Trigger is met, the SMF sends the Non-3GPP Device Identifier and the user plane address of the non-3GPP device to the PCF. At reception of the Non-3GPP Device Identifier and user plane address, the PCF creates PCC rule(s) considering the Non-3GPP Device Identifier. The PCF determines QoS parameters in the PCC rule(s) based on the Non-3GPP Device Identifier Information which is retrieved from the UDR and/or based on operator policy. The PCF installs the PCC Rules at the SMF to enable QoS handling of the traffic to/from the non-3GPP device behind the UE.

* * * End of Changes * * * *