SA WG2 Meeting #166	S2-2412104

18 - 22 November, 2024, Orlando, USA

Title:	[Draft] Reply LS on UE-Satellite-UE Communication Architectures

Response to:	LS (S2-2411348/s3i240752) on "Reply LS on UE-Satellite-UE Communication Architectures"

Release:	Rel-19

Work Item:	5GSAT_Ph3_ARCH

Source:	SA2

To:	SA3-LI

Cc:	SA3, CT1, CT3, CT4

Contact Person:

Name:	Hucheng Wang

E-mail Address:	wanghucheng@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks SA3-LI for the Reply LS on UE-Satellite-UE Communication Architectures and would like to make the following clarifications.

For the Point 1 (where I-UPF and IMS-AGW on-board the satellite are selected), SA2 confirms that P-CSCF will be able to determine the continuity of the UE-Satellite-UE communication when the IMS-AGW is changed during the IMS call irrespective of LI. SA2 has discussed IMS-AGW changing procedure and the solution is agreed (CR#xxxx of TS 23.228).

For the Point 2 (where only I-UPF on-board the satellite is used), SA2 has not discussed the detailed solutions and procedures for this scenario yet. However, based on the existing solutions in the TR 23.700-29, there are two options:

Option A: The MO (mobile originating) P-CSCF negotiates with the MT (mobile terminating) P-CSCF to determine whether UE-sat-UE communication between the MO UE and the MT UE can be activated based on the satellite IDs serving the two UEs. If UE-sat-UE communication can be activated, the P-CSCF provides 5GC with the IP addresses of the two UEs to trigger the insertion of I-UPF on-board the satellite into the IMS PDU sessions, and a N19 tunnel is established between the MO I-UPF and the MT I-UPF. (see the Sol#28, Sol#29 and Sol#31 in the TR)

Option B: The MO P-CSCF negotiates with the MT P-CSCF to determine whether UE-sat-UE communication can be activated based on the satellite IDs serving the two UEs, and requests the 5GC to insert a UL-CL and a local PSA on the satellite into the PDU session. The 5GC will provide the N6 tunnel info (allocated by local PSA) to the P-CSCF, then the N6 tunnel info can be sent to the MT P-CSCF, which allows to establish N6 tunnel between the local PSA serving MO UE and the local PSA serving MT UE. (see the Sol#40 and Sol#43)

Thus, SA2 has following responses to the bullets from SA3-LI:

- P-CSCF is able to identify the I-UPF. For this, I-UPF address/identity will be made available to the P-CSCF.

[SA2 Answer]: Option A doesn’t need the I-UPF address, but option B may need it.

- P-CSCF is able to identify the tunnel used for the IMS media flow. For this, the tunnel ID used at the I-UPF will be made available to the P-CSCF. If a point-to-point tunnel is used (e.g. on N6), then the UDP port number along with the IP address of the I-UPF shall be made available to the P-CSCF.

[SA2 Answer]: Option A doesn’t need tunnel ID allocated by the I-UPF, but option B may need it.

- P-CSCF is able to identify the UDP port number (RTP) used for the media packets. Port numbers from the access side (N3) and the core-side (N6) will be available.

[SA2 Answer]: Both option A and B don’t need UDP port number.

- P-CSCF is able to identify the QoS flow. For this, the QFI associated with QoS flow will be made available to the P-CSCF.

[SA2 Answer]: Both option A and B don’t need QFI.

2. Actions:

To SA3-LI group.

ACTION: 	SA2 kindly asks SA3-LI to:

-	Check the agreed CR for the Point 1 and provide feedback if further actions are required for SA2 considering LI requirement;

-	For the Point 2, take above information into consideration and provide feedback on whether LI requirements can be satisfied if solution option A and/or B is adopted.

3. Date of Next SA WG2 Meetings:

SA2#166-Ad Hoc-e                     January 20th – 24th, 2025                                      Online meeting

SA2#167                                      February17th - 22th, 2025                                      Athens, Greece