3GPP SA WG2 Meeting #166	S2-2412119

Orlando, US, 18 - 22 November 2024

Source: 	Sateliot, Samsung, Thales, Novamint

Title: 	DP on the “List of Satellite IDs” and “S&F Monitored List”

Document for: 	Agreement

Agenda Item: 	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC / Rel-19

Abstract of the contribution: This paper discusses on the concepts and intentions of the “List of Satellite IDs” and “S&F Monitored List” as per conclusions in TR 23.700-29, proposes an option consisting of specifying two separate lists and concludes with a proposal of the options that could be acceptable.

# 1	Discussion

SA2#165 meeting discussed the 23.401 CR3801 on 5GSAT_ARCH_Ph3 (S2-2410989) that introduces the general clause on the Store and Forward architecture. The CR was postponed due to, among others, a controversial issue arisen about how to specify the use of the “S&F Monitoring List” information element and expected UE behaviour.

In our view, to progress on that issue, it’s important to distinguish between the concept and intention of the (1) “List of Satellite IDs” information element proposed for the split-MME architecture and (2) the “S&F Monitored List” proposed for the full EPC onboard architecture.

Conclusions of TR 23.700-2 related to “List of Satellite IDs” for the split-MME architecture are as follows (text related to the point at stake is highlighted in yellow):

With the following normative impacts: 1) When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information). NOTE 1: The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal. 2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB. 3) How the UE process this information is up to UE implementation and during the wait timer the UE can search for another terrestrial or satellite PLMN to get normal service. 4) MME may indicate to HSS the "Request Time", allowing the HSS to check that no other (e.g. terrestrial) MME has sent an Update Location Request after the "Request Time", and fetches the authentication vector and other details from HSS following current Authentication and security procedures. The MME may triggers Update location with the HSS and Update location ACK is received by MME. i.e. all the subscription details are retrieved by MME-ground. The Update Location Request includes an indication that this location update is provisional i.e. the HSS must not consider the UE as registered until it receives the final Update Location Request. 5) When the wait timer has expired given to the UE in step 2, if the UE has not successfully attached to another PLMN and the UE finds the cell which broadcast the Satellite ID valid to re-attempt the attach procedure, the UE re-sends the Attach or TAU Request message. 6) During the Attach or TAU procedure with the UE, the MME may also provide a list of Satellite IDs over which the UE may exchange the signalling and data, and a wait-timer that indicates to the UE the time it should wait before attempting signalling and data exchanges in those satellites. 7) The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

Conclusions of TR 23.700-29 related to “S&F Monitoring list” for the full EPC onboard architecture are as follows (text related to the point at stake is highlighted in yellow):

With the following normative impacts: - Store and forward is only supported by EPS. - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data. - The UE needs to be aware that a satellite supports S&F mode. NOTE 9: How the UE is aware that a satellite supports S&F mode of operation depends on RAN. - A UE may be rejected if the satellite cannot support the UE at this time. The attach reject may provide a timer for the time the UE should wait before reattempting and S&F monitoring list which the UE can attempt attach again.

Observation 1: “List of Satellite IDs” as concluded in TR 23.700-29 for the split-MME and “S&F Monitoring list” as concluded in TR 23.700-29 for the full EPC onboard architecture are not intended for the same purpose.

For the split-MME architecture, the “List of Satellite IDs” is mainly intended to indicate to the UE which are the subset of satellites of the selected PLMN from which the UE is expected to receive service in S&F mode. According to the agreed 23.401 CR3800 (S2-2410990) for the annex with the description of the split-MME architecture, the “List of Satellite IDs” would actually correspond to the satellites hosting the multiple MME-onboard parts, that together with a MME-GND part, behave as the (single) MME serving the UE. Therefore, once the UE gets the “List of Satellite IDs”, the UE should restrict interaction to such satellites, given that other satellites of the same PLMN will not provide access to the same MME where the UE is registered or trying to register.

The fact that the “List of Satellite IDs” was described as optional in the TR 23.700-29 was mainly because it was assumed that the default case is “all” the satellites. That is, if a UE is not provided with the “List of Satellite IDs”, this UE could be interacting with any of the satellites of the selected PLMN, given that the MME-onboard parts in all of them would form part of a single MME.

Observation 2:  For the split-MME architecture, the “List of Satellite IDs” is mainly intended to indicate to the UE which are the subset of satellites of the selected PLMN from which the UE is expected to receive service in S&F mode.

For the full EPC onboard architecture, the “S&F Monitored List” is mainly intended to assist UEs in retrieving MT data. As the UE is expected to attach and detach in each satellite pass, the “S&F Monitored List” gives an indication to the UE in which satellites the UE should preferably keep attaching/detaching at least to retrieve potential MT traffic. Therefore, the fact that a UE may try access to a different satellite of the same PLMN not included in the “S&F Monitored List” is not so critical, assuming that UE credentials could be available in other satellites.

Observation 3: For the full EPC onboard architecture, the “S&F Monitored List” is mainly intended to assist UEs in retrieving MT data.

Observation 4: Both intentions (i.e. (1) establish the set of satellites of the selected PLMN from which the UE is expected to receive service in S&F mode and (2) indicate which of those serving satellites will be used by the network to serve MT traffic) are relevant and do not exclude each other.

Proposal 1: Consider the specification of two satellite lists for S&F mode operation:

| Information Element | Purpose | Comment |
| --- | --- | --- |
| S&F Serving Satellites List | If provided along with a “Reject cause due to S&F operation” during a NAS procedure, the UE can attempt to retry the rejected NAS procedure in any of the satellites of the selected PLMN listed in the “S&F Serving Satellites List” not later than a validity timer. If provided during a successful NAS procedure, a UE should consider for subsequent NAS procedures only the satellites of the selected PLMN listed in the “S&F Serving Satellites List”. A “S&F Serving Satellites List” is valid as long as the UE registration remains valid. If not provided, the UE could interact with any satellite of the selected PLMN. | For the split-MME, this is applicable to an scenario where a “single MME” is not deployed in all the satellites of the constellation. For the full EPC onboard, this could be applicable to a scenario where UE credentials are not provisioned in all the satellites of the constellation. |
| S&F Monitoring Satellites List | If provided during a successful NAS procedure, UE should only expect MT traffic to be delivered through the satellites of the selected PLMN listed in the “S&F Monitoring Satellites List”. If provided together with the “S&F Serving Satellites List”, the “S&F Monitoring Satellites List” should be a subset of the former. If not provided, the UE should expect MT traffic in any of the satellites listed in “S&F Serving Satellites List” or, if the latter is not provided either, in any of the satellites of the selected PLMN. |  |

Proposal 2: Decide a solution among the following options:

1) Use separate list, as in Proposal 1 above.

2) Use common list and have an indication whether it is a mandatory or advisory.

3) Use common list and use term “should” for UE behaviour.

# 2	Proposal

Observation 1: “List of Satellite IDs” as concluded in TR 23.700-29 for the split-MME and “S&F Monitoring list” as concluded in TR 23.700-29 for the full EPC onboard architecture are not intended for the same purpose.

Observation 2:  For the split-MME architecture, the “List of Satellite IDs” is mainly intended to indicate to the UE which are the subset of satellites of the selected PLMN from which the UE is expected to receive service in S&F mode.

Observation 3: For the full EPC onboard architecture, the “S&F Monitored List” is mainly intended to assist UEs in retrieving MT data.

Observation 4: Both intentions (i.e. (1) establish the set of satellites of the selected PLMN from which the UE is expected to receive service in S&F mode and (2) indicate which of those serving satellites will be used by the network to serve MT traffic) are relevant and do not exclude each other.

Proposal 1: Consider the specification of two satellite lists for S&F mode operation:

| Information Element | Purpose | Comment |
| --- | --- | --- |
| S&F Serving Satellites List | If provided along with a “Reject cause due to S&F operation” during a NAS procedure, the UE can attempt to retry the rejected NAS procedure in any of the satellites of the selected PLMN listed in the “S&F Serving Satellites List” not later than a validity timer. If provided during a successful NAS procedure, a UE should consider for subsequent NAS procedures only the satellites of the selected PLMN listed in the “S&F Serving Satellites List”. A “S&F Serving Satellites List” is valid as long as the UE registration remains valid. If not provided, the UE could interact with any satellite of the selected PLMN. | For the split-MME, this is applicable to an scenario where a “single MME” is not deployed in all the satellites of the constellation. For the full EPC onboard, this could be applicable to a scenario where UE credentials are not provisioned in all the satellites of the constellation. |
| S&F Monitoring Satellites List | If provided during a successful NAS procedure, UE should only expect MT traffic to be delivered through the satellites of the selected PLMN listed in the “S&F Monitoring Satellites List”. If provided together with the “S&F Serving Satellites List”, the “S&F Monitoring Satellites List” should be a subset of the former. If not provided, the UE should expect MT traffic in any of the satellites listed in “S&F Serving Satellites List” or, if the latter is not provided either, in any of the satellites of the selected PLMN. |  |

Proposal 2: Decide a solution among the following options:

1) Use separate list, as in Proposal 1 above.

2) Use common list and have an indication whether it is a mandatory or advisory.

3) Use common list and use term “should” for UE behaviour.