SA WG2 Meeting #166 	S2-2412209

# 18 - 22 Nov 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5113 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UPF event exposure service at N4 session release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia, ETRI, ZTE,] Samsung, [Huawei, HiSilicon] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are several issues in existing specification - UPF event exposure service handling at PDU session release was integrated including UPF relocation case and was accepted in TS23.501 at the last meeting. However, the related procedure was not included in TS23.502 yet. - In clause 4.15.4.5.6 the Relocation report indication is provided to UPF in the unsubscribe request, while in clause 4.15.4.5.7 the Relocation report indication is provided to UPF in the Subscribe request. It is proposed to provide the Relocation report indication in the Subscribe request. - According to clause 5.8.2.17 of TS 23.501, if the Analytics filter information includes the Area of Interest or DNAI, the consumer shall not use direct subscription to UPF event. The clause 4.15.4.5.7 needs to be corrected to align with 23.501. - The new parameters are missing in the service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PDU session release and UPF relocation are need to be integrated to N4 session release. One Subscription termination reporting indication parameter is used instead of Relocation report indication and Subscription termination indication. - clarify that the SMF provide the Subscription termination reporting indication in the Subscribe request to the UPF and remove Editor’s Note - remove the parameters not used in clause 4.15.4.5.7 - Add new parameters in the SMF/UPF service operation - Change the Figure for clarification and simplification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment within the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.6, 4.15.4.5.7, 5.2.8.3.3, 5.2.26.2.2, 5.2.26.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

##### 4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF when N4 session is released

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF when N4 session is released

This information flow is used in the scenario of PDU session release or UPF relocation for the existing PDU Session, e.g. the source UPF is I-UPF or local PSA UPF which is relocated, or the source UPF is PSA UPF which is relocated in SSC mode 3. Case when SMF relocation happens is not specified in this release.

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE. The consumer may include a Subscription termination reporting indication, which indicates to receive a subscription termination notification when N4 session is released.

2.	The SMF subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation or N4 Session Modification with PFCP depending on UPF exposure event and which mechanism applies for each event as described in clause 5.2.26.2.1. The SMF may include the Subscription termination reporting indication if received from the UPF event consumer.

3.	The source UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.	The SMF determines that the N4 session of the UPF needs to be released (e.g. UPF relocation or PDU session release).

5.	The SMF unsubscribes to the source UPF by invoking Nupf_EventExposure_Unsubscribe service operation operation and releases the N4 session with the source UPF providing the reason of the release.

6.	The UPF may report to the consumer any data that has been collected but not sent to the consumer yet. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. Or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet .

7.	In case of PDU Session release the SMF may send a subscription termination indication to the UPF event consumer by invoking Nsmf_EventExposure_Notify service operation and may also include a cause code indicating the reason for subscription termination, e.g., N4 session release. The procedure terminates at this step.

8-10.	In case of UPF relocation step 7 is omitted and to continue the subscription of the UPF event consumer, if the UPF related information is matched with the subscription request in step 1 and after N4 session establishment with target UPF, SMF may re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. The target UPF will send the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

##### 4.15.4.5.7	Information flow for subscription directly to UPF event exposure service when N4 session is released

Figure 4.15.4.5.7-1: Subscription directly to UPF event exposure service when N4 session is released

1.	If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2], steps 1-2 as described in Figure 4.15.4.5.5-1 are performed.

2.	The UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to the discovered UPF, and the Target of Event Reporting contains a single UE. The consumer may include a Subscription termination reporting indication, which indicates to receive subscription termination notification when the N4 session of the UPF is released (e.g., due to UPF relocation or PDU session release).

3.	The UPF invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

4.	When the N4 session corresponding to the UE is released,, the UPF determines that the subscription to the UPF event has been terminated.

5.	If step 2 included Subscription termination reporting indication, the UPF sends the notification to the UPF event consumer by invoking Nupf_EventExposure_Notify service operation, which includes a subscription termination indication to notify that the subscription to the UPF event has been terminated, and optionally include a cause code specifying the reason for subscription termination, e.g. UPF relocation or PDU session release. The UPF may report to the consumer any data that has been collected but not sent to the consumer yet.

6. New N4 session corresponding to the UE is established to the Target UPF.

7.The UPF event consumer (e.g., NWDAF) may re-subscribe to UPF event to the target UPF as in the steps 1-3.

* * * Next Change * * *

##### 5.2.8.3.3	Nsmf_EventExposure_Subscribe service operation

Service operation name: Nsmf_EventExposure_Subscribe.

Description: This service operation is used by an NF to subscribe or modify a subscription for event notifications on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE.

Input, Required: NF ID, Target of Event Reporting as defined in clause 5.2.8.3.1, (set of) Event ID(s) defined in clause 5.2.8.3.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Input, Optional: Event Filter(s) associated with each Event ID; Event Filter(s) are defined in clause 5.2.8.3.1, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, DNAI, UPFId, UPF event exposure information (Type of measurement, granularity of measurement, reporting suggestion information, Subscription termination reporting indication, etc. associated with the UPF Event IDs as described in clause 4.15.4.5).

NOTE:	The SMF is generally meant to determine the UPF to contact for a subscription related to UPF event exposure. UPF ID is only provided to indicate a UP Path as defined in Table 6.4.1-1 of TS 23.288 [50], i.e. when the NWDAF has received the target UPF as part of statistics of observed service experience on an UP path involving that UPF.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1).

Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by SMF or UPF with this subscription.

* * * Next Change * * *

##### 5.2.26.2.2	Nupf_EventExposure_Notify service operation

Service operation name: Nupf_EventExposure_Notify

Description: This service operation reports the event and information to the consumer that has subscribed (implicitly or explicitly).

Input Required: Event ID, UE address (i.e. IP address or MAC address), Notification Correlation Information.

Input, Optional: UE ID, Event specific parameters as described in clause 5.2.26.2.1, time stamps for the measurements, Application Id(s), Packet Filter Set(s), Achieved sampling ratio, Subscription termination indication (with optionally a Cause code).

Output Required: Result Indication.

Output, Optional: None.

##### 5.2.26.2.3	Nupf_EventExposure_Subscribe service operation

Service operation name: Nupf_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe or modify a subscription to UPF event exposure notifications e.g. for the purpose of UPF data collection on a specified PDU Session or for all PDU Sessions of one UE or any UE.

Input, Required: NF ID, Target of Event Reporting "any UE" or as defined in Table 4.15.4.5.1-1 when a specific UE is targeted), (set of) Event ID(s) defined in clause 5.2.26.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1 (TS 29.564 [93] defines the possible parameters).

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information, Type of Measurement, Granularity of Measurement, Reporting suggestion information, Subscription termination reporting indication, UE (private) IP address, IP domain, IP address of the remote end, port number of the remote end.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by UPF with this subscription.

* * * End of Change * * *