3GPP TSG-SA2 Meeting #166	S2-2412296

# Orlando, US, 18 – 22 November, 2024														(was S2-2410989)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3801 | rev | 21 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Store and Forward feature in EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia, Sateliot, Qualcomm, LGE,] Intel, [OPPO, SHARP, Google, CATT, Novamint, NEC, Tencent, Vivo] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARC |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved 5GSAT_ARCH_Ph3 work item (SP-240986) is set to specify the architecture enhancements, functionalities and procedures to support Store and Forward operation mode based on conclusions of TR 23.700-29 (clause 8.2). This paper adds the general description of the Store and Forward feature and corresponding architecture enhancements in TS 23.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduction of Store and Forward Architectural options. 2. Generic principles of Store and Forward Satellite operation. Revision 21 changes (compared to S2-2410989): Added clarifications on the timestamp handling at the HSS that were discussed during the SA2#165 meeting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support of Store and Forward feature in 5GSAT_ARCH_Ph3 in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, (new) 4.13.X, (new) 4.13.X.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# FIRST CHANGE

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

MME Pool Area: An MME Pool Area is defined as an area within which a UE may be served without need to change the serving MME. An MME Pool Area is served by one or more MMEs ("pool of MMEs") in parallel. MME Pool Areas are a collection of complete Tracking Areas. MME Pool Areas may overlap each other.

Serving GW Service Area: A Serving GW Service Area is defined as an area within which a UE may be served without need to change the Serving GW. A Serving GW Service Area is served by one or more Serving GWs in parallel. Serving GW Service Areas are a collection of complete Tracking Areas. Serving GW Service Areas may overlap each other.

PDN Connection: The association between a PDN represented by an APN and a UE, represented by one IPv4 address and/or one IPv6 prefix (for IP PDN Type) or by the UE Identity (for Non-IP and Ethernet PDN Types).

Default Bearer: The EPS bearer which is first established for a new PDN connection and remains established throughout the lifetime of the PDN connection.

Default APN: A Default APN is defined as the APN which is marked as default in the subscription data and used during the Attach procedure and the UE requested PDN connectivity procedure when no APN is provided by the UE.

eCall Only Mode: A UE configuration option that allows the UE to attach at EPS and register in IMS to perform only eCall Over IMS, and an IMS call to a non-emergency MSISDN or URI for test and/or terminal reconfiguration services. For a short period following either such call, an incoming call (e.g. callback from a PSAP or HPLMN operator) or other incoming session (e.g. for USIM reconfiguration) is possible. At other times when the UE is configured in this mode, the UE is required to refrain from any signalling to a network. Use of eCall Only Mode is configured in the USIM for the UE.

PDN Connection to the SCEF: The association between a UE, represented by the UE Identity, and a PDN represented by an APN to external packet data network via SCEF to allow transfer of Non-IP data. It includes establishment and persistence of T6 connection between MME and SCEF (see TS 29.128 [79]).

Emergency attached UE: A UE which only has bearer(s) related to emergency bearer service.

NOTE 1:	The above term is equivalent to the term "attached for emergency bearer services" as specified in TS 24.301 [46].

LIPA PDN connection: a PDN Connection for local access (e.g. for IP or Ethernet access) for a UE connected to a HeNB.

en-gNB: As defined in TS 37.340 [85].

SIPTO at local network PDN connection: a PDN connection for SIPTO at local network for a UE connected to a (H)eNB.

Correlation ID: For a LIPA PDN connection, Correlation ID is a parameter that enables direct user plane path between the HeNB and L-GW.

SIPTO Correlation ID: For a SIPTO at local network PDN connection, SIPTO Correlation ID is a parameter that enables direct user plane path between the (H)eNB and L-GW when they are collocated.

Local Home Network: A set of (H)eNBs and L-GWs in the standalone GW architecture, where the (H)eNBs have IP connectivity for SIPTO at the Local Network via all the L-GWs.

Local Home Network ID: An identifier that uniquely identifies a Local Home Network within a PLMN.

Presence Reporting Area: An area defined within 3GPP Packet Domain for the purposes of reporting of UE presence within that area due to policy control and/or charging reasons. In the case of E-UTRAN, a Presence Reporting Area may consist in a set of neighbor or non-neighbor Tracking Areas, or eNodeBs and/or cells. There are two types of Presence Reporting Areas: "UE-dedicated Presence Reporting Areas" and "Core Network pre-configured Presence Reporting Areas" that apply to an MME pool.

RAN user plane congestion: RAN user plane congestion occurs when the demand for RAN resources exceeds the available RAN capacity to deliver the user data for a prolonged period of time.

NOTE 2:	Short-duration traffic bursts is a normal condition at any traffic load level, and is not considered to be RAN user plane congestion. Likewise, a high-level of utilization of RAN resources (based on operator configuration) is considered a normal mode of operation and might not be RAN user plane congestion.

IOPS-capable eNodeB: an eNodeB that has the capability of IOPS mode operation, which provides local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs via a Local EPC when the eNodeB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNodeBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNodeBs in IOPS mode of operation use, instead of the Macro EPC, in order to support public safety services.

Macro EPC: the EPC which serves an eNodeB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNodeB(s)), local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs in the absence of normal EPS

Multi-USIM UE: a UE with multiple USIMs, capable of maintaining a separate registration state with a PLMN for each USIM at least over 3GPP Access and supporting one or more of the features described in clause 4.3.33.

IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

Cellular IoT: Cellular network supporting low complexity and low throughput devices for a network of Things. Cellular IoT supports IP, Ethernet and Non-IP traffic. Unless otherwise stated in this specification, Cellular IoT and all functionality applicable to Cellular IoT also apply to satellite access.

Narrowband-IoT: a 3GPP Radio Access Technology that forms part of Cellular IoT. It allows access to network services via E-UTRA with a channel bandwidth limited to 180 kHz (corresponding to one PRB). Unless otherwise indicated in a clause, Narrowband-IoT is a subset of E-UTRAN. Unless otherwise stated in this specification, Narrowband-IoT also includes satellite access.

LTE-M: a 3GPP RAT type Identifier used in the Core Network only, which is a sub-type E-UTRAN RAT type, and defined to identify in the Core Network the E-UTRAN when used by a UE indicating Category M in its UE radio capability. Unless otherwise stated in this specification, LTE-M also includes satellite access.

WB-E-UTRAN: in the RAN, WB-E-UTRAN is the part of E-UTRAN that excludes NB-IoT. In the Core Network, the WB-E-UTRAN also excludes LTE-M. Unless otherwise stated in this specification, WB-E-UTRAN also includes satellite access.

DCN-ID: DCN identity identifies a specific dedicated core network (DCN).

For the purposes of the present document, the following terms and definitions given in TS 23.167 [81] apply:

eCall Over IMS: See TS 23.167 [81].

RLOS attached UE: A UE is attached only for accessing Restricted Local Operator Services (see TS 23.221 [27]).

IAB-donor: For the purposes of this specification, this is a NR Secondary RAN node is further described in TS 37.340 [85] that supports Integrated access and backhaul (IAB) feature and provides connection to the core network to IAB-nodes. It supports the CU function of the CU/DU architecture for IAB defined in TS 38.401 [90].

IAB-node: A relay node that supports wireless in-band and out-of-band relaying of NR access traffic via NR Uu backhaul links. It supports the UE function and the DU function of the CU/DU architecture for IAB defined in TS 38.401 [90].

Feeder link: as defined in TS 36.300 [6].

Service link: as defined in TS 36.300 [6]

Store and Forward Satellite operation: An operation mode that provides to the UE a communication service when the serving satellite has a discontinuous connection to the ground network and such connection is not available when the satellite is interacting with the UE.

S&F Mode: The mode in which the UE, RAN and core network entities perform Store and Forward Satellite operation.

# SECOND CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GS	5G System

AF	Application Function

ARP	Allocation and Retention Priority

AMBR	Aggregate Maximum Bit Rate

CBC	Cell Broadcast Centre

CBE	Cell Broadcast Entity

CIoT	Cellular IoT

CSG	Closed Subscriber Group

CSG ID	Closed Subscriber Group Identity

C-SGN	CIoT Serving Gateway Node

CSS	CSG Subscriber Server

DAPS	Dual Active Protocol Stacks

DCN	Dedicated Core Network

DeNB	Donor eNode B

DL TFT	DownLink Traffic Flow Template

DRX	Discontinuous Reception

ECGI	E-UTRAN Cell Global Identifier

ECM	EPS Connection Management

ECN	Explicit Congestion Notification

EMM	EPS Mobility Management

eNodeB	evolved Node B

EPC	Evolved Packet Core

EPS	Evolved Packet System

E-RAB	E-UTRAN Radio Access Bearer

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

GBR	Guaranteed Bit Rate

GUMMEI	Globally Unique MME Identifier

GUTI	Globally Unique Temporary Identity

GW	Gateway

HeNB	Home eNode B

HeNB GW	Home eNode B Gateway

HFN	Hyper Frame Number

IAB	Integrated Access and Backhaul

IMEI/TAC	IMEI Type Allocation Code

IOPS	Isolated E-UTRAN Operation for Public Safety

IoT	Internet of Things

ISR	Idle mode Signalling Reduction

LAA	Licensed Assisted Access

LBI	Linked EPS Bearer Id

L-GW	Local GateWay

LIPA	Local IP Access

LWA	LTE/WLAN Aggregation

LWIP	LTE/WLAN Radio Level Integration with IPsec Tunnel

MBR	Maximum Bit Rate

MME	Mobility Management Entity

MMEC	MME Code

MTC	Machine-Type Communications

MT-EDT	Mobile Terminated Early Data Transmission

M-TMSI	M-Temporary Mobile Subscriber Identity

NB-IoT	Narrowband IoT

NR	New Radio

NR-U	New Radio Unlicensed

NTN	Non-Terrestrial Network

OCS	Online Charging System

OFCS	Offline Charging System

OMC-ID	Operation and Maintenance Centre Identity

P-GW	PDN Gateway

PCC	Policy and Charging Control

PCRF	Policy and Charging Rules Function

PRA	Presence Reporting Area

PDCP	Packet Data Convergence Protocol

PMIP	Proxy Mobile IP

PSAP	Public Safety Answering Point

PSM	Power Saving Mode

PTI	Procedure Transaction Id

QCI	QoS Class Identifier

RACS	UE Radio Capability Signalling optimization

RCAF	RAN Congestion Awareness Function

RFSP	RAT/Frequency Selection Priority

RLOS	Restricted Local Operator Services

RN	Relay Node

RUCI	RAN User Plane Congestion Information

S&F	Store and Forward

S-GW	Serving Gateway

S-TMSI	S-Temporary Mobile Subscriber Identity

SDF	Service Data Flow

SIPTO	Selected IP Traffic Offload

TAC	Tracking Area Code

TAD	Traffic Aggregate Description

TAI	Tracking Area Identity

TAU	Tracking Area Update

TI	Transaction Identifier

TIN	Temporary Identity used in Next update

UCMF	UE radio Capability Management Function

URRP-MME	UE Reachability Request Parameter for MME

UL TFT	UpLink Traffic Flow Template

ULR-Flags	Update Location Request Flags

# THIRD CHANGE

## 4.13	Introduction of satellite support for Cellular IoT

### 4.13.x	Support of Store and Forward Satellite Operation

#### 4.13.x.1	General

The Store and Forward Satellite Operation applies in E-UTRAN with satellite access and is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc). If the satellite does not support Store and Forward Satellite operation and there is no feeder link connection to the ground network, then the satellite cannot provide any service to any UEs. When both the service link and feeder link are available, all services can be provided. To support Store and Forward Satellite operation, some network functionality needs to be deployed on the satellite payload.

Example deployments are described in Annex X

Figure 4.13.x.1-1: Store and Forward (S&F) Satellite operation

NOTE 1:	In the Figure 4.13.x.1-1, the NTN GW belongs to the satellite deployment and is not subject to 3GPP specifications.

In Store and Forward Satellite operation, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the ground link when the satellite can exchange data with the core network.  This is depicted in Fig. 4.13.x.1.1.

For example, a simple sequence of events for the transmission of data from the UE to a server on the ground may involve two steps: firstly, when service link is available (e.g., location L1 in Fig. 4.13.x.1.1) the UE sends the data to the satellite. Subsequently (e.g., location L2 in Fig. 4.13.x.1.1) the satellite carrying the payload connects to the ground network and delivers the UE data to the network.

Downlink data can be stored onto the same or a different satellite and provided to the UE later using the first step, and the process is repeated.

When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the onboard eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5] which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode, and if a UE is Enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

NOTE 2:	Some deployments cannot support all system features, for example they cannot support user plane establishment and user plane data transfer, when operating in S&F Mode in the split MME deployment described in Annex X.

The MME may adapt periodic update timer and Implicit Detach timer to the fact that the UE is served in S&F mode

Editor’s Note: S&F Monitoring List related aspects are FFS

If the UE did not indicate Store and Forward capability in NAS signalling, if the network determines to reject the UE, the network shall reject the UE request with a cause non-specific to S&F Mode

The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

Editor’s Note: following text from the TR conclusions needs to be clarified:	The estimated delivery time is the estimated time to send data from the UE to Gateway

The MME may trigger a Location Update procedure with the HSS along with the authentication procedure to fetch subscription information from the HSS. The MME may indicate the timestamp information to HSS during the Location Update procedure. This timestamp information shall be used by the HSS to ensure that newer location for that UE are not cancelled. If MME makes an Update Location Request before the completion of the authentication procedure, it shall include an indication that this Location Update is provisional, i.e. the HSS shall not consider the UE as registered until it receives the final Update Location Request (without indication that the Location Update is provisional). The timestamp information is the time when the on-board of satellite MME part has received the NAS procedure from the UE. The HSS compares the timestamp received in the Update Location Request with any stored timestamp of a previous Update Location Request and determines whether to accept or reject the request. If the received Update Location Request does not include a timestamp, the HSS assumes the present time as the timestamp of the received Update Location Request. The HSS shall reject the Update Location Request, if the timestamp associated with this request is older than the stored timestamp. If the HSS accepts the Update Location Request, the HSS shall store the timestamp associated with the latest Update Location Request. If the HSS does not support the timestamp, the MME shall reject an UE Attach request for S&F Mode.

NOTE 7:	The timestamp information is used by the HSS to ensure that interactions with MMEs for a UE are handled in the correct order.

NOTE X:	The absence of timestamp in the Update Location Request implies that the UE is connecting from a network that does not operate in S&F mode.

The EPS may expose whether a UE is in S&F Mode, and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE.  This is further described in clause 5.6.3.x of TS 23.682 [74].

# End of CHANGEs