3GPP TSG-WG2 Meeting #166	S2-2412556

# Orlando, USA, 2024-11-18 – 2024-11-22

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2131 | rev | 7 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying devices behind 5G-RG via DHCPv6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conclusions for work item FS_UIA_ARC Rel 19 has been made in 3GPP meeting#163 and. This CR is part of normative work to include agreed features regarding 5G-RG and DHCPv6 in the specifications as per TR conclusion: “For non-3GPP devices requiring QoS differentiation, UE/5G-RG may also send the Device Identifier of the non-3GPP device to the SMF in DHCPv6 message to associate the Device Identifier with the user plane address.” The user plane address is defined in the conclusion as: “When using an IP PDU Session Type, the user plane address is the UE IP Address(es) / Port Number(s) that carry the non-3GPP device's traffic.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes have been made : - General overview is provided in a new section 4.10x - Complementry explanation is added to the IP address allocation section - the procedure is clarified in a new section 7.3.8.x Furthermore, compared to the previous version, the following changes have been made: - The scope of the solution is limited to scenarios where devices interact with 5G-RG with DHCPv6 and 5G-RG only relays the device’s DHCPv6 messages, which minimizes the impact to 5G-RG. - The text related to DUID is removed since in this version the 5G-RG simply relays the DHCPv6 message received from the non-3GPP device. - In the procedure, step 8 is removed since no configuration change is needed to assign the IPv6 address to the device (the 5G-RG only relays SMF message to the device) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications not aligned with study conclusions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.6.2.2, 4.10x (new), 7.3.8.x (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The referenced clause 5.x of 23.501 and the referenced clause 6.1.3.x of TS 23.503 refere to functional description of QoS provisioning for devices behind UE/5G-RG, which are expected to be icluded by CRs from other companies. The reference clause 6.1.3.5 of TS 23.503 refer to the enhancement of PCRT added by the CR S2-2411925. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[6]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[7]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[8]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[9]	BBF TR-456 issue 2: "AGF Functional Requirements".

[10]	BBF WT-457: "FMIF Functional Requirements".

NOTE:	Technical Report of BBF WT-457 will be TR-457 which will be available when finalized by BBF.

[11]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[12]	BBF TR-177 Issue 1 Corrigendum 1: "IPv6 in the context of TR-101".

[13]	IETF RFC 6788: "The Line-Identification Option".

[14]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[15]	Void.

[16]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[17]	Void.

[18]	BBF TR-069: "CPE WAN Management Protocol".

[19]	BBF TR-369: "User Services Platform (USP)".

[20]	IETF RFC 3046: "DHCP Relay Agent Information Option".

[21]	IETF RFC 4604: "Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast".

[22]	3GPP TR 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 38.413: "NG RAN; NG Application Protocol (NGAP)".

[24]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[25]	3GPP TS 22.011: "Service accessibility".

[26]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[28]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[29]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[30]	BBF TR-198: "DQS:DQM systems functional architecture and requirements".

[31]	3GPP TS 23.203: "Policy and charging control architecture".

[32]	3GPP TS 33.126: "Lawful Interception Requirements".

[33]	IETF RFC 2236: "Internet Group Management Protocol, Version 2".

[34]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[35]	IETF RFC 1112: "Internet Group Management Protocol".

[36]	IETF RFC 2710: "Multicast Listener Discovery Version for IPv6".

[37]	IETF RFC 2010: "Operational Criteria for Root Name Servers".

[38]	BBF TR-470: "5G FMC architecture".

[39]	3GPP TS 29.519: "Policy Data, Application Data and Structured Data for exposure".

[40]	3GPP TS 23.041: "Public Warning System".

[41]	IEEE Publication (2017): "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)". https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[42]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[43]	Void.

[44]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via non-3GPP access networks".

[45]	3GPP TS 23.402: " Architecture enhancements for non-3GPP accesses".

[46]	BBF TR-181: "Device Data Model for TR-069".

[47]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[XX]	IETF RFC 4649: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6) Relay Agent Remote-ID Option".

# * * * * Second change * * * *

#### 4.6.2.2	IPv6 Address Allocation using DHCPv6

Optionally, and instead of using Stateless IPv6 Address Autoconfiguration, individual 128-bit IPv6 address(es) may be assigned to a PDU Session.

In this case, after PDU Session Establishment, the SMF sends a Router Advertisement message (solicited or unsolicited) towards the RG. The SMF shall set the Managed Address Configuration Flag (M-flag) in the Router Advertisement messages to indicate towards the RG that IPv6 Address allocation using DHCPv6 is available, as described in RFC 4861 [34]. In that case the IPv6 address of the RG is allocated using DHCPv6 Identity Association for Non-temporary Addresses (IA_NA) and mechanisms defined in RFC 8415 [47].

The SMF may receive a Router Solicitation message, soliciting a Router Advertisement message.

When using DHCPv6 address allocation, a prefix (e.g. /64) may be allocated for the PDU Session at PDU Session Establishment from which the /128 addresses are selected. The SMF determines the size of the prefix for a PDU Session to a specific DNN and S-NSSAI based on subscription data and local configuration. The individual /128 address(es) allocated to the RG as part of DHCP IA_NA procedure are then selected from the prefix allocated to the PDU Session. For statically assigned prefix, the subscription data in UDM for a DNN and S-NSSAI includes the prefix. Alternatively, individual 128-bit address(es) are allocated for the PDU Session without allocating a prefix to the PDU Session and provided to the RG as part of DHCP IA_NA procedure.

When a prefix is allocated to the PDU Session, the SMF provides the prefix to the PCF instead of each /128 address. When individual /128 address(es) are allocated without allocating a prefix to the PDU Session, the SMF provides the /128 bits address(es) to PCF. Whether the SMF allocates a prefix for the PDU Session or individual 128-bit addresses is transparent to the RG and W-5GAN.

If Prefix Delegation (as described in clause 4.6.2.3) is also supported, a SMF may receive both DHCP options for IA_NA and IA_PD together in a single DHCPv6 message. An SMF may provide a reply to both IA_NA and IA_PD in the same message or alternatively process the DHCPv6 IA_NA before the DHCPv6 IA_PD.

The SMF may receive multiple different IA_NA related DHCP requests within the same PDU Session.

NOTE 1:	This is applicable if the RG acts as a DHCP relay for devices behind the RG.

To enable QoS differentiation in 5GS for a device behind a 5G-RG that utilizes DHCPv6, the 5G-RG can act as a DHCPv6 relay agent and relay IA_NA related DHCPv6 solicit, rebind, or renew messages to SMF.

The solicit message may contain IA Address options, including preferred addresses, encapsulated within IA_NA. The SMF allocates IPv6 address to the device based on the received message.

NOTE 2:	The returned IPv6 address can be different than what the 5G-RG provided as per DHCPv6 specification IETF RFC 8415 [47].

The 5G-RG includes the Non-3GPP Device Identifier in the relayed DHCPv6 messages as described in clause 4.10x. The SMF associates the Non-3GPP Device Identifier with the non-3GPP device address.

NOTE 3:	How the 5G-RG interacts with the non-3GPP device behind the 5G-RG is out of scope of this specification.

When IPv6 Address Allocation using DHCPv6 is used, 5GC does not support IPv6 multi-homing for enabling SSC mode 3 and PDU Sessions with multiple PDU Session Anchors.

# * * * * Third change * * * *

## 4.10x	Identification of Non-3GPP Devices Behind 5G-RG using DHCPv6

This clause only applies to scenarios where devices behind 5G-RG use DHCPv6 to obtain their IPv6 address and 5G-RG acts as a DHCPv6 relay agent for the devices behind it. To identify and provide differentiated QoS in 5GC for non-3GPP devices behind the 5G-RG using DHCPv6, the 5G-RG includes the Non-3GPP Device Identifier (see clause 5.x of TS 23.501 [2]) in the DHCPv6 relay-forward message that encapsulates the device’s solicit, renew, or rebind message. The Non-3GPP Device Identifier is included via the INTERFACE-ID option as described in RFC 8415 [47] or REMOTE_ID option as described in RFC 4649 [XX].

5G-RG shall only use the INTERFACE-ID option and REMOTE_ID option for requesting QoS differentiation for devices behind 5G-RG when using DHCPv6.

Upon receiving the DHCPv6 relay-forward message that includes a solicit message with the Non-3GPP Device Identifier, the SMF may allocate the Non-3GPP device a IPv6 address based on the DHCPv6 message as described in clause 4.6.2. The SMF shall consider the IA Address values for allocation of IPv6 address for the device, if they are included in the IA_NA option.

If an IPv6 address is already assigned to the non-3GPP device, the 5G-RG can provide QoS differentiation when the device sends a DHCPv6 renew or a rebind message..

For either case (solicit, rebind, or renew), the SMF forwards to the PCF the Non-3GPP Device Identifier and IPv6 address of the Non-3GPP Device. The PCF uses this to create/update PCC rules for the Non-3GPP Device as described in clause 6.1.3.x of TS 23.503 [4]. The procedure for identifying non-3GPP devices behind the 5G-RG using DHCPv6 is described in clause 7.3.x.

In addition to 5G-RG, this clause also applies to UEs that support DHCPv6 relaying option.

This is only applicable to IPv6 PDU sessions.

# * * * * Fourth change * * * *

#### 7.3.x	IPv6 Session Management Procedures for Identifying Non-3GPP  Devices Behind 5G-RG using DHCPv6

This clause describes the procedure to enable 5GC to identify the non-3GPP devices behind a 5G-RG using DHCPv6 messaging.

Figure 7.3.x-1: Identifying device behind RG.

0.	The Non-3GPP Device Identifier Information is provided in the UDR by an AF as described in clause 5.x of TS 23.501 [2].

1.	The Device is connected to the 5G-RG sends a DHCPv6 solicit, renew, or rebind message..

2.	The 5G-RG either establishes a new IPv6 PDU session or use an existing one. The RG decides on this either based on configuration or the URSP rules.

3.	The 5G-RG decides that the Non-3GPP Device needs QoS differentiation. Then it acts as a DHCPv6 relay agent and creates a DHCPv6 relay-forward message encapsulating the received DHCPv6 message in step 1 and includes the Non-3GPP Device Identifier via INTERFACE-ID option as described in RFC 8415 [47] or REMOTE_ID option as described in RFC 4649 [XX].

In case of solicit message, the 5G-RG may also provide a preferred IPv6 address of the device in the message.

4.	The 5G-RG sends the DHCPv6 message to SMF via User Plane as per current specifications.

5.	The SMF obtains the Non-3GPP Device Identifier from the DHCPv6 relay-forward message. If the relay-forward message contains a solicit message, the SMF allocates IPv6 Address to the device and considers the IA Address values if they are included in the IA_NA options in the DHCPv6 relay-forward message. For both solicit and renew messages, the SMF associates the IPv6 address with the non-3GPP Device Identifier.

6.	The SMF initiates SM policy association modification request, which includes the Non-3GPP Device Identifier and the IPv6 Address of the non-3GPP device as described in clause 6.1.3.5 of TS 23.503 [4]. The PCF obtains the Non-3GPP Device Identifier Information stored in the UDR. The PCF updates the PCC rules for the 5G-RG based on the Non-3GPP Device Information in UDR. For further information, see TS 23.503 [4] clause 6.1.3.x

Editor’s Note: depending on how max number of identifiable devices are enforced, the text in step 6 may need to be updated.

7.	The SMF sends the DHCPv6 relay-reply message to 5G-RG and 5G-RG relays them to the non-3GPP device. If four messages are used for allocating the IPv6 address, at this step the non-3GPP device and SMF exchange DHCPv6 advertise, request and reply messages.

8.	The SMF initiates a PDU session modification procedure due to new PCC rules.

9.	Traffic to and from the device is identified and associated with the Non-3GPP Device Identifier and gets QoS differentiation.

# * * * * End of changes * * * *