3GPP TSG-WG SA2 Meeting #166	S2-2412389

# Orlando, USA, November 18 - 22, 2024										   (revision of S2-2412229)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5785 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Time to Next Burst provisionsing to NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification of XRM_Ph2 conclusions on the KI#5. Support of Time to Next Burst provisioning to NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following enhancements are introduced: - Protocol Description in PDRs is also used to assist Time to Next Burst value identification. - QER is enhanced to include the Time to Next Burst in the GTP-U header in the last PDU(s) of a data burst. - 5.37.x.y new clause to describe the Time to Next Burst provisioning to the NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Time to Next Burst provisioning to NG-RAN is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.5.3, 5.8.5.4, 5.37.x.y (new), 6.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.8.5.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.5.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR. NOTE 5. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied. |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| Detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3). | packet filter set, application identifier, Ethernet PDU Session |
| Information. NOTE 4. | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet. | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application identifier |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI. | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN), or from "5G VN internal" (i.e. local switch). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14. | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
|  | FQDN Filter for DNS Query | Contains one or more FQDN, FQDN range, and/or any FQDN. | The FQDN or FQDN range only used for detection of plain DNS Query message (i.e. not subject to ciphering). The usage is described in TS 23.548 [130]. |
|  | Protocol Description | Indicates service protocol used by the flow (NOTE 8). |  |
| Packet replication and detection carry on information | Packet replication skip information NOTE 7 | Contains UE address indication or N19/N6 indication. If the packet matches the packet replication skip information, i.e. source address of the packet is the UE address or the packet has been received on the interface in the packet replication skip information, the UP function neither creates a copy of the packet nor applies the corresponding processing (i.e. FAR, QER, URR). Otherwise the UPF performs a copy and applies the corresponding processing (i.e. FAR, QER, URR). |  |
| NOTE 6 | Carry on indication | Instructs the UP function to continue the packet detection process, i.e. lookup of the other PDRs. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. - UPF "local switch" may be used for DNN/S-NSSAI dedicated for PIN. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4: When in the architecture defined in clause 5.34, a PDR is sent over N16a from SMF to I-SMF, the Packet Detection Information may indicate that CN tunnel info is to be locally determined. This is further defined in clause 5.34.6. NOTE 5: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 6: Needed in the case of support for broadcast/multicast traffic forwarding using packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2. NOTE 7: Needed in the case of packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2, to prevent UPF from sending the broadcast/multicast packets back to the source UE or source N19/N6. NOTE 8: Not for PDR matching. It may be provided to assist PDU Set identification when PDU Set Identification and marking applies to the PDR and/or to assist identification of the last packet of the Data burst in downlink when End of Data Burst identification and marking in downlink applies to the PDR. It may assist determination of the Time to Next Burst when Time to Next Burst determination and marking in downlink applies to the PDR. See TS 26.522 [179]. |  |  |  |

# * * * * Next change * * * *

#### 5.8.5.4	QoS Enforcement Rule

The following table describes the QoS Enforcement Rule (QER) that defines how a packet shall be treated in terms of bit rate limitation and packet marking for QoS purposes. All Packet Detection Rules that refer to the same QER share the same QoS resources, e.g. MFBR.

Table 5.8.5.4-1: Attributes within QoS Enforcement Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this QER |  |
| Rule ID | Unique identifier to identify this information. |  |
| QoS Enforcement Rule correlation ID (NOTE 1) | An identity allowing the UP function to correlate multiple Sessions for the same UE and APN. | Is used to correlate QoS Enforcement Rules for APN-AMBR enforcement. |
| Gate status UL/DL | Instructs the UP function to let the flow pass or to block the flow. | Values are: open, close, close after measurement report (for termination action "discard"). |
| Maximum bitrate | The uplink/downlink maximum bitrate to be enforced for the packets. | This field may e.g. contain any one of: - APN-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of all PDN Connections to an APN) (NOTE 1). - Session-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of the PDU Session) - QoS Flow MBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - SDF MBR (for a QER that is referenced by the uplink/downlink Packet Detection Rule of a SDF) - Bearer MBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Guaranteed bitrate | The uplink/downlink guaranteed bitrate authorized for the packets. | This field contains: - QoS Flow GBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - Bearer GBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Averaging window | The time duration over which the Maximum and Guaranteed bitrate shall be calculated. | This is for counting the packets received during the time duration. |
| Down-link flow level marking | Flow level packet marking in the downlink. | For UPF, this is for controlling the setting of the RQI in the encapsulation header as described in clause 5.7.5.3. |
| QoS Flow ID | QoS Flow ID to be inserted by the UPF. | The UPF inserts the QFI value in the tunnel header of outgoing packets. |
| Paging Policy Indicator | Indicates the PPI value the UPF is required to insert in outgoing packets (see clause 5.4.3.2). | PPI applies only for DL traffic. The UPF inserts the PPI in the outer header of outgoing PDU. |
| Packet rate (NOTE 1) | Number of packets per time interval to be enforced. | This field contains any one of: - downlink packet rate for Serving PLMN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections using CIoT EPS Optimisations as described in TS 23.401 [26]). - uplink/downlink packet rate for APN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections to the same APN using CIoT EPS Optimisations as described in TS 23.401 [26]). |
| End of Data Burst Marking Indication | Indicates to the UPF to provide an End of Data Burst indication of the last PDU of a Data burst to the NG-RAN over GTP-U | NG-RAN can configure UE power management schemes like connected mode DRX when UPF provides an indication of the End of Data Burst, see clause 5.37.8.3. |
| Time to Next Burst Marking Indication | Indicates to the UPF to insert the Time to Next Burst to the GTP-U header of the last PDU(s) of the data burst. | NG-RAN may use the received Time to Next Burst to assist radio resource and power management schemes, as described in clause 5.37.x.y. |
| PDU Set Information marking Indicator | Indicates the UPF to insert PDU Set Information related to packets belonging to a PDU Set into GTP-U header. | UPF identifies PDU Sets in DL traffic and forwards PDU Set related information of each PDU to the NG-RAN over GTP-U, as described in clause 5.37.5. |
| ECN marking for L4S indicator | Indicates the UPF to perform ECN marking for L4S for the corresponding QoS Flow. | UPF uses information sent by NG-RAN in GTP-U header extension to perform ECN marking for L4S for the corresponding direction. |
| NOTE 1: This parameter is only used for interworking with EPC. |  |  |

# * * * * Next change * * * *

### 5.37.x	Supporting dynamically changing traffic characteristics via the User Plane

#### 5.37.x.y	Time to Next Burst marking

The Time to Next Burst (TTNB) is the time interval between the transmission of the last packet in the current burst and the first packet of the next burst, i.e., the inter-burst time. The TTNB may be provided to the NG-RAN by the UPF in order to assist radio resource management like DL scheduling and connected mode DRX configuration. As described in TS 23.503 [45] clause 6.1.3.27.5 the PCF may include a Time to Next Burst Marking Indication within a PCC Rule to request the UPF to identify and mark the Time to Next Burst for the corresponding QoS Flow. The PCF also includes the DL Protocol Description received from AF.

Based on the TTNB indication in a PCC Rule, the SMF instructs the UPF to identify and mark the TTNB in downlink. The SMF may provide the UPF the Protocol Description used by the service data flow received in the PCC Rule.

According to the TTNB indication and DL Protocol Description from the SMF, the UPF identifies the TTNB carried in an N6 RTP Header Extension/transport protocol header as defined in TS 26.522 [179] of the last PDU(s) of the data burst. The UPF sends the TTNB to NG-RAN in a downlink GTP-U header of the last PDU(s) of the data burst.

# * * * * Next change * * * *

#### 6.3.3.3	Selection of an UPF for a particular PDU Session

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

-	UPF's dynamic load.

-	Analytics (i.e. statistics or predictions) for UPF load, Service Experience analytics and/or DN Performance analytics per UP path (including UPF and/or DNAI and/or AS instance) and UE related analytics (UE mobility, UE communication, and expected UE behavioural parameters) as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

-	UPF's relative static capacity among UPFs supporting the same DNN.

-	UPF location available at the SMF.

-	UE location information.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

-	Data Network Name (DNN).

-	PDU Session Type (i.e. IPv4, IPv6, IPv4v6, Ethernet Type or Unstructured Type) and if applicable, the static IP address/prefix.

-	SSC mode selected for the PDU Session.

-	UE subscription profile in UDM.

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

-	Local operator policies.

-	S-NSSAI.

-	Access technology being used by the UE.

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	5G-AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

-	Identifiers (i.e. a FQDN and/or IP address(es)) of N3 terminations provided by a W-AGF or a TNGF or a TWIF;

NOTE 1:	A W-AGF or a TNGF may provide Identifiers of its N3 terminations when forwarding over N2 uplink NAS signalling to the 5GC. The AMF may relay this information to the SMF, as part of session management signalling for a new PDU Session.

-	Information regarding the user plane interfaces of UPF(s). This information may be acquired by the SMF using N4;

-	Information regarding the N3 User Plane termination(s) of the AN serving the UE. This may be deduced from 5G-AN-provided identities (e.g. CellID, TAI);

-	Information regarding the N9 User Plane termination(s) of UPF(s) if needed;

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

-	RSN, support for redundant GTP-U path or support for redundant transport path in the transport layer (as in clause 5.33.2) when redundant UP handling is applicable.

-	Information regarding the ATSSS Steering Capability of the UE session (e.g. any combination of ATSSS-LL capability, MPTCP capability, MPQUIC capability) and information on the UPF support of RTT measurements without PMF.

-	Support for UPF allocation of IP address/prefix.

-	Support of the IPUPS functionality, specified in clause 5.8.2.14.

-	Support for High latency communication (see clause 5.31.8).

-	Support for functionality associated with high data rate low latency services, eXtended Reality (XR) and interactive media services, specified in clause 5.37 (for example, ECN marking for L4S, specified in clause 5.37.3, PDU Set Marking, specified in clause 5.37.5, UE power saving management, specified in clause 5.37.8, TTNB marking, specified in clause 5.37.x.y).

-	User Plane Latency Requirements within AF request (see clause 5.6.7.1 and clause 6.3.6 of TS 23.548 [130]).

-	List of supported Event ID(s) for exposure of UPF-related information via service based interface (see clause 7.2.29 and clause 5.2.26.2 of TS 23.502 [3]).

-	Information regarding required and/or preferred UPF functionalities. If received from UDM, the SMF selects a PSA UPF supporting the required UPF functionalities and the best set of preferred functionalities based on their priorities.

-	Support for operator configurable UPF capability as described in clause 5.8.2.21.

NOTE 2:	How the SMF determines information about the user plane network topology from information listed above, and what information is considered by the SMF, is based on operator configuration.

NOTE 3:	In this release the SMF uses no additional parameters for UPF selection for a PDU Session serving TSC or Deterministic Networking. If a PDU Session needs to connect to a specific UPF hosting a specific TSN 5GS bridge or 5GS router, this can be achieved e.g. by using a dedicated DNN/S-NSSAI combination.

If there is an existing PDU Session, and the SMF receives another PDU Session request to the same DNN and S-NSSAI, and if the SMF determines that interworking with EPC is supported for this PDU Session (as specified in clause 4.11.5 of TS 23.502 [3]), the SMF should select the same UPF if it supports all capabilities required for the new PDU Session. Otherwise, if the SMF determines that interworking with EPC is not supported for the new PDU Session or the UPF of the existing PDU Session does not support all capabilities required for the new PDU Session, a different UPF may be selected according to operator policy.

For the same DNN and S-NSSAI if different UPFs are selected at 5GC, when the UE is moved to EPC network, there is no requirement to enforce APN-AMBR. Whether and how to apply APN-AMBR for the PDN Connection associated with this DNN/APN is implementation dependent, e.g. possibly only AMBR enforcement per PDU Session applies.

# * * * * End of changes * * * *