3GPP TSG SA WG2#166	S2-2412415

# 18 - 22 November, 2024, Orlando, USA		(revision of S2-2411806)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0240 | rev | 8 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Policy provisioning for PDU session with local SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to the conclusion as in the clause 8.1 of TR 23.700-49: - I-SMF can be configured with or receive the local offloading policy from PCF via SMF which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and used to select local UPF(s). Rev7: - Unclear how the SMF forward the offloading policy to I-SMF. - Unclear who generates the N4 rule for ULC/L-PSA controlled by I-SMF. - Unclear whether the SMF can send the AF traffic influence to I-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new subclause 6.X.3 for support of policy provisioning for PDU sessions supporting Local Offloading Management. Rev7: - Clarify that the SMF forwards the recived offloading policy received from PCF to I-SMF transparently. - Clarify that the N4 rule for ULCL, L-PSA is generated by the I-SMF. - Clarify that the AF traffic influence information associated with the DNAI(s) supported by the I-SMF is sent by the SMF to the I-SMF, and the I-SMF takes it into account for L-PSA UPF (re)selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Forwarding local offloading policy and PCC rule to I-SMF is not clear. Rev7: - Unclear how the N4 rule is generated. - Unclear whether the AF traffic influence information is sent to I-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.X.3	Policy provisioning for Local Offloading Management

6.X.3.1	General

For Local Offloading Management, traffic routing related policy (e.g., AF traffic influence information in PCC rule, local offloading information) need be provided to the I-SMF.

Based on the receiving traffic routing related policy information and EAS Deployment information, the I-SMF may (re)discover and (re)select ULCL and local PSA UPF for routing the traffic locally.

6.X.3.2	Local offloading management policy handling for Local Offloading Management

The Local Offloading Management Policy is generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.X.2. The Local offloading Management Policy may also be provided with a Local Offloading Management Policy Identifier. The Local Offloading Management Policy Identifier is assigned by PCF, and is unique within the PLMN.The SMF and I-SMF for Local Offloading Management may also be preconfigured with Local Offloading policy and tagged with related Offload Identifier(s). In this case, the PCF only provides the Local Offloading Management Policy Identfier to the SMF.

The detailed information of Local Offloading Management Policy is described in clause 6.4 of TS 23.503 [4].

The SMF forwards the received Local Offloading Management Policy and/or Local Offloading Management Policy Identfier to the I-SMF transparently.

6.X.3.3	PCC rule handling for Local Offloading Management

6.X.3.3.1	AF request for Local Offloading Management

The AF may send to NEF an AF request to influence traffic routing as described in caluse 4.3.6 of TS 23.502[3]. When the SMF receives AF traffic influence information in the PCC rule and provides it to I-SMF, the DNAI within that information only includes the DNAI(s) supported by I-SMF. The I-SMF takes the received AF traffic influence informaion into account for local PSA UPF (re)selection.

Editor’s Note: the detail information within the AF traffic influence information transferred to I-SMF is FFS.

NOTE X:	The AF does not need to to be aware of whether Local Offloading Management is applied or not.

# * * * * End of changes * * * *