SA WG2 Meeting #166 	S2-2412426

# 18 - 22 November 2024, Orlando, USA									 (revision of S2-2412053)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1253 |  | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support AI/ML model performance monitoring by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Xiaomi, Nokia, Ericsson], Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.18 of TS 23.273, Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result. Editor's note: The Performance monitoring procedures are FFS. In this paper it proposes the procedure for NWDAF containing MTLF to perform ML performance monitoring. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add feature description for AI/ML Model performance monitoring in NWDAF side by referring to data collection by NWDAF with additional enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIML_CN feature is not completed in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.273 CR 0574 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 5: Adds description on the possible options for the MTLF to evaluate the performance of an ML model used for AI/ML based positioning. Option 1: LMF provides accuracy feedback information, Option 2: MTLF determines ML model performance by comparing ground truth data against its estimated location. |  |  |  |  |  |  |  |  |

* * * First of Change * * *

### 6.2E.X	MTLF-based AI/ML model performance monitoring for LMF-based AI/ML Positioning

NWDAF containing MTLF supports to perform the performance monitoring for AI/ML model.

When the LMF requests for an ML model for LMF-based AI/ML positioning from the NWDAF containing MTLF using the Nnwdaf_MLModelProvision_Subscribe service, the request may include an ML Model accuracy threshold which is used as a trigger to monitor the ML model performance. The request may also include the ML model identifier of the ML model whose performance needs to be monitored.

NOTE X: The NWDAF containing MTLF can be triggered to monitor the performance of an ML model based on a local trigger.

The NWDAF containing MTLF with ML Model accuracy checking capability is able to determine ML Model degradation for LMF-based AI/ML positioning ML models by:

-	Alt 1. Collecting the input data which includes Ground truth data and measurement data of PRU(s)/UE(s) by using the procedure of input data collection by NWDAF as specified in clause 6.X.4 of TS 23.273 [39].

-	Alt. 2. Collecting the input data which includes ground truth data of PRU(s)/UE(s) and the estimated location of PRU(s)/UE(s) (i.e. by using the LMF-based AI/ML positioning) using the procedure of input data collection by NWDAF as specified in clause 6.X.4 of TS 23.273 [39].

-	Comparing Ground truth data of PRU(s) and/or UE(s) against the PRU(s)/UE(s) estimated location using LMF-based AI/ML based positioning using the Nnwdaf_MLModelMonitor_Subscribe service.

Before collecting the input data from UE(s), LMF needs to check the user consent in UDM during the data collection procedure.

The result may trigger NWDAF containing MTLF to retrain the ML model, or sent to LMF to trigger LMF to change the positioning method, e.g., from LMF-based AI/ML Positioning to legacy positioning.

* * * End of Changes * * *