3GPP TSG-WG2 Meeting #166	S2-2412429

# 18th – 22nd November 2024, Orlando, USA                                       (was S2-2412350)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0597 | rev | 11 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 - Addressing Editor´s Note: Procedure for Data Collection for AI/ML based positioning from NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Xiaomi, Nokia, Interdigital, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Resolution of the following Editor´s Note in clause 5.18, see DP, S2-2410215. Editor’s note: The procedure of LMF collects input data from NG-RAN for ML model training is FFS. Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: Further details on user consent for data collection for model training will be aligned with SA WG3. Editor's note: Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS. Other aspects: The current description of the performance monitoring in clause 5.18 is not aligned with the conclusions in the technical report: Principle #4: LMF or MTLF performs model performance monitoring for AI/ML based Positioning. - Principle #4.1 The result of model performance monitoring may trigger the LMF to change the positioning method, e.g. from AI/ML based positioning to the legacy positioning, or vice versa. - Principle #4.2 The result of model performance monitoring may trigger ML model retraining in the training entity. When the training entity is LMF, the performance monitoring is also performed by LMF as per Principle#4.2. Changes in the merged revisions: Proposal to resolve the following. Editor's Note: Conditions for the LMF to trigger data collection is FFS, e.g., whether and how the LMF can consider load to determine when to perform step 4 is FFS. That is part of Data Collection from the UE but consider applicable to Data collection from NG-RAN too. Proposal to resolve the following: - CR 1196 to 23.288 includes this Editor´s Note, Editor’s Note: Applicability of the parameters of the Contents of ML Model Provisioning for LMF-based AI/ML Positioning is FFS. - Also Positioning case information (e.g. Uplink/downlink, or case 2b, 3b) can be included in NF profile as an optional parameter is FFS. This CR proposes to specify the applicability of the parameters for ML Model provisining in 23.273 instead, and not in 23.288. The reason is that 23.288 defines a framework for any consumer to request a ML Model for an AnalyticsID, however we have defined that NWDAF can request a MLModel that is specific for UE Positioning, as such it is suggested to add it to 23.273. Other aspects: PRUs can be associated only to one LMF. Therefore it is possible that LMF which initiates data collection does not have all PRU in the TA or does not have PRU at all. LMF needs to query NRF for those LMF which have PRU assosiated to certain LMF. LMF needs to ask PRU serving LMF to execute measurements. R06 - For performance monitoring, the ground truth data is needed, and also the calculated location is also needed and which is calculated by LMF-based AI/ML Positioning based one the measurements. The data collection is used for AI model training, and also used for performance monitoring. So Update the description to support AI/ML model performance monitoring, besides AI/ML model training. And clarify that the calculated location derived from measurements data by LMF-based AI/ML Positioning R09 - Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training. Once the data collection aspects are coordinated, then the associated model training procedure needs to be modified to include an optional request from MTLF at the beginning, and also a optional data transfer step to MTLF at the end. |  |  |  |  |  |  |  |  |
| Summary of changes: |  | A new clause for Data collection from NG-RAN is added, using Network Assisted Positioning Procedure. The LMF uses the same procedure to retrieve and enforce user consent as defined in Rel-18 in TS 23.288, except that NWDAF is not the consumer but the LMF. The LMF triggers data collection based on internal trigger, and requests AMF(s) to provide the list of SUPIs in an area of interest. When the training entity is LMF, the performance monitoring is performed by LMF. Changes in the merged revisions: - The LMF to retrieve AnalyticsID on Network Performance in NG-RAN to determine when data collection should be initiated (step 4). - Defining how LMF retrieves a ML Model from NWDAF. - For the retrieval, the LMF includes the Positioning method. - Handling of PRUs that are served from a different LMF. - R05 Address EN: Various editorial corrections and address EN in similar CR 0584 Conditions for the LMF to trigger data collection is FFS, e.g., whether and how the LMF can consider load to determine when to perform step 4 is FFS - Update the description to support AI/ML model performance monitoring, besides AI/ML model training. And clarify that the calculated location derived from measurements data by LMF-based AI/ML Positioning - R07 - General descriptions of inference and user consent are added; - R09 - Once the data collection aspects are coordinated, then the associated model training procedure needs to be modified to include an optional request from MTLF at the beginning, and also a optional data transfer step to MTLF at the end. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AI/ML based positioning is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.13, 5.18, 6.x.3 (new), 6.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0584, CR074 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Merge with CR0597R5, CR0597R6, CR0597R7 , CR0597R9 |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 4.4.13	NL8 Reference Point

The NL8 reference point supports LMF to receive location related analytics or network performance analytics from NWDAF as defined in TS 23.288 [37].

* * * Next Change * *

## 5.18	Support for UE positioning based on a ML Model at the LMF

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

Once the ML model for LMF-based AI/ML Positioning is trained and located in the LMF, the LMF may perform inference after it receives a location determination request from AMF.

Editor´s Note: Whether a user consent for data collection for inference is needed is FFS.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 6.x.2. The LMF collects input data from NG-RAN for ML model training is described in clause 6.x.3.

The LMF may also request a trained ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF as described in clause 6.x.y. The LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37] with the following considerations:

- the LMF provide a request to retrieve an AI/ML Model for UE Positioning and positioning case information, the NWDAF can derive whether the data collection from the UE or from NG-RAN is needed depending on the positioning case information. In addition:

- the LMF provides an AoI, the ML Model Interoperability indicator to request an AI/ML Model that the LMF can run. The LMF requests a NWDAF with ML Model accuracy checking capability to be able to report to LMF that the model is degraded.

NOTE x: Other NWDAF discovery parameters listed in clause 5.2 such as AnalyticsID, FL capability and related time period, S-NSSAI or any roaming capabilities are not required by LMF.

The LMF requests ML model for LMF-based AI/ML Positioning with ML Model Filter Information, which may indicate the area of interest.  The LMF requests the NWDAF to provide an AI/ML Model as described in clause 6.2A.2 of TS 23.288 [37] with the following considerations:

- The LMF provides a request for a trained AI/ML Model for UE Positioning, the Notification Target Address to receive the ML Model, the positioning case information and the Target of ML Model Reporting set to any UE.

- If some vendor specific information is needed to allow running the ML Model, then the ML Model Interoperability Information,

- If the LMF supports multiple AI/ML Models, the support for multiple AI/ML Models, optionally with the maximum number of ML Models.

Editor´s Note: The granularity of AI/ML positioning model (e.g., per positioning case) is FFS, which needs to be coordinated with RAN WGs.

The NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.x.4, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37] with the following considerations.

- The NWDAF provides the ML Model identifier and ML Model Information for the ML Model for UE Positioning and indication of whether the ML Model identifier is updated if model is retrained and the ML Model accuracy Information.

- Some parameters related to the AI/ML Model to be trained, such as the requested representative ratio of the provided list of SUPIs or the ML Model Target Period and desired model metrics if different from those provided in the request.

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

For training in LMF-based AI/ML Positioning, LMF  checks with UDM the user consent status before collecting UE related data. For performance monitoring, LMF must also check with UDM the user consent for any UE data not collected from PRUs, see clause 6.x.3 and clause 6.x.4.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. When the ML model that is used for LMF-based AI/ML Positioning is trained by LMF the LMF monitors the performance of the ML model.  When the ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF monitors the performance of the ML model.  LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

Editor's note:	The Performance monitoring procedures are FFS.

* * * Next Change * * *

### 6.x.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN maessurements

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.x.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

- The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs or to monitor the AI/ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.x.4-1.

- The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model using a PRU, which are associated to this LMF that executes data collection, the LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF in order to identify the LMF(s) which has PRU associated to a list of TAIs. The LMF may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs in order to get the data collection and the known PRU location in this case step 6 follows.

If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest.

- For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

- The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed.

- The LMF checks whether the SUPI provided user consent for data collection for the purpose of training with UDM. If user consent is provided, then step 6 follows. if user consent is not provided, the LMF does not perform step 6 for this UE, that will not be part of the data collection, and its data is not used for the LMF-based AI/ML Model positioning.

Editor's note:	Further details on user consent for data collection for model training or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3.

- The LMF may use Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo_Request (Analytics ID="Network Performance", Target of Analytics Reporting “any UE”, Analytics Filter Information = "Area of Interest" and optionally Analytics Reporting Information = ("Reporting Thresholds" and Analytics target period(s))) to the NWDAF to request the predictions of the gNB status and gNB resource usage in the Area of Interest for data collection from NG-RAN.

- The LMF checks the gNB status and gNB resource usage in the area of interest, the LMF may decide to delay the request for data collection from NG-RAN during the current Analytics Target period if the gNB status indicates that gNBs in the area of interest are not available or the resource average usage indicates a high resource usage, in this case step 8 and 9 are skipped and no data is collected from NG-RAN or from the UE happens. At the time the Analytics target period elapses, step 6 may be executed by the LMF, if data collection from NG-RAN is needed.

- The LMF request input data for the target UE.

- The LMF receives ground truth data from the UE.

The measurements and ground truth data are used for training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 8 and 9 may occur in paralell for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

* * * Next Change * * *

### 6.x.y	LMF retrieval of a trained AI/ML Model for positioning

The LMF may retrieve a trained AI/ML Model from NWDAF. The LMF obtains a trained AI/ML model prior to a UE positioning request that requires a trained ML Model.

Figure 6.x.3-1: The LMF retrieves a trained AI/ML based positioning model from NWDAF

- The LMF determines using local policies, that a positioning method requires a trained ML Model, the trained ML Model is not trained locally but is retrieved from NWDAF.

- The LMF discovers the NWDAF that can provide a trained model, using Nnrf_NFdiscovery including a request for AI/ML Model for UE positioning, ML Model Filter Information that includes the positioning case information and an Area(s) of Interest, the ML Model Interoperability indicator applicable to the trained ML Model for UE Positioning, optionally the ML Model Accuracy checking capability.

Editor´s Note: The granularity of AI/ML positioning model (e.g., per positioning case) is FFS, which needs to be coordinated with RAN WGs.

NOTE 1: 	NWDAF discovery and selection is described in clause 5.2 of TS 23.288 [37], the applicable NWDAF parameters used for discovery of an NWDAF supporting an AI/ML Model for UE Positioning are described in clause 5.18.

- The LMF receives a list of candidates NWDAF instances, validity period for the discovery, per NWDAF instance the NWDAF instance ID, FQDN or IP address, and the ML Model Interoperability Indication.

- The LMF selects a NWDAF out of the list of candidates NWDAF instances, that can provide a ML Model that supports the same ML Model Interoperability Indication then request a trained AI/ML Model using Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe including the request for a trained AI/ML Model for UE Positioning according to the information listed in clause 5.18.

Editor´s Note: Whether the LMF provides Inference Input Data information to NWDAF in the Nnwdaf_MLModelProvision service is FFS.

- The NWDAF provides one or more AI/ML Models to LMF including a ML Model Identifier and the ML Model Information as described in clause 5.18.

- Same as in step 5.

The LMF updates the received trained ML Model(s) when the NWDAF provides new ones.

* * * End of Changes * * *