3GPP TSG-SA2 Meeting #166	S2-2412438

# 18th – 22nd November, 2024, Orlando, US

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1296 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics for QoS and Policy Assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT, ZTE, China Mobile, China Telecom, China Unicom, Lenovo, ICS, Futurewei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As it has been concluded in TR 23.700-84, we propose a new Analytics ID of NWDAF to provide QoS and Policy assistance information. In particular, When PCF needs assistance information to determine QoS or policy parameters, it may send an assistance information request or subscription to NWDAF with one or multiple sets of QoS parameters and expected service experience (e.g. QoE). NWDAF provides the candidate set(s) of QoS parameters and the corresponding predicted QoE can be achieved by the candidate set(s) of QoS parameters (the candidate set(s) of QoS parameters are within the sets of QoS parameters provided by the PCF). The predicted QoE(s) corresponding to the candidate set(s) of QoS parameters should fulfil the target service experience provided by the PCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new NWDAF-based QoS and Policy Assistance Analytics ID to support KI#3 use case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case for FS_AIML_CN KI#3 cannot be supported by R19 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes (new text) * * *

6.x	QoS and Policy Assistance Analytics

6.x.1	General

Clause 6.x describes how NWDAF can provide the QoS and policy assistance analytics, in the form of predictions, based on the consumer request. The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. a Request-Response model).

The service consumer may be a PCF.

When the service consumer needs assistance information to determine QoS or policy parameters, it may send an Analytics request or subscription to NWDAF with one or multiple sets of QoS parameters and the expected service experience (e.g. QoE). NWDAF provides the candidate set(s) of QoS parameters and the corresponding predicted QoE that is expected to be achieved by applying the candidate set(s) of QoS parameters (the candidate set(s) of QoS parameters are within the sets of QoS parameters provided by the PCF, if they are present). The predicted QoE(s) corresponding to the candidate set(s) of QoS parameters should meet the expected service experience provided by the service consumer, if it is present.

The QoS and policy assistance analytic may be provided for individual UE or group of UEs (e.g. the QoE is for the application ID associated to one or more QoS flow(s) of the UE(s)), or for an application (e.g. the QoE is for the service flow of the application ID).

The consumer of these analytics indicates the following information in the request or subscription:

-	Analytics ID = " QoS and Policy Assistance".

-	Target of Analytics Reporting as defined in clause 6.1.3.

-	An Analytics target period indicates the time period over which the analytics are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The consumer of these analytics shall provide one or all of the following information in the request or subscription:

-	A list of one or more QoS parameter set(s), optionally each associated with a QoS parameter set ID. The QoS parameter set includes the following individual parameters, and	non-empty value list(s) for every individual parameter(s) in every QoS parameter set(s).

-	5QI (standardized or pre-configured) or QoS Characteristics attributes (as defined in clause 5.7.3 of TS 23.501): Resource Type, Priority Level, PDB, PER, Averaging Window, Maximum Data Burst Volume, and the corresponding value of each individual QoS Characteristics attributes.

-	For GBR Resource Type, GFBR, MFBR (as defined in clause 5.7.2 of TS 23.501).

Editor’s Note: It is FFS whether QoS value list(s) for every individual parameter(s) are provided in every QoS parameter set(s).

-	Input QoE, to indicate NWDAF to only provide the candidate QoS set(s) and their corresponding value(s) for which the predicted QoE is equal or higher than the Input QoE.

The consumer of these analytics may optionally provide the following information in the request or subscription:

-	Analytics Filter Information optionally includes:

-	DNN;

-	Application ID;

-	Area of Interest (AOI(s));

-	S-NSSAI

-	An Analytics target period indicates the time period over which the analytics are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Preferred order of results for the list of candidate QoS parameter sets:

-	ordering criterion: "QoE" (i.e. QoE that is associated to candidate QoS parameter set) or GFBR, PDB or PER,

-	order: ascending or descending.

-	Reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the respective analytics subsets (see clause 6.x.3).

6.x.2	Input Data

The NWDAF supporting QoS and policy assistance analytics shall be able to collect the following data from AF, OAM and 5GC NFs.

-	Service data flow information and corresponding QoS parameters, as described in Table 6.x.2-1;

-	Input data for Observed Service Experience Analytics as defined in clause 6.4.2;

-	Input data for Network Performance Analytics as defined in clause 6.6.2;

-	Input data for QoS Sustainability Analytics (e.g., measurement of QoS monitoring) as defined in clause 6.9.2.

NOTE:	The NWDAF can also collects the Observed Service Experience Analytics, Network Performance Analytics and QoS Sustainability Analytics from other NWDAFs.

Table 6.x.2-1 Application service information and corresponding QoS parameters collected by the NWDAF

| Information | Source | Description |
| --- | --- | --- |
| Application Identifier | PCF | An application identifier of the service data flow. |
| QoS parameters | PCF | 5QI (standardized or pre-configured) or QoS Characteristics attributes (as defined in clause 5.7.3 of TS 23.501): Resource Type, Priority Level, PDB, PER, Averaging Window, Maximum Data Burst Volume, and the corresponding value of each individual QoS Characteristics attributes. For GBR Resource Type, GFBR, MFBR (as defined in clause 5.7.2 of TS 23.501). |
| QoS binding events (1..max) | SMF | SMF provides such event when specific QoS parameters are applied to some QoS flows |
| > Used QoS parameter | SMF | 5QI (standardized or pre-configured) or QoS Characteristics attributes (as defined in clause 5.7.3 of TS 23.501): Resource Type, Priority Level, PDB, PER, Averaging Window, Maximum Data Burst Volume, and the corresponding value of each individual QoS Characteristics attributes. For GBR Resource Type, GFBR, MFBR (as defined in clause 5.7.2 of TS 23.501). |
| > Used QoS profile | SMF | The QoS profile associated to the used QoS parameters. |
| > Traffic descriptor | SMF | An Application Identifier. |
| > S-NSSAI | SMF | Slice associated with the QoS flows. |
| > DNN | SMF | DNN associated with the QoS flows. |

Editor’s Note: the input data (e.g. data collected from OAM, NF and AF, or the output of other analytics IDs) in this clause is FFS.

6.x.3	Output Analytics

The NWDAF supporting QoS and Policy Assistance analytics is able to provide prediction to consumer NFs, e.g. PCF, as defined in Table 6.x.3-1.

Table 6.x.3-2: QoS and Policy Assistance predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > QoS and Policy Assistance information (1…max) (NOTE 1) | List of QoS and Policy Assistance information. Max. is the number of the candidate QoS parameter set(s), if applicable. |
| >> QoS parameter set identifier | Identifies the QoS set for which the entry applies |
| >> Application ID | Identifies the application that associated to the PCC rule or QoS. |
| >> service traffic duration time | Average duration time of the service data being transferred through the network. |
| >> Predicted QoE (NOTE 5) | The predicted service experience (e.g. QoE) of the corresponding QoS parameter set. |
| >> QoS parameter values (NOTE 4) | QoS parameter value combinations provided by the analytics consumer for each predicted QoE. |
| >>> 5QI | The reference to 5G QoS characteristics and QoS parameters. |
| >>> Packet Delay Budget | Packet Delay Budget (PDB) indicates the upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [2]. |
| >>> Packet Error Rate | Packet Error Rate (PER) defines an upper bound for a rate of non-congestion related packet losses, as defined in TS 23.501 [2]. |
| >>> Flow Bit Rates (NOTE 2) | The flow bit rates only applies to GBR QoS Flow. |
| >>>> GFBR | Guaranteed Flow Bit Rate (GFBR) for UL and/or DL. |
| >>>> MFBR | Maximum Flow Bit Rate (MFBR) for UL and/or DL. |
| >>> Averaging Window (NOTE 2) | The Averaging window is applied when the resource type is GBR QoS. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. in the (R)AN, UPF, UE), as defined in TS 23.501 [2]. |
| >>> Maximum Data Burst Volume (NOTE 3) | The Maximum Data Burst Volume (MDBV) applies to GBR QoS Flow with Delay-critical resource type. The MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB, as defined in TS 23.501 [2]. |
| >> Applicable duration of QoS and Policy Assistance information | The applicable duration/ time window of the QoS and Policy Assistance information. |
| >> Validity period | The validity period within the time slot for the analytics on service experience associated to QoS in clause 6.1.3. |
| >> Spatial validity | Area where the analytics on service experience associated to QoS applies. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", and "Reporting Thresholds". NOTE 2: The output analytics only applies to GBR QoS Flow. NOTE 3: The output analytics only applies to GBR QoS Flow with Delay-critical resource type. NOTE 4: Only the parameters for which candidate values have been provided shall be included, and only values which were provided by the consumer are allowed. NOTE 5: The predicted QoE is reported for the entire QoS set if only one parameter value has been provided for each parameter and otherwise separately for each combination of parameter values. |  |

6.x.4	Procedures for Providing QoS Policy Assistance Analytics

Figure 6.x.4-1: Procedure for NWDAF providing QoS policy assistance information analytics

1.	The PCF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID=" QoS and Policy Assistance", Target of Analytics Reporting, Analytics Filter Information, QoS parameter set(s), expected service experience) to the NWDAF.

2a-2b. NWDAF subscribes to service data from AF for observed service experience as defined in clause 6.4.2 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service operation.

2c-2d.	NWDAF subscribes to the network data from 5GC NF(s) (e.g. AMF, SMF, UPF) for observed service experience as defined in clause 6.4.2, network performance as defined in clause 6.6.2 and QoS sustainability as defined in clause 6.9.2, by invoking Nnf_EventExposure_Subscribe service operation. NWDAF may also requests UE location by invoking Ngmlc_Location_ProvideLocation service as defined in TS 23.273 [39].

2e-2f.	NWDAF subscribes to the QoS policy information as defined in clause 6.x.2 from the PCF by invoking Npcf_EventExposure_Subscribe service operation.

2g.	NWDAF subscribes to OAM services to collect data for observed service experience analytics as defined in clause 6.4.2, network performance analytics as defined in clause 6.6.2 and QoS sustainability as defined in clause 6.9.2.

3.	Based on the input data, parameters indicated in the analytics request/subscription and local configuration, the NWDAF generates the QoS policy assistance information, i.e. Candidate QoS parameters and the corresponding predicted QoE.

4.	The NWDAF provides the QoS policy assistance information to the PCF via Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5.	The PCF determines QoS policy taking into account of the QoS policy assistance information (i.e. adopting or modifying the QoS policy in the QoS policy assistance information to get the QoS policy for enforcement), and provisions the determined QoS policy to the SMF. The SMF interacts with the NG-RAN, UPF and UE to enforce the QoS policy as specified in TS 23.503 [4].

6-9.	The AF, AMF/SMF/UPF/GMLC, PCF and OAM sends notifications of new data related to service experience, network performance and UE location after enforcement of the QoS policy to the NWDAF. The NWDAF may optimize the ML model based on service experience and network performance analytics derived with the new input data, to generate new QoS policy assistance information analytics to provide to the PCF. The PCF may then update the QoS policy based on the new optimal QoS policy in the QoS policy assistance information and provide the updated QoS policy towards the SMF.

* * *Next Change * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |
| QoS and Policy Assistance | Analytics ID: QoS and Policy Assistance | Choose candidate QoS parameter sets and provide the corresponding predicted QoE |

* * *End of Changes * * *