3GPP TSG-WG SA2 Meeting #166	S2-2412513

Orlando, Nov 18 – 22, 2024	(revision of S2-2412049)

Source:	Nokia

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#3 are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#3.

- It is proposed to remove the temporary disable/enable services as there are no solutions available to support those services. This can be discussed again in the next release.

- The principle of the procedure based on the indirect RRC architecture option is proposed.

- Information that needs to be provided by the AF, how to select and authorize the readers are newly proposed.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.1.0.

FIRST CHANGE

### 8.3.3	NEF exposure

NEF supports to expose Ambient IoT service towards the AF.

-	New NEF services (e.g., Nnef_AIoT) to support the AIoT services will be defined.

NOTE:	Whether new events are defined and/or existing events (e.g., Area Of Interest),  are extended for exposure by the NEF to external AFs to support AF logic will be further discussed during the normative phase.

### 8.3.x	Information provided by the AF

The AF provides one or some of the following information:

- Information about AIoT services

  - Service type indicating e.g., inventory or command

- Information to be used for AIoT reader selection

  - At least one of the UE reader ID or the target area information

- Optionally, Information about the target AIoT device(s)

  - At least one of the AIoT device ID(s) or

  - the filtering information that is applied to Permanent device IDs to group these devices together

- Optionally, Information to be used for resource allocation

  - Expected number of AIoT devices

  - Expected D2R message size

NOTE:	What additional information can be provided by the AF will be further discussed during the normative phase in coordination with RAN WGs.

END OF CHANGES