3GPP TSG-WG SA2 Meeting #166	S2-2412526

# 18th – 22nd November, 2024, Orlando, US

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5822 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of framed routing for N3G device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As concluded in the TR 23.700-32 KI#4, the UE will report the Non-3GPP Device Identifier and the related IP address/Port Number(s) to the network. To resolve this issue, the UE firstly needs to obtain the IP addresses for the Devices. It is proposed that the SMF sends the IPv4 range obtained from the Framed Route information to the UE during the PDU Session Establishment procedure, then the UE can bind the traffic from the non-3GPP devices to the IP address which is visible to the 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE obtains the IP range information from the network, and binds the traffic from the non-3GPP device to the IP address. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion of KI#4 would not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The refernce to TS 23.503 [45] clause 6.1.3.xx in CR 1310 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.x.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS:

-	The SMF may obtain the Framed Route information from the UDM. If the UE supports to allocate IPv4 address to the non-3GPP device, the SMF sends the IPv4 range obtained from the Framed Route information to the UE during the PDU Session establishment (i.e. delivered in NAS PDU Session Establishment Accept), as defined in TS 23.502 [3] clause 4.3.2.2.1.

-	The UE allocates an IPv4 address belonging to the IPv4 range if received from the SMF to the Non-3GPP device, and sends the Non-3GPP Device Identifier and the IPv4 Address, in case of IPv4 PDU session type associated with the Non-3GPP Device Identifier, to the SMF during PDU Session Modification procedure.

NOTE x:	The UE allocates an IPv4 address belonging to the IPv4 range received from the SMF to the Non-3GPP device, by means not to be specified by 3GPP.

# * * * * End of changes * * * *