3GPP TSG-WG SA2 Meeting #166	S2-2412633

# Orlando, USA, 18th Nov – 22nd Nov, 2024		(revision of S2-2411802)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5688 | rev | 5 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE served by a MWAB: authorization aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, LG Electronics, Huawei, ZTE, Samsung, Ericsson, Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved VMR_Ph2 work item (SP-240632) is set to specify the architecture enhancements, functionalities and procedures to support MWAB based on conclusions of TR 23.700-06 (clause 8). This CR introduces the descriptions on how the MWAB is authorized to operate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause added to describe how the MWAB is auuthorized to operate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MWAB feature cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev4: Additional clarifications on authorization aspects. |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.49.3	Authorization aspects

5.49.3.1	General

There are two aspects related to the authorization of an MWAB. One aspect is the authorization of the MWAB-UE to establish a BH PDU Session. The other aspect is related to the authorization of the MWAB-gNB to operate as a gNB logically belonging to a MWAB Broadcasted PLMN/SNPN which the MWAB-gNB announces in the SIB.

5.49.3.2	MWAB-UE authorization

The MWAB-UE is authorized based on its subscription information which includes dedicated S-NSSAI(s) and DNN for MWAB operation. Such dedicated S-NSSAI(s) and DNN(s) shall be part of the subscription information for the MWAB-UE as specified in clause 5.2.3.3.1 of TS 23.502 [3].

The authorization of the MWAB-UE to establish connectivity in a BH PLMN/SNPN is based on whether dedicated S-NSSAI(s) for MWAB operations are received as part of Allowed NSSAI in BH PLMN/SNPN following existing procedures as described in clause 5.15, and whether it can establish BH PDU session(s) in the BH PLMN/SNPN using the dedicated S-NSSAI(s) and DNN(s).

The MWAB-UE authorization supports time-based or location-based control.

-	For time-based control, existing time-based network slice subscription control can be reused (including the enhancements in clause 5.15.16).

-	For location-based control, existing mechanism, e.g. service area restriction, LADN based control (see e.g. clauses 5.6.5 and 5.6.5a) can be reused.

NOTE 1:	When the MWAB-UE can no longer use S-NSSAI(s) dedicated to MWAB operation,, e.g. the dedicated S-NSSAIs and DNNs for MWAB operation are removed from the subscription (i.e. the MWAB becomes unauthorized) and a UE subscription update (including a UE configuration update) step happens, or the MWAB-UE moves to an area where the S-NSSAI is not available, if the AMF is configured to do so for the dedicated S-NSSAI and DNN for MWAB operation, the AMF delays the corresponding PDU session release based on an operator configuration (e.g. a local configured timer to allow time for the MWAB-gNB to handover the UE(s) it serves to other cells).

NOTE 2:  If the S-NSSAI(s) used by BH PLMN for MWAB operation is not dedicated for MWAB, the behaviours of AMF as in NOTE 1 cannot be limited to MWAB-UEs only. Additionally, it may also not be possible to configure specific treatment for MWAB-UEs (different from other UEs) in the NG-RAN.

The MWAB-UE may be connected to the BH PLMN/SNPN for other services, if allowed by the subscription data, even if the S-NSSAIs and DNNs for Backhaul PDU sessions are no longer authorized.

5.49.3.3	MWAB-gNB authorization

The authorization of the MWAB-gNB is managed by the OAM system of the MWAB-Broadcasted PLMN/SNPN as specified in TS 38.401 [42]. The MWAB-gNB establishes a secure and trusted connection to the OAM server using the IP connectivity provided by the MWAB-UE via the Backhaul PDU Session(s) as specified in clause 5.49.1.

The OAM of the MWAB Broadcasted PLMN/SNPN determines when/where the MWAB-gNB can operate or when/where it needs to shut down. The OAM of the MWAB Broadcasted PLMN/SNPN may pre-configure the MWAB-gNB to turn on/shut down the operations of respective MWAB Broadcasted PLMN/SNPN part. When the MWAB-gNB is no longer authorized to operate by OAM of the MWAB Broadcasted PLMN/SNPN, the MWAB-gNB should handover the UE(s) of the MWAB Broadcasted PLMN/SNPN to other cells.

NOTE: 	It is expected the HPLMN/Subscribed SNPN network ensures that the BH PDU sessions authorization for MWAB UE does not interfere with the ability of the OAM of the MWAB Broadcasted PLMN/SNPN to manage the MWAB-gNB, e.g. the S-NSSAI(s) for the BH PDU session(s) of the MWAB-UE is(are) maintained long enough in the MWAB-UE subscription and are available for the MWAB-UE for a location and time that is sufficient for the MWAB-UE, so that the BH PDU sessions are available for the MWAB-gNB to be able to perform OAM-controlled shut down, which may include handing over the UEs it serves.

When the MWAB-gNB stops operating under OAM control, the MWAB-gNB may trigger the MWAB-UE to release the BH PDU sessions.

End of CHANGES