3GPP TSG-SA WG2 Meeting #165	S2-2412671

# Orlando, USA, 18-22 Nov, 2024								                  (revision of S2-2412448)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1492 | rev | 7 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication in IMS with IMS-AGW on board |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, [NTT DOCOMO,] China Telecom, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on 5GSAT_Ph3_ARCH WID work task, the functionality and procedure of UE-satellite-UE communication via IMS-AGW deployed on satellite should be captured in TS 23.228. For call setup: When the originating P-CSCF receives the initial INVITE request from the originating UE using satellite access, it is not able to determine whether the UE-Satellite-UE communication can be activated before negotiating with the terminating side. The originating P-CSCF selects an IMS-AGW on ground for the originating UE by default after receiving the INVITE message and triggers media negotiation by sending SDP offer to the terminate side. If the terminating side determines that it is possible to activate UE-Satellite-UE communication, then, a re-selection/re-negotiation procedure is needed for the originating P-CSCF to re-select the IMS-AGW on satellite for the originating UE to enable the UE-Satellite-UE communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of support of UE-Satellite-UE communication in IMS, including: - Call setup procedures of UE-SAT-UE communication. Rev06: modified step 2 of procedure, clean up text, removed support for Rx removed EN about subscription to UE-SAT-UE Rev07: Removed editor’s note about early media; reworded NOTE W, reworded NOTE Z; added service operation names to transfer Access Network Information to P-CSCF; added service operation names used to notify the P-CSCF of change of satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.5 (new), AX.5.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after TS 23.228 CR 1428. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change (All following text are new) ---

AX.5	Procedures for optimizing media routing

AX.5.1	At call setup

When the originating P-CSCF receives the initial INVITE request from the originating UE using regenerative NR satellite access, it selects the IMS-AGW on ground and triggers media negotiation by sending an SDP offer to the terminating side indicating that satellite access is being used on the originating side. If the terminating P-CSCF activates UE-satellite-UE communication, the originating P-CSCF re-selects an IMS-AGW on satellite. See Figure AX.5.1-1 that describes the procedure. In the procedure, UE A, P-CSCF A and IMS-AGW A/A' represent originating side function or entity, UE B, P-CSCF B and IMS-AGW B/B' represent terminating side function or entity.

NOTE X: To support the UE-Satellite-UE communication in IMS, the P-CSCFs (AF) need to support the N5 interface for AF influence traffic routing and user plane event subscription/notification

Figure AX.5.1-1:	Session establishment procedure with activation of optimized media routing

1.	UE A sends a SIP INVITE request, containing an initial SDP offer, to P-CSCF A. The access information in the SIP INVITE request indicates that the UE is accessing from NR satellites access as defined in TS 24.229 [10a].

2.	After receiving the SIP INVITE request indicating NR satellite access, if UE A is not roaming and the P-CSCF A supports UE-Satellite-UE communication the P-CSCF requests Access Network Information from the PCF via Npcf_PolicyAuthorisation. The PCF obtains the Access Network Information from the SMF via Npcf_SMPolicyControl_Update.

3.	The P-CSCF A allocates an IMS-AGW A' on the ground. P-CSCF A interacts with IMS-AGW A' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 4-20 are performed if UE-Satellite-UE communication in IMS is possible based on the decision in step 2 (i.e., UE A is using NR regenerative payload satellite access and is not roaming). Otherwise, the P-CSCF selects an IMS-AGW on the ground and SIP message routing procedures with IMS-AGW on ground are performed (details have been omitted from the diagram).

4.	P-CSCF A updates the SIP INVITE request inserting the identification of the satellite serving UE A and updates SDP offer with the IMS-AGW transport addresses allocated in step 3. P-CSCF A forwards the updated SIP INVITE request to the terminating side via IMS core (details have been omitted from the diagram).

5.	After receiving the SIP INVITE request which contains a satellite identification indicating that UE A is using regenerative satellite access, if UE B is not roaming, the P-CSCF B retrieves Access Network Information for UE B from the PCF via Npcf_PolicyAuthorization_Notify. The PCF obtains Access Network Information from the SMF via Npcf_SMPolicyControl_Update

6.	Based on the Access Network Information received at step 5, if UE B is using regenerative payload satellite access and if the two satellites identified in step 4 and 5 are the same or connected with ISL(s), P-CSCF B determines to activate the UE-Satellite-communication in IMS.

If UE-Satellite-UE communications in IMS is not activated, P-CSCF B selects an IMS-AGW B' on ground for UE B; otherwise, P-CSCF B selects an IMS-AGW B on-board a satellite for UE B.

NOTE Y:	How the terminating P-CSCF uses the satellite identifications to derive whether the two satellites are connected with ISL(s) and are equipped with UPF and IMS-AGW is based on implementation or operators’ policy e.g., by querying transport network system, or based on node level information reported by the IMS-AGW deployed on the satellites which reports whether the two satellite are connected or not.

6a-6b. Based on the decision in step 6, P-CSCF B interacts with either IMS-AGW B on-board the satellite or IMS-AGW B' on ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 7-20 are performed if UE-Satellite-UE communication in IMS is activated. Otherwise, SIP message routing procedures with IMS-AGW on the ground should be performed after P-CSCF B selecting a ground IMS-AGW B' for usage (details have been omitted from the diagram).

7-9. P-CSCF B forwards the updated SIP INVITE request to UE B. UE B returns an 18X response in the SDP Answer as normal routing SIP message routing procedures.

9a-9b. After receiving the SDP Answer, P-CSCF B interacts with the IMS-AGW B to allocate transport resources based on the mechanism defined in TS 23.334 [74].

10.	P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for the media via Npcf_PolicyAuthorization_Update service operation with following additional actions.

To indicate to the SMF to insert a UL CL/BP and L-PSA UPF for the IMS PDU Session to enable UE-Satellite-UE communication in IMS, the P-CSCF B should send following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW B;

-	A DNAI associated with the IMS-AGW B on-board the satellite and corresponding N6 traffic routing information.

To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AX.5.2, P-CSCF B subscribes Notification of User Plane Management Events (i.e., UP path change) via Npcf_PolicyAuthorisation_Update to the PCF which receives the events from the SMF via Nsmf_EventExposure_Notify :

-	Early and late notifications about UP path management events;

-	AF transaction identifier.

NOTE Z: If Rx is used by P-CSCF,  subscription is done implicitly as part of PCC rules setting.

11.	P-CSCF B updates the 18X response to carry the identifier of the satellite serving UE B to indicate that UE-Satellite-UE communication in IMS is activated. P-CSCF B returns the updated 18X response to originating side via IMS core (details have been omitted from the diagram).

12.	Based on the identifier of the satellite serving UE B received in step 11, P-CSCF A determines to perform UE-satellite-UE communication in IMS and select an IMS-AGW A on satellite.

12a-12c. P-CSCF A interacts with IMS-AGW A on-board the satellite to allocate transport resources for both IMS access and IMS Core network sides. P-CSCF A also releases the IMS-AGW A' on ground.

13. P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation. 

To indicate to the SMF to insert UL CL/BP and L-PSA UPF for the IMS PDU Session to enable UE-Satellite-UE communication in IMS, P-CSCF B should send following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW A;

-	A DNAI associated with the IMS-AGW A on-board the satellite and corresponding N6 traffic routing information.

To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AX.5.2, P-CSCF A subscribes to Notification of User Plane Management Events (i.e., UP path change) via Npcf_PolicyAuthorisation_Update to the PCF which receives the events from the SMF via Nsmf_EventExposure_Notify:

-   Early and late notifications about UP path management events;

-   AF transaction identifier.

NOTE W: If Rx is used by P-CSCF,  subscription is done implicitly as part of PCC rules setting.

14.	P-CSCF A updates the SDP Answer in the 18X response with the IMS-AGW transport addresses allocated in step 12, and forwards the updated 18X response to UE A.

15.	UE A acknowledges the 18X response and sends the Response Confirmation to P-CSCF A.

16-17. P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes the SDP offer modified/generated by P-CSCF A which contains the IMS-AGW transport address obtained in step 12. Upon receiving the SDP offer, P-CSCF B updates the allocated IMS-AGW on-board the satellite for UE B with the transport address in the SDP offer.

18.	P-CSCF B sends the Response Confirmation to UE B.

19.	Procedure continues to setup the call as defined in clause 5.6 and 5.7.

## ---End of the Change---