3GPP TSG-WG SA2 Meeting #166	S2-2412764

# Orlando, USA, November 18 – 22, 2024		(revision of S2-2412518, S2-2411056)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5739 | rev | 9 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: UE subscription and policy control for energy efficiency and energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Samsung, [vivo, ZTE, NEC, Huawei, China Mobile, MediaTek Inc., China Unicom, TOYOTA, China Telecom, CATT, Deutsche Telekom, LG Electronics] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the SA2#165 meeting the CR rev 3 was agreed. It proposes a new “Indication for network energy saving”. For the Subscription data, there are following Editor’s Notes: Editor’s note: How the indication of network energy saving is used in the network is FFS. Editor’s note: Details and granularities of the indication (e.g. per UE, per slice) is FFS. For the policy control, the folowing Eitor’s Notes are removed: Editor’s note: It’s FFS if other entities could also get the energy related subscription data. Editor’s note: Further details of possible policy decisions are FFS. By proposing that the energy related subscription data is ”Energy Saving” indicator and the AMF receives the indicator and forwards it to NG-RAN and PCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to use the name “Energy Saving” indicator as the energy related subscription data. The Energy Saving indicator is per UE granularity. The AMF receives the indicator together with the UE subscription data. The AMF forwards the inidcator to the NG-RAN and to the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The subscription and policy control for energy efficiency and energy saving is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y (new), 5.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | S2-2410192, S2-2410752, S2-2410801, S2-2411056, S2-2412094, S2-2412518 |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.X.Y	Subscription information aspects

The Subscription data for a UE may include an Energy Saving indicator  that the UE is subject to network energy saving operation. The AMF receives this indication in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF.

Editor’s note:	. The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. Multiple values could e.g. allow configuring in a PLMN different energy saving behaviours like: only affecting RAN behaviour; or only affecting the AM policies for the UE; or both; or to have possibility to select among several RAN behaviours of the AM policies.

Editor’s note:	Whether the indicator is provided to the NG-RAN and how  it is used in the NG-RAN is FFS.

### 5.X.Z	Policy control aspects

During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator  as described in clause 5.x.y. Based on the Energy Saving indicator and potential energy related notifications from EIF, the PCF may make policy decisions reusing the existing parameters based on the operator policy, e.g. modify the RFSP index or the UE-AMBR in order to achieve anergy saving.

Editor’s note: The notification for the energy related information, if any, should not be provided from the UDM/UDR. The details of notification for the energy related information, e.g. what the notification would be, or which entity (e.g. EIF) provides the notification, is FFS.

# * * * End of changes * * * *