3GPP TSG-SA WG2#166	S2-2413025

Orlando, FL,USA, 18-22 November 2024	(was S2-2412782)

Source:	Qualcomm Incorporated

Title:	Key issue #1 conclusion updates

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes updates to the key issue 1 conclusions.

# 1.	Text proposal

It is proposed to agree the following changes to TR 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

### 8.1.3	Architecture to Support Topology 2

#### 8.1.3.1	General

The following principles apply:

-	Architecture: Two architecture options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

-	Subscription aspects

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor’s note: Additional subscription information, e.g. validity information, for the UE Reader is FFS.

-	UE Reader subscription information is available to AMF and AIoTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Editor’s note: Whether and how to enable authorization to the UE is FFS.

-	Radio resource management for UE Reader operation

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g. taking into account assistance information received from the AIoTF) from the gNB.

NOTE:	Assistance information is further clarified in other clauses.

NOTE:	The details of UE Reader radio resource management will be defined by RAN2.

#### 8.1.3.2	User-plane option

The following additional principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

-	To operate as a UE Reader, the UE establishes a PDU Session to a specific DNN/S-NSSAI and establishes an association with the AIoTF identified by an FQDN using the AIoT AP protocol. In this release, the UE is preconfigured with the specific DNN/S-NSSAI and the AIoTF FQDN.

Editor’s note:	Whether and which additional methods to dynamically provision the UE with this information can be specified and in which release is FFS.

-	If the AIoTF determines that the UE is not authorized to operate as a UE Reader according to the UE’s subscription, then the AIoTF rejects the UE’s association request.

-	If the AIoTF detects that the Reader function in the UE does not respond to a request by the AIoTF (e.g., an Inventory Request or Command Request), then the AIoTF considers the UE Reader unreachable und locally deletes the association for the UE Reader.

routing

#### 8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIoTF. The related protocol stack is shown in Figure 8.1.3.3-1.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details how the RRC option works are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option

# >>>>END OF CHANGES<<<<