3GPP TSG-SA WG4 Meeting #130	S4-241842

# USA, Orlando, 18 – 22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.956 | CR |  | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on Scenario: UE-to-UE Beyond 2D Video Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Philips, Interdigital, Deutsche Telekom, Fraunhofer HHI, Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Beyond2D |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An evaluation scenario is proposed that handles the UE-to-UE Beyond 2D (B2D) Video communication that provides experiences beyond what is achievable with 2D content. The scenario allows to evaluate the real-time communication of B2D video content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause with scenario description on UE-to-UE Beyond 2D Video Communication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Real-time communication of B2D video content cannot be evaluated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Initial text on this scenario is included in the PD (v0.0.4) clause 2.4. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

== CHANGE 1 (all new) ===

## 7.x	Scenario x: UE-to-UE Beyond 2D Video Communication

### 7.x.1 Scenario name

UE-to-UE Beyond 2D Video Communication

### 7.x.2 Motivation for the scenario

The proposed scenario handles the UE-to-UE Beyond 2D (B2D) video communication that provides real-time experiences beyond what is achievable with 2D content.

B2D content may be in the form of volumetric video, which is a frame-based immersive experience whereby each frame represents a volumetric region in 3D space in which any point is either non-occupied or having a color that may depend on the viewing direction. Real-time communication of volumetric video has been previously considered in 3GPP for some use cases described in TR 26.928 [R1] and TR 26.998 [R2].

What is the market relevance of the proposed scenario within the next few years? Are there any commercially available or pre-released products or prototypes?

Market relevance key indicators:

- Technology evaluation on the market

Are there indications of pre-evaluation by service providers, device manufacturers, and/or network operators?

- Forma Vision offers a platform that offers face-to-face B2D meeting experiences: https://www.formavision.io/

- Nokia: https://www.nokia.com/blog/3d-live-communication-becomes-part-of-everyday-life-with-volumetric-video/

- 8i offers a B2D conferencing platform: https://8i.com/communication/

- Google is partnering with HP to enable their Starline technology for video conferencing: https://blog.google/technology/research/google-project-starline-hp-partnership/

- Industry activities

Is there relevant work in 3GPP MRPs, industry collaborations or among market stakeholders?

- 5G-MAG provides a “V3C immersive platform” which contains reference tools for volumetric video. The tools are provided as a Unity package that includes a decoder plugin library and a player that includes an MIV renderer.

  - https://www.5g-mag.com/post/an-introduction-to-the-v3c-immersive-platform

  - More detailed information is in the LS S4-241819 sent from 5G-MAG to 3GPP SA4.

- Production tools/companies

What is the availability of capturing setups, and  production software? Are there endorsed formats for representation, contribution, compression, and storage? Is there an ecosystem of content creators?

Capture setups/devices and tools are made available by vendors such as:

- Apple: https://developer.apple.com/documentation/avfoundation/additional_data_capture/streaming_depth_data_from_the_truedepth_camera

  - TrueDepth camera provides depth data in real time that allows you to determine the distance of a pixel from the front-facing camera.

- Google: https://developers.google.com/ar/develop/java/depth/developer-guide

  - Depth API helps a device’s camera to understand the size and shape of the real objects in a scene.

  - Note: Android depth API does not require presence of dedicated ToF sensor

Two representation formats are considered for this scenario:

- Stereoscopic video as defined in clause 4.3.2

- Multi-view video as defined in clause 4.3.4

Contribution, compression and storage formats are available for both representation formats, see clause 7.x.3.

- Delivery solutions

Which delivery type is expected to be used? What are the expected transport formats? Is there SW or HW support and providers?

It is expected that RTP-based delivery will be used. RTP payload formats for the considered codecs for stereoscopic video and multi-view video are available.

- Content decoding and rendering

Is there decoding SW/HW support, and providers? Are there rendering devices and displays available yet?

Hardware video decoder capabilities can be used for all pixel data.

Rendering and display systems are available. Those are described for stereoscopic video in clause 4.3.2.3 and for multi-view video in clause 4.3.4.3, respectively.

### 7.x.3 Description of the scenario

This provides a description of beyond 2D video end-to-end workflows, which includes identifying and defining beyond 2D formats being used in the context and representation technologies to delivery these formats. The following aspects may be considered for each workflow:

This scenario considers UE-to-UE format exchange (conversational). Typically, the exchange is symmetrical and both UEs capture and render video.

In this scenario (Figure 7.x.3-1), a first UE captures beyond 2D video and transmits to a second UE that renders one or two viewports of the transmitted scene.

Figure 7.x.3-1: Direct UE->UE communication [Ed.(BK): The figure will be redrawn if needed for copyright reasons.]

- Capturing and processing

Capture setup, production tools and workflows are linked to the representation format.

-	For stereoscopic video, capture systems and production tools are described in clause 4.3.2.2.

-	For multi-view video, capture systems and production tools are described in clause 4.3.4.2.

- Encoding

Contribution, compression and storage formats are linked to the representation format.

Well-defined contribution formats exist that carry the raw texture/depth images and camera parameters, e.g. as  described in clause 4.3.4.2.

-	For stereoscopic video, compression formats are described in clause 4.3.2.4.

-	For multi-view video, compression formats are described in clause 4.3.4.4.

One codec that may be considered is the multiview extension of HEVC (MV-HEVC).

Another codec that can be used to realize this scenario is MPEG Immersive Video (MIV) specified in ISO/IEC 23090-12.

The multiple camera views and depth maps (if available) are encoded to create a unified representation. An example could be MIV constrained to one atlas and packed video data. The single video sub-bitstream would be encoded with HEVC Main10 profile. The bitstream would contain all camera parameters that are necessary for 6DoF rendering.

An example workflow for real-time encoding of multi-view video is described in the paper [R3].

In addition to MV-HEVC and MIV, other codecs may be studied as part of this scenario.

- Packaging and delivery

The encoded bitstream is packaged according to the RTP payload format of the used codec.

For example, if MIV codec is used, MIV bitstream may be packaged as a sequence of V3C units including V3C units for the packed video data, or the MIV bitstream can be packaged as three separate tracks, one for common atlas data, atlas data, and packed video data.

IETF proposal avtcore-rtp-v3c [R4] enables carriage of MIV as a RTP stream. In this proposal at least the V3C parameter sets (VPS) and possibly also atlas data is carried over SDP. Integration in WebRTC is possible.

The common atlas data with camera parameters will only change infrequently (once per second or less), for instance each time an online camera calibration is updated. While it is possible to transmit common atlas data at non-IRAP frames, this is not expected in this scenario.

The atlas data with patch information may be statictransmitting only full views or dynamic when an encoder selects regions of the source views for transmission based on e.g. occlusion detection or depth segmentation. In MIV, patch information is always intra-coded (unlike V3C in general).

- Decoding

The decoder(s) will make use of hardware video decoder capabilities for pixel data, and metadata describing information needed for rendering is decoded/parsed by a CPU.

- *Post-processing

- Rendering

- For stereoscopic video, rendering and display systems are described in clause 4.3.2.3.

- For multi-view video, rendering and display systems are described in clause 4.3.4.3.

- General constraints on latency, bandwidth, reliability and complexity

The codec should support a low-delay reference frame structure. End-to-end latencies below 500 ms may be considered suitable for communication.

For example, the MIV access units and the video sub-bitstream can be organized using a low-delay reference frame structure. All sub-bitstreams could have the same GOP structure, but atlas data and common atlas data frames may be skipped.

All encoder, decoder and renderer processes are real-time and may have a latency of at most 500 ms.

Bitrates between 5 and 50 Mbit/s should be considered.

### 7.x.4 Supporting companies and 3GPP members

[Ed.(SG): Remove this clause while integrating to the TR]

- This documents the 3GPP members that support this scenario in terms of providing the information, test material, test requirements and the characterization for the tests. For each of the identified necessities, a tick box is created in the template.

- Preferably several 3GPP members are included in the support, and in addition a video service provider may be included (not necessarily a 3GPP member).

- Cross-verification is preferably done by the supporters of the scenario

Deutsche Telekom, Fraunhofer HHI, InterDigital, Nokia Corporation, Philips, Sony

### 7.x.5 Source format properties

This defines a clear range of the considered and relevant source formats, including the signal properties, but also the characteristics of the content. As an example, the texture and depth format properties of the source may be used which include:

The source format has 2 views with perspective projection (PSP). Study of 1, 3 or 4 views is optional.

Each view has the following components:

- Required: Texture (color)

- Optional: Depth coded as normalized disparity or linear normalized depth.

- Optional: Object ID map with ordinal values, 0 indicating "invalid pixel" (to support depth cameras), 1 indicating "background" and all other values indicating "foreground object i".

All views have view parameters: camera ID, camera intrinsics, camera extrinsics (pose) and depth quantization parameters (optional).

Views may be undistorted, otherwise distortion parameters have to be provided.

For stereoscopic video, the signal properties defined in clause 4.3.2.1 apply with the following constraints:

- Frame rates: 25 or 30 fps

For multi-view video, the signal properties defined in clause 4.3.4.1 apply with the following constraints:

- Frame rates: 25 or 30 fps

### 7.x.6 Encoding and decoding constraints and settings

Typical encoding constraints and settings such as

- Relevant Codec and Codec Profile/Levels according to TS26.119

MV-HEVC: HEVC Multiview Main

MIV:

- HEVC Main 10 MIV Main

  - MIV level 2.0 or 2.5.

- HEVC Main 10 MIV Extended

  - MIV level 2.0, 2.5 or 3.0 whereby the level 3.0 is only allowed if there is a single video sub-bitstream.

- Random access frequency

Typical random-access frequency of 32 frames can be considered. It is up to the service provider to define the exact random access frequency.

- Error resiliency requirements

No specific requirements.

- Bitrates and quality requirements

Typically, bitrates between 5 and 50 Mbit/s may be considered.

- Bitrate parameters (CBR, VBR, CAE, HRD parameters)

MIV: Bitrate parameters related to video sub-bitstreams need to be configured by the streaming service provider. Transfer characteristics are signalled in the video sub-bitstreams. Video sub-bitstreams encoding may be independently configurable and possibly independently decodeable. Video sub-bitstream containing geometry information should be encoded using full range color space instead of studio-swing

- ABR encoding requirements (switching frequency, etc.)

No specific requirements. Since RTP is used, ABR may be achieved by real-time congestion algorithms e.g. as in WebRTC.

- Latency requirements and specific encoding settings

Picture reordering is not allowed.

E2E latencies below 500ms are acceptable.

- Encoding context: real-time encoding, on device encoding, cloud-based encoding, offline encoding, etc.

Real-time encoding is required.

- Required decoding capabilities

# It is expected that devices support HW accelerated video decoding.

MV-HEVC:

- HEVC Multiview Main

- HEVC level 5.2 would enable 16 texture or 8 texture + depth at 1080p30

# MIV:

- HEVC Main 10

- HEVC levels are determined according to the maximum HEVC Level that is needed for a video sub-bitstream decoder to fulfill the MIV level.

  - HEVC level 5.1 for MIV level 2.0

  - HEVC level 5.2 for MIV level 2.5

  - HEVC level 6.1 for MIV level 3.0

- Video sub-bitstreams need to be independently decodable. This helps implementations on various platforms that may have only high-level APIs. For instance, geometry needs to be full range.

- Synchronization requirements

MIV: Samples in the sub-bitstreams should be temporally aligned.

### 7.x.7 Performance Metrics and Requirements

- A clear definition on how the performance needs to be evaluated including metrics, etc addressing the main KPIs of the scenario.

[Ed.(BK): To be aligned with the agreed evaluation framework.]

The tests are run for a chosen level as described in clause 7.x.6. Bitstreams are provided. Camera calibration, depth estimation, and encoding are not evaluated.

The test will have four rate points and QP values are selected for each sequence to approximately match the 5 to 50 Mbps range. When saturation occurs before 50 Mbps a lower value may be chosen in consultation. When there are multiple video components or packed regions then the other QP values need to be directly derived from the texture QP using an equation or look-up table. (They cannot depend on the sequence.)

- Objective measures such as PSNR, VMAF, etc, may be used.

[Ed.(BK): To be aligned with the agreed evaluation framework.]

The IV-PSNR tool, available at https://gitlab.com/mpeg-i-visual/ivpsnr, is available to compute full-reference objective metrics:

- Weighted sphere PSNR (WS-PSNR)

- Immersive video PSNR (IV-PSNR)

All source views that were used for encoding are provided. Each source view is reconstructed by decoding and rendering (view synthesis). The IV-PSNR tool is then run on all source views and the score is averaged over all views.

Depending on bit rate, quality of depth maps and rendering, either the video codec or view synthesis is the limiting factor. BD-PSNR is calculated for both metrics because the metric behaves more predictably than BD-rate.

- Justification on whether objective metrics are sufficient and representative of the subjective performance.

[Ed.(BK): To be aligned with the agreed evaluation framework. The discussion here on correlation of objective and subjective metrics may need to be moved to that framework after more deliberation.]

There is experience in testing of immersive video in MPEG context. The test conditions as described are a simplification and evolution of the common test condition for MIV defined in [R5].

The main challenge with testing of Beyond 2D video is that codecs are asymmetric. The input is a number of source views (with depth maps), and the output of the decoder + renderer can be any viewport within a spatial region around those source views. In the mentioned CTC two tests are used:

- Objective evaluation at source view positions

- Subjective evaluation of pose trace videos (dynamic viewports)

This has resulted in a lack of correlation between objective and subjective results, but despite that it is the best-known approach. Alternatives that have been tried and dismissed (for now):

- Objective evaluation at dynamic viewports: It includes view synthesis in the reference condition and this skews the results towards a specific renderer. It prevents an A/B comparison of different renderers.

- Subjective evaluation at source view positions: This is not how the end-user will interact with the content, and it does not evaluate artifacts due to viewport dynamics.

For this test, because the aim is to prove feasibility of a scenario, objective evaluation may be sufficient, especially when supplemented with (informal) real-time demonstration of the same bitstreams that were used for objective evaluation.

### 7.x.8 Interoperability Considerations for the application

- Streaming with DASH/HLS/CMAF/QUIC

- RTP based delivery

The B2D video bitstream needs to be carried over RTP with SDP for this scenario. WebRTC can be considered. It is not necessary to prove this as part of the feasibility test, if written evidence can be provided.

In the example of using MIV as a codec, there are implementations RTP + SDP by Nokia and an open source implementation by Tampere University: https://github.com/ultravideo/uvgRTP.

### 7.x.9 Test Sequences

A set of selected test sequences that are provided by the proponents in order to do the evaluation. 	They should cover a set of source format properties

Test sequences that were used during the development of a codec are discouraged because they may create a bias towards that specific codec. Sequences that were used in a verification test are permissible.

Preferably test sequences match with the intended use case both in terms of technical requirements and content semantics.

[Ed.(SG): For multi-view video, a test sequence captured using the system described in [R3] will be provided.]

### 7.x.10 Detailed test conditions

Provides a proposal for detailed test conditions, for example based on a reference software 	together with the sequences and configuration parameters.

[Ed.(BK): To be aligned with the agreed evaluation framework.]

For each candidate codec, a suitable decoder + renderer needs to be made available for testing purposes.

For MIV a reporting template or script will be provided to compute BD-PSNR based on IV-PNSR log files of all rates and sequences.

For MIV the common test conditions defined in [R5] are followed.

### 7.x.11 External Performance data

References to external performance data that can be added, for example other SDOs, public documents and so on.

For MIV the performance data is available from the verification test report [R6].

### 7.x.12 Additional Information

- Industry activities

Is there Relevant work in industry forums?

See clause 7.y.12 [Ed.(SG): y refers to the clause number for the scenario in S4-241841]

- Implementation constraints

Are there any indications about scalability of the technology with regards to network and devices?

The technology is expected to be highly scalable since it uses well-established transport technologies like RTP and 2D video coding techniques.

- Innovation

Does the technology address a current or a future need on the market? Can it potentially disrupt existing markets?

B2D real-time video communication has many applications in different domains. Having lifelike communication between UEs will reduce the need for travel and contribute to sustainability goals.

== CHANGE 2 ===

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

…

[R1] 3GPP TR 26.928: “Extended Reality (XR) in 5G”.

[R2] 3GPP TR 26.998: “Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices”.

[R3] L. Ilola et al., "Real-time Delivery of Visual Volumetric Video-based Coding Data," 2023 IEEE International Symposium on Multimedia (ISM).

[R4]  RTP Payload Format for Visual Volumetric Video-based Coding (V3C), https://datatracker.ietf.org/doc/draft-ietf-avtcore-rtp-v3c/

[R5] Dziembowski, B. Kroon, J. Jung (Eds.), Common test conditions for MPEG immersive video, ISO/IEC JTC 1/SC 29/WG 04 N 0372, July 2023, Geneva.

[R6] D. Mieloch (Ed.), Verification test report of MPEG immersive video, ISO/IEC JTC 1/SC 29/WG 04 N 0341, April 2023, Antalya.

[Ed.(SG): Consider while integrating to TR that some of the above references overlap with the ones given in S4-241841 for the streaming scenario.]