3GPP SA4 #130	S4-241912

Orlando, FL, US, November 18-22 2024                                     Revision of S4aR240073

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.854 | CR | pseudo | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_HapticsMedia] Support for existing actuators with proprietary haptic media formats in cloud rendering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_HapticsMedia |  |  |  |  |  | Date: |  |  | 10/09/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A large portion of the currently depolyed actuators use respective proprietary haptic media formats. To facilitate cloud-based rendering for haptics in commercial deployment, it is important to support these actuators without changing them and with mimimal changes to the devices. There were comments from SA4 RTC Ad Hoc meeting on Oct 23, 2024: - The comments: - “We aim to achieve interop here so not sure why we would look into proprietary formats.” - “Why need to manage haptics different than audio/video? Why need to have the proprietary transcoding in the cloud?” - Response: “Want to minimize changes to the device and no change to current actuators.” There is a need to further clarify the motivation for supporting proprietary haptic media formats. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the issue and potential solutions; add further clarification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 3GPP SA4 spec expected to come out of the study item may not be commercially relevant for the near term. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change (all new) * * * *

### 6.x.x	Standard support for existing actuators with proprietary haptic media formats in cloud rendering

There are many actuators currently depolyed on devices such as smart phones and wearables, and a large portion of them use proprietary haptic media formats. This is not an issue if the haptic media is created on a device and fed to an actuator on the same device. This is shown in Figure 6.x.x-1 (a). With split rendering, the split rendering server needs to generate the haptic media along with other types of meida such as video and audio. This gives rise to the issue of how to make the rendered haptic media recognizable to the actuator.

It is unrealistic to mandate the adoption of standardized haptic media formats by a wide variety of existing devices. Also, it is unrealistic to mandate such adoption by the future devices yet to be released in the near term. To facilitate mass commercial deployment of cloud-based rendering for haptics, the haptic media generated by the cloud needs to be in a format such that (1) there is no need to change the actuators, and (2) the changes to the devices is minimum.

There are two design approaches:

- The device converts the haptics media from the format used by the cloud server to the format that is recognizable to the actuator.

- The cloud generates the haptics media in the format that is recognizable to the actuator.

The second design approach is preferred, because the device does not have to implement the format converter. This is shown in Figure 6.x.x-1 (b).

Ahtough for many existing devices the supported haptic media formats is proprietary, it is beneficial to develop a standardized interface and procedure to enalble (b).

NOTE: This does not mean that there is no need for developing standardized haptic media formats, which may be beneficial for devices to be released in the long term.

| (a) | (b) |
| --- | --- |

Figure 6.x.x-1: (a) the control logic generates a haptic media in a proprietary format as the input to the actuator; (b) the cloud server generates a haptic media in the proprietary format of the actuator and the haptic media passes through the control logic.

# * * * * end of 1st change * * * *