Source:	InterDigital Communications

Title:			[SR_IMS] Discussion on delay adjustment based on QoE metrics

Agenda Item:		10.5

Document for:		Discussion

# 1	Introduction

User Quality-of-Experience (QoE) is a key indicator for the adoption of eXtended Reality (XR) applications over a 3GPP network. 3GPP Release 18 (section 2 below) defines Quality-of-Experience (QoE) metrics based on delay (e.g., pose-to-render-to-photon, roundtrip-interaction delay), associated delay thresholds to guarantee the corresponding QoE, as well as corresponding QoE measurement procedures.

Delay adjustment based on QoE metrics measurement (also referred as QoE-based delay adjustment) is a key functionality to maintain an acceptable QoE for the user, when network conditions are changing (congestion, retransmission due to packet errors).

The SR_IMS work item [1] justification and objective 4 details the need to achieve a seamless QoE experience for the user from adjusting the split rendering operations:

- The justification introduces the need for adjusting the split rendering operations to guarantee the QoE based on changes in UE or server capabilities or network conditions: “Split rendering can leverage rendering resources in a network function as well as the rendering resources in the UE to provide a seamless experience. Adjusting the split of the rendering operations based on changes in UE or network conditions may enable a consistent QoE for the end users”

- Objective 4 defines QoE adaptation/adjustments procedures: “procedures for the adaptation of split rendering operations between the split rendering client and the split rendering server” (e.g., depending on the type of device, battery level, content type, actions).

This contribution introduces (section 3) adjustment of processing delays of the XR scene rendering operations to guarantee the QoE. The objects of an XR scene are typically represented within a node graph having several representations. These object representations have a significant impact on the processing delays of the XR scene rendering operations. An XR application can select different object representations corresponding to a range of processing delays to adjust the latencies involved in the XR scene rendering operations (e.g., pose-to-render-to-photon, roundtrip-interaction delay).

The contribution shows how a QoE-based delay adjustment may be supported in a Split Rendering IMS architecture. A call flow proposal for QoE-based delay adjustment between an SR-DCMTSI client and an MF is provided in the contribution S4aR240101.

# 2	QoE latency metrics for XR applications

XR is mostly about immersion, creating a physical presence in a non-physical world. The users should not experience a delay between the physical movement and the display output to have the best immersion. Otherwise, the sense of physical presence in a virtual world would be lost. For example, high motion-to-photon latency causes spatial disorientation and dizziness, referred to as Virtual Reality (VR) sickness or motion sickness.

The impact of latency on the user QoE and the corresponding acceptable delay thresholds are well documented in clause 4.2 of TR 26.928 [2]. Latency requirements of less than 20ms for the motion-to-photon delay, and between 50ms and 200ms for the roundtrip interaction delay have been defined.

The measurement of QoE metrics based on roundtrip delay such as the pose-to-render-to-photon and the roundtrip interaction delay are defined in clause 9.3 of TS 26.565 [3] and clause 11 of TS 26.119 [4]. These measurements are based on time information gathered during the roundtrip at the four Observation Points (OPs) defined in the XR Baseline Client (Figure 1) of TS 26.119 [4].

Figure 1: Observation Points in the XR Baseline Client

QoE latency metrics, their corresponding target values to provide a good user QoE, and their measurements during the rendering loop in an XR Baseline Client have been addressed in 3GPP Release 18. The remaining part is the definition of an efficient delay adjustment to maintain an acceptable QoE for the user, when network conditions are changing (congestion, retransmission due to packet errors).

# 3	Processing delays of an XR scene with different object representations

An XR scene is composed of objects (e.g., geometries, media, light sources) to be rendered by considering animation, user interactivity, physics simulation for interactive immersive XR experiences.

An XR scene is typically represented by a node graph (Figure 2):

- An object corresponds to a node (or a group of nodes) of the graph, each node providing spatial parameters (translation, rotation, scale) to place the object in the XR scene

- The spatial relationships between the objects correspond to the connections between the nodes of the graph

This node can further contain several components (e.g., mesh, material, texture…) which provide the geometric and material data for the rendering of the object.

Figure 2: glTF node graph representation of an XR scene

During the rendering loop, a typical game engine application (e.g. Unity 3D, Unreal Engine)

- updates the graph representation of the XR scene by considering animation, user interactivity, physics simulation,

- processes culling techniques (e.g., camera frustrum, back-face) with respect to the user current point of view to exclude objects or portions of the XR scene that are not visible to the user,

- renders the XR scene with the proper lighting conditions.

An object may have several representations (i.e., several variants of its geometry, texture, media). These object representations are often called object Levels-of-Details (LoDs) to be selected by the XR application for optimizing the trade-off between rendering quality and processing delay during the runtime. This trade-off between object rendering quality and processing delay is mentioned in the introduction section of the Level of Detail for 3D Graphics book [5] p.30:

“when rendering, use a less detailed representation for small, distant, or unimportant portions of the scene. This less detailed representation typically consists of a selection of several versions of objects in the scene, each version less detailed and faster to render than the one before.”

The criteria to select one object representation among the others may vary and is application dependent.

For example, a distance criteria from the current user viewpoint can be used to represent far objects with lower geometry and/or texture details with respect to close objects as shown in Figure 3.

Figure 3: LOD based on the number of mesh triangles with a distance criteria from Level of Detail for 3D Graphics book [5]

LoD generation of 3D asset is widely supported by the Digital Content Creation (DCC) tools such as Autodesk Maya, Unreal Engine, Houdini, Unity 3D.

A content creator may provide different objects LoDs in a scene description file for portability purpose between game engines. For example, the LoD functionality is supported by the Web3D X3D v4 with the LOD node and by the Khronos glTF with the Microsoft MSFT_lod extension.

Figure 4 provides an example of creating three LOD levels for an object based on a screen coverage criteria using the glTF MSFT_lod extension:

- The “High_LOD” object representation is rendered for a screen coverage from 0.5 to 1.0,

- The “Medium_LOD” object representation is rendered for a screen coverage from 0.2 to 0.5,

- The “Low_LOD” object representation is rendered for a screen coverage from 0.01 to 0.2,

- The object is not rendered for a screen coverage below 0.01.

"nodes": [

{

"name": "High_LOD",

"mesh": 0,

"extensions": {

"MSFT_lod": {

"ids": [

1,

2

]

}

},

"extras": {

"MSFT_screencoverage": [

0.5,

0.2,

0.01

]

}

},

{

"name": "Medium_LOD",

"mesh": 1

},

{

"name": "Low_LOD",

"mesh": 2

}

]

Figure 4: Example of glTF MSFT_lod extension defining three level of details

To point out that this glTF MSFT_lod extension is generic in the sense that the selection criteria and the corresponding thresholds are provided in a MSFT_screencoverage metadata specified in extras parameter (i.e., non-normative). Other criteria (e.g., a QoE latency metric-based criteria) may be defined by a content creator to select these three object representations.

As described, the LoD mechanism handling multiple representations of an object is a generic method to introduce processing delay flexibility by modifying the object rendering quality in a controllable manner.

An XR scene may contain many objects having multiple LoDs, leading to offer to the XR application a wide range of processing delay capabilities.

# 4	Conclusion and Proposal

Delay adjustment based on QoE latency metrics measurement, defined as QoE-based delay adjustment above, is a key functionality to maintain an acceptable QoE for the user when the network conditions are changing (congestion, packet retransmission due to errors).

QoE-based delay adjustment allows an XR application to select different representations of the XR scene objects to adjust the processing delay of an XR scene rendering operations.

SR-IMS procedures for adaptation of split rendering are defined in clause 7.3 of TS 26.567. Our proposal is to extend adaptation of split rendering procedures to support QoE-based delay adjustment based on the availability of multiple object representations.

A call flow proposal for QoE-based delay adjustment between an SR-DCMTSI client and an MF is provided in the contribution S4aR240101.

# 5	References

[1] 3GPP SP-240492 “Work item description for Split rendering over IMS”.

[2] 3GPP TR 26.928 “Extended Reality (XR) in 5G”

[3] 3GPP TS 26.565 “Split Rendering Media Service Enabler”

[4] 3GPP TS 26.119 “Device Media Capabilities for Augmented Reality Services”

[5] Level of Detail for 3D Graphics, David Luebke, Martin Reddy, Jonathan D. Cohen, Amitabh Varshney, Benjamin Watson, Robert Huebner, Book, 2003