Title:		          TR 26.933 Review

Source:	Panasonic Holdings Corporation

Document for:	Discussion & Agreement

Agenda item:	7.6

1. Background

In the SA4 #129-e post meeting, a contribution (S4aA240046) was submitted, identifying gaps and inconsistencies in the TR. As a continuation, further review was performed, data has been collected for inclusion into TR 26.933 [1].

2. Discussion

Option 1:

TR 26.933[1] tried to capture information on the following aspects.

- Clause 4: Size and Structure Evolution of UEs: This clause reviews wide range of UE categories size and structure.

- Clause 5: Microphones used in audio capture: This clause discusses various microphone types widely used in audio capture

- Clause 6: Audio Capture format: Different capture formats from stereo to spatial audio capture were discussed

- Clause 7: Microphone Integration in UEs: Compared to clause 4, this clause limits the scope of microphone integration to few UE categories such as mobile phones, headphones, tablets, laptops, watches

- Clause 8: Signal Processing: This clause touches following in a high-level AEC, Microphone Array Signal Processing, Beamforming, Noise reduction.

- Clause 9: Example audio capture processing solutions: Capture solution for end-user devices were discussed

Clause 4 reviews the size and structural evolution of various UEs, while Clause 6 examines different audio capture formats. However, it remains unclear how these capture formats relate to or influence the data provided in Clause 4. Additionally, one of the TR’s objectives is to study the supported audio formats for UEs—including mono, stereo, binaural, MASA, multi-channel, object-based, and scene-based audio—produced by different audio capture configurations. Despite some high-level clarification attempts in Clause 8, it remains unclear to the source how raw microphone array capture by different UEs can be effectively used for IVAS MASA, multi-channel, object-based, or scene-based audio.

Given the absence of these details in the TR, the source believes it would be practical to limit the scope of the study item to mono, stereo, and binaural formats, as these are the formats UEs can readily support without new additional signal processing. If the group believes that study item needs to be concluded by this meeting, the source proposes to limit the scope of the study accordingly.

Option 2:

It is to be noted that 3GPP IVAS supports following coding modes

Table 1: IVAS coding Modes

| Coding modes | Full Name |
| --- | --- |
| Mono | EVS/IVAS Mono |
| Stereo | Stereo Operation |
| SBA | Scene-based Audio (SBA, Ambisonics) Operation (1-3) |
| MASA | Metadata-assisted Spatial Audio (MASA) Operation (1 or 2 channels) |
| ISM | Objects (Independent Streams with Metadata, ISM) Operation |
| MC | Multi-Channel (MC) Operation (5_1, 7_1, 5_1_2, 5_1_4, 7_1_4) |
| OMASA | Combined Objects and MASA (OMASA) Operation |
| OSBA | Combined Objects and SBA (OSBA) Operation |

After reviewing the TR [1], the source finds it unclear how IVAS coding modes—such as SBA, MASA ISM, OMASA and OSBA-- fit different UE categories.

The minimum number of microphones required for generating specific coding modes like MASA, ISM, etc., is also unspecified. Additionally, guidance is lacking on how UEs with more than two microphones can be used to capture and generate IVAS suitable coding modes like Scene-Based audio or MASA input. The source believes that this information is essential to meet the objectives agreed upon during the study item phase.

To start the discussion on the above aspects, source has tried to map suitable coding modes with respect to UE’s capturing capabilities.

Mobile phone UE

Clause 7 covers the number and placement of microphones for each UE category. According to Clause 7.1, modern mobile phone UEs can support up to 4 microphones, enabling various capture configurations, including mono, binaural, stereo, and multi-channel capture. With up to 4 microphones, these UEs can potentially support IVAS coding modes such as mono, binaural, stereo, ISM, SBA, and MASA. However, to make these captured signals suitable for IVAS coding modes, further processing is required, which is currently not clearly captured in the TR. Clarification on the following would be beneficial, as it remains unclear how immersive audio can be achieved from the raw multi-channel capture.

- Minimum and maximum number of raw audio microphone inputs needed to convert to 3GPP IVAS MASA format. Similarly for ISM, SBA

- Steps to generate the metadata for the IVAS coding format

- Suitable coding format per capture mode.

- Signal processing steps (from UE’s raw multi-channel capture to IVAS coding mode) required to produce MASA, ISM, SBA signals, etc.

Headphone UE

Clause 7.2 highlights microphone integration for the headphone UE category, including the number and placement of microphones. Although up to 9 microphones may be available in this category, the source believes that the most suitable capture format would likely be binaural or mono, depending on the actual number of microphones on the UEs.

If group believes that more than binaural can be achieved for headphone UE category, clarification may be needed suitable IVAS coding formats and processing steps for conversion to IVAS coding modes.

Tablets UE

Clause 7.3 highlights microphone integration for the Tablets UE category, including the number and placement of microphones. This UE category supports up to 5 microphones, possible capture using UE’s microphone will be: Mono (1) or stereo (2) or multi-channel capture (min 3 – max 5)., with more than 2 microphones enables various capture configurations similarly to mobile phone UE category as discussed above. Microphone array signal processing steps for IVAS coding modes are essential to transform raw UE microphone captures into an immersive audio input format.

Laptops UEs

Clause 7.4 details microphone integration for the Laptop UE category, including the number and placement of microphones. This category can support up to 3 microphones, enabling possible capture configurations of mono (1 microphone), stereo (2 microphones), or multi-channel (3 microphones). With more than 2 microphones, suitable IVAS coding modes such as ISM (1/2), SBA (2D), and MASA can be achieved. However, as with other categories, Microphone array signal processing steps for IVAS coding modes are essential to transform raw UE microphone captures into an immersive audio input format.

Watches

Clause 7.5 describes microphone integration for the Watches UE category, including the number and placement of microphones. This category supports up to 2 microphones, allowing for capture configurations such as mono (1 microphone), stereo or binaural (2 microphones). However, as with other categories, Microphone array signal processing steps for IVAS coding modes are essential to transform raw UE microphone captures into an immersive audio input format.

XR devices

No information was captured in TR in clause 7

Cars

No information was captured in TR in clause 7

Summary

To meet the study item objectives, following needs to be clarified in the TR under appropriate clause if group considers option 2 as the way forward. Alternatively, limiting the scope of TR to stereo/binaural capturing may be a viable path forward.

- Minimum and maximum spacing between microphones per UE category for achieving immersive audio capture.

- Minimum and maximum number of raw audio microphone inputs needed to convert to 3GPP IVAS MASA format. Similarly for ISM, SBA.

- Given that each UE has different dimension and microphone spacing, clarification on suitable IVAS coding modes per UE category is needed. Signal processing steps (from UE’s raw multi-channel capture to IVAS coding mode) required to produce MASA, ISM, SBA signals, etc.

- Steps to generate the metadata for the IVAS coding format.

- Suitable coding format per capture mode.

References

[1] S4aA240055:TR 26.933.v.1.3.0