3GPP TSG-SA5 Meeting #158 	S5-246342

Orlando, USA, 18 – 22 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0622 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Corrections of subcounters and filters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As discussed in DP (S5-246341), the subcounter and filters are specified in TS32.404. However, its definition and format are not clearly specified, which results many issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding missing subcounter definition and format Adding corrections on filter definition and format |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear definition and format may lead incorrect implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.1a (new), 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

## 4.1	Performance indicators

Performance indicators are the performance data aggregated over a group of NFs, such as, for example, average latency along the network slice. The Performance Indicators can be derived from the performance measurements collected at the NFs that belong to the group. The aggregation method is identified in the performance indicator definition.

Performance indicators at the network slice subnet level can be derived from the performance measurements collected at the NFs that belong to the network slice subnets or to the constituent network slice subnets. The performance indicators at the network slice subnet level can be made available via the corresponding performance management service for network slice subnet.

The performance indicators at the network slice level, can be derived from the network slice subnet level Performance Indicators collected at the constituent network slice subnets and/or NFs. The network slice level performance indicators can be made available via the corresponding performance management service for network slice.

When providing a communication service to a tenant, the performance indicators can be derived from corresponding performance indicators related to network slice, network slice subnet and NFs and they can be made available via the corresponding performance management service, consumed by a tenant. Tenant(s) may be associated with S-NSSAI or sNSSAIList in which case, the performance indicators are split into subcounters per S-NSSAI for individual tenant.

A performance measurement may be split into more than one subcounter and/or filter based on multiple criteria as defined at the performance measurement definition.

A performance measurement may be identified by a consumer with/without any subcounter or filter, if it is allowed by the performance measurements definition. The resulting performance measurement name shall be in the following form:

<familyName>.<measurementName>

*** START OF NEXT CHANGE ***

## 4.1a	Subcounter Naming

The separator '.' or vendor-specific separator shall be used to append the Subcounter to a given performance measurement name. The resulting performance measurement name with single Subcounter shall be in the following form:

<familyName>.<measurementName>.<Subcounter>

If more than one Subcounters are specified for a given performance measurement, it shall be specified in a reasonable order. The separator '.' or vendor-specific separator shall be used to append an additional Subcounter to a given Subcounter. The resulting performance measurement name with multiple Subcounters shall be in the following form:

<familyName>.<measurementName>.<Subcounter1>.<…>.<SubcounterN>

The Subcounter shall be an alphanumeric string with at least one alphabet character as specified by following regular expression:

"^(?=.*[A-Za-z])[A-Za-z0-9]+$"

When a performance measurement is identified with Subcounter, one measurement result, which fulfilled the requested Subcounter criteria, shall be provided by the producer. It is possible to specify a Subcounter with multiple applicable values, e.g. Cause. In this case, a list of measurement results, which fulfilled each applicable values of the requested Subcounter, shall be provided by the producer.

The following are examples of possible resulting performance measurement names with:

- no Subcounter: DRB.PacketLossRateUl

- one Subcounter with multiple applicable values: RRC.ConnEstabAtt.Cause;

- one Subcounter without multiple values: TMM.HoPrepInterReqMLB.ResOpt;

- two Subcounters with and without multiple applicable values: MM.HoExeInterFailMLB.UeCtxtRelCmd.Cause;

*** START OF NEXT CHANGE ***

## 4.2	Filters and filter naming

### 4.2.0	General

In case a performance measurement is split into more than one sub-counter based on multiple criterias, it is convenient to use Filters to define the performance measurement of interest.

### 4.2.1	Filters

Performance measurements may be subdivided by using applicable filters to form new performance measurements. Any applicable Filter(s) is(are) specified in each performance measurement definition. Performance measurements may be identified by a consumer with/without any applicable Filters, if it is allowed by the performance measurements definition.

For a performance measurement that indicates a single Filter, the separator '_' or vendor-specific separator shall be used to append the Filter to a given performance measurement name. The resulting performance measurement name shall be in the following form:

<familyName>.<measurementName>_<Filter>

For a performance measurement that indicates multiple Filters, the separator ‘_’ or vendor-specific separator shall be used between Filters. The order of the Filters is not important. The resulting performance measurement name shall be in the following form:

<familyName>.<measurementName>_<Filter1>_..._<FilterN>

It is possible to to append the Filter to a Subcounter. However, it is not allowed to have a Subcounter appended to a Filter.

### 4.2.2	Filter naming

The Filter shall be in the form: NameValue(s)

The Filter Name shall be an alphanumeric string specified in the performance measurement definition. The Filter Name shall always end with an alphabet character as specified by following regular expression:

"^[0-9a-zA-Z]*[a-zA-Z]$"

The following are examples of possible Filter Names:

-	5QI

-	QCI

-	SNSSAI, where SNSSAI represents the S-NSSAI

-	PLMN, where PLMN represents the PLMN ID

The Filter Value shall be a numeric string with one or more numeric characters as specified by following regular expressions:

"^[0-9]+$".

It is possible to specfiy a Filter in the form of Filter Name without Filter Value, or with single Filter Value, or with multiple Filter Values. When multiple Filter Values are provided for a given Filter Name, Filter Values shall be separated by ‘|’.  Filter Value ranges shall be defined in the form of ‘x-y’. Combination of using ‘|’ and ranges for the Filter Values of a given Filter Name is possible. The order of the Filter Values is not important. Any other vendor-specific multiple value form is allowed for the combination.

When a performance measurement is identified with a Filter in the form of Filter Name and Filter Value(s), a measurement result, which fullfiles all requested Filter Values of the requested Filter Name, shall be provided by the producer.

Optionally, a performance measurement may be identified with a Filter in the form of Filter Name only. In this case, a list of measurement results for each applicable Filter Values of the requested Filter Name, shall be provided by the producer.

The following are examples of possible performance measurement names with:

-	no Filters: DRB.PdcpF1DelayDl

-	single Filter as single Filter Name with no Filter Value: DRB.PdcpF1DelayDl_5QI

-	single Filter as single Filter Name with single Filter Value: DRB.PdcpF1DelayDl_5QI22

-	single Filter as single Filter Name with multiple Filter Values: DRB.PdcpF1DelayDl_5QI32|35-40

-	two Filters as single Filter Value per Filter Name: DRB.PdcpF1DelayDl_PLMN12_5QI20

-	two Filters as multiple Filter Values per Filter Name: DRB.PdcpF1DelayDl_PLMN2|5_5QI20-25

*** END OF CHANGE ***