3GPP TSG-SA5 Meeting #158 	S5-246359

Orlando, USA, 18 – 22 November 2024

# Source:	Ericsson, Vodafone, Deutsche Telekom, Telecom Italia

# Title:	Signalling traffic monitoring Report Format of the drafted TS28.abc

# Document for:	Approval

Agenda Item:	6.19.22

# 1	Decision/action requested

For approval.

# 2	References

[1]	S5-245981 new WID signalling monitoring

[2]	S5-245336, initial skeleton of draft TS28.abc signalling monitoring

# 3	Rationale

Defines Signalling traffic monitoring management report format

# 4	Detailed proposal

*** START OF NEXT CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x7]	IETF RFC8086: GRE-in-UDP Encapsulation

[x8]	IETF draft-ietf-opsawg-pcapng-04: PCAP Next Generation (pcapng) Capture File Format

*** START OF NEXT CHANGE ***

## 7.1	Reporting format

### 7.1.1	Encapsulation

Encoding of STM Payloads shall be performed using GRE in UDP Encapsulation as specified in IETF RFC8086 [x7]. The packet is encapsulated as IP header, UDP header, GRE header, and STM Payload.

The encapsulated STM Payload is shown in Figure 7.1.1-1.

Figure 7.1.1-1 STM encapsulation

Other vendor specific encapsulation is possible.

### 7.1.2	GRE Keys

Key Field in the GRE Header are used for the STM payload type. The followings are the possible values for the Key Field in the GRE Header:

| 0 1 2 3 |  |  |
| --- | --- | --- |
| 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 |  |  |
| Payload type | stmRef | R |

Figure 7.1.2-1 Key Field in the GRE Header

The Payload Type is bit 0 to 6. It is reserved for vendor specific usage of the payload type and contents.

The stmRef field uses the remaining 25 bits (bit 7 to 30). It is a unique identifier for STM object instance. Bit 7 to 14 is used for consumerId. Bit 15 to 30 is used for stmId.

Bit 31 is reserved for vendor specific usage.

### 7.1.3	STM Payload

The STM payload contains Metadata and PCAPNG payload (IETF draft-ietf-opsawg-pcapng-04 [x8]). The PCAPNG payload contains PCAPNG header and the collected signalling messages. The STM payload has following attributes, as specified in the Table 7.1.3-1.

Table 7.1.3-1: STM Payload

| STM Payload attribute name | Description |
| --- | --- |
| Metadata (O) | Metadata represented by a list of vendor specific string. |
| PCAPNG Payload (M) | PCAPNG header and the collected signalling messages. The collected signalling message is sent before security encapsulation, or received after security decapsulation. |

Editor’s note: the identification of the peer NFs of the message is FFS.

*** START OF NEXT CHANGE ***

# Annex B (informative):
Plant UML source code

## B.4	STM encapsulation

The following PlantUML source code is used to describe STM encapsulation. As depicted by Figure 7.1.1.-1:

@startuml

frame "STM encapsulation" {

rectangle "GRE header" as GRE

rectangle "STM Payload" as Payload

rectangle "UDP header" as UDP

rectangle "IP header" as IP

IP-[hidden]>UDP

UDP-[hidden]>GRE

GRE-[hidden]>Payload

}

@enduml

*** END OF CHANGE ***