3GPP TSG-SA5 Meeting #158	S5-246373

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0142 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS28.104 add MDAType Enumerations which are used as aIMLInferenceName |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For MDA type, the potential values (e.g. CoverageAnalytics.CoverageProblemAnalysis, MDAAssistedFaultManagement.FailurePrediction) are specified for each MDA capability in clause 8.4, however, the Enumeration for the attribute “mDAType” is missing, especially for stage3 definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes to add Enumerations for the attribute “mDAType” for both stage 2 and stage 3 according to the value of MDA type defined for each MDA capability in clause 8 in TS 28.104. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation issue for MDA type, especially for the scenario of reusing MDAType for aIMLInferenceName. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.X(new), OpenAPI/TS28104_MdaNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1449 at commit 3bd23bf3c464495f32b9bc8597a2c5bdbce4420b |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

8.X	Enumerations

8.X.1	MDAType <<enumeration>>

Table 8.X.1-1: <<enumeration>> MDAType

| Enumeration value | Description |
| --- | --- |
| "COVERAGEANALYTICS_COVERAGEPROBLEMANALYSIS" | Indicates that the MDA type for the Coverage problem analysis defined in clause 8.4.1.1 |
| "COVERAGEANALYTICS_PAGINGOPTIMIZATION" | Indicates that the MDA type for the Paging Optimization defined in clause 8.4.1.2 |
| "SLSANALYSIS_SERVICEEXPERIENCEANALYSIS" | Indicates that the MDA type for the Service experience analysis defined in clause 8.4.2.1 |
| "SLSANALYSIS_NETWORKSLICETHROUGHPUTANALYSIS" | Indicates that the MDA type for the Network slice throughput analysis defined in clause 8.4.2.2 |
| "SLSANALYSIS_NETWORKSLICETRAFFICANALYSIS" | Indicates that the MDA type for the Network slice traffic prediction defined in clause 8.4.2.3 |
| "SLSANALYSIS_E2ELATENCYANALYSIS" | Indicates that the MDA type for the E2E latency analysis defined in clause 8.4.2.4 |
| "SLSANALYSIS_NETWORKSLICELOADANALYSIS" | Indicates that the MDA type for the Network slice load analysis defined in clause 8.4.2.5 |
| "MDAASSISTEDFAULTMANAGEMENT_FAILUREPREDICTION" | Indicates that the MDA type for the MDA assisted failure prediction defined in clause 8.4.3.1 |
| "MDAASSISTEDENERGYSAVING_ENERGYSAVINGANALYSIS" | Indicates that the MDA type for the Energy saving analysis defined in clause 8.4.4.1 |
| "MOBILITYMANAGEMENTANALYTICS_MOBILITYPERFORMANCEANALYSIS" | Indicates that the MDA type for the Mobility performance analysis defined in clause 8.4.5.1 |
| "MOBILITYMANAGEMENTANALYTICS_HANDOVEROPTIMIZATION" | Indicates that the MDA type for the Handover Optimization analysis defined in clause 8.4.5.2 |
| "MAINTENANCE_MAINTENANCEANALYTICS" | Indicates that the MDA type for the Maintenance.MaintenanceAnalytics defined in clause 8.4.6.1 |
| "RESOURCEANALYTICS_VIRTUALIZEDRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Virtualized resource utilization analysis defined in clause 8.4.7.1.1 |
| "RESOURCEANALYTICS_PHYISCALRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Physical resource utilization analysis defined in clause 8.4.7.1.2 |
| "RESOURCEANALYTICS_5GCCONTROLPLANECONGESTIONANALYSIS" | Indicates that the MDA type for the 5GC Control plane congestion analysis defined in clause 8.4.7.1.3 |
| "PREDICTIONS_PMDATA" | Indicates that the MDA type for the MDA assisted PM predictions defined in clause 8.4.8.1 |

2nd Change

*** OpenAPI/TS28104_MdaNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MDA NRM

version: 18.5.0

description: >-

OAS 3.0.1 specification of the MDA NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.104; MDA

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.104/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MDAType:

type: string

enum:

- COVERAGEANALYTICS_COVERAGEPROBLEMANALYSIS

- COVERAGEANALYTICS_PAGINGOPTIMIZATION

- SLSANALYSIS_SERVICEEXPERIENCEANALYSIS

- SLSANALYSIS_NETWORKSLICETHROUGHPUTANALYSIS

- SLSANALYSIS_NETWORKSLICETRAFFICANALYSIS

- SLSANALYSIS_E2ELATENCYANALYSIS

- SLSANALYSIS_NETWORKSLICELOADANALYSIS

- MDAASSISTEDFAULTMANAGEMENT_FAILUREPREDICTION

- MDAASSISTEDENERGYSAVING_ENERGYSAVINGANALYSIS

- MOBILITYMANAGEMENTANALYTICS_MOBILITYPERFORMANCEANALYSIS

- MOBILITYMANAGEMENTANALYTICS_HANDOVEROPTIMIZATION

- MAINTENANCE_MAINTENANCEANALYTICS

- RESOURCEANALYTICS_VIRTUALIZEDRESOURCEUTILIZATIONANALYSISNF

- RESOURCEANALYTICS_PHYISCALRESOURCEUTILIZATIONANALYSISNF

- RESOURCEANALYTICS_5GCCONTROLPLANECONGESTIONANALYSIS

- PREDICTIONS_PMDATA

MDATypes:

type: array

items:

$ref: '#/components/schemas/MDAType'

MDAOutputs:

type: array

items:

$ref: '#/components/schemas/MDAOutputPerMDAType'

AnalyticsScopeType:

oneOf:

- type: object

properties:

managedEntitiesScope:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- type: object

properties:

areaScope:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

MDAOutputPerMDAType:

type: object

properties:

mDAType:

type: string

mDAOutputIEFilters:

type: array

items:

$ref: '#/components/schemas/MDAOutputIEFilter'

MDAOutputIEFilter:

type: object

properties:

mDAOutputIEName:

type: string

filterValue:

type: string

threshold:

$ref: '#/components/schemas/ThresholdInfo'

analyticsPeriod:

$ref: '#/components/schemas/AnalyticsSchedule'

timeOut:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

ReportingMethod:

type: string

enum:

- FILE

- STREAMING

- NOTIFICATION

ReportingTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

AnalyticsSchedule:

oneOf:

- type: object

properties:

timeDurations:

type: array

items:

$ref: 'TS28104_MdaReport.yaml#/components/schemas/TimeWindow'

- type: object

properties:

granularityPeriod:

type: integer

ThresholdInfo:

type: object

properties:

monitoredMDAOutputIE:

type: string

thresholdDirection:

type: string

enum:

- UP

- DOWN

- UP_AND_DOWN

thresholdValue:

oneOf:

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

hysteresis:

oneOf:

- type: integer

minimum: 0

- type: number

format: float

minimum: 0

#-------- Definition of types for name-containments ------

SubNetwork-ncO-MdaNrm:

type: object

properties:

MDAFunction:

$ref: '#/components/schemas/MDAFunction-Multiple'

MDAReport:

$ref: '#/components/schemas/MDAReport-Multiple'

ManagedElement-ncO-MdaNrm:

type: object

properties:

MDAFunction:

$ref: '#/components/schemas/MDAFunction-Multiple'

#-------- Definition of abstract IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

MDAFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

supportedMDACapabilities:

$ref: '#/components/schemas/MDATypes'

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

aIMLInferenceFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

MDARequest:

$ref: '#/components/schemas/MDARequest-Multiple'

MDAReport:

$ref: '#/components/schemas/MDAReport-Multiple'

MDARequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

requestedMDAOutputs:

$ref: '#/components/schemas/MDAOutputs'

reportingMethod:

$ref: '#/components/schemas/ReportingMethod'

reportingTarget:

$ref: '#/components/schemas/ReportingTarget'

analyticsScope:

$ref: '#/components/schemas/AnalyticsScopeType'

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

recommendationFilter:

$ref: '#/components/schemas/AnalyticsScopeType'

MDAReport-Single:

$ref: 'TS28104_MdaReport.yaml#/components/schemas/MDAReport'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

MDAFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/MDAFunction-Single'

MDARequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MDARequest-Single'

MDAReport-Multiple:

type: array

items:

$ref: '#/components/schemas/MDAReport-Single'

#-------- Definitions in TS 28.104 for TS 28.532 ---------------------------------

resources-mdaNrm:

oneOf:

- $ref: '#/components/schemas/MDAFunction-Single'

- $ref: '#/components/schemas/MDARequest-Single'

- $ref: '#/components/schemas/MDAReport-Single'

<CODE ENDS>

End of Changes