3GPP TSG-SA5 Meeting #158	S5-246562

Orlando, USA, 18 - 22 November 2024

# Source:	Huawei

# Title:	Conclusion and recommendation for multi-dimensional energy efficiency metrics

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.880 v1.1.0: Study on energy efficiency and energy saving aspects of 5G networks and services

# 3	Rationale

This contribution proposes to add conclusion and recommendation for multi-dimensional energy efficiency metrics in TR 28.880 [1].

# 4	Detailed proposal

This document proposes the following changes in TR 28.880 [1].

1st Change

## 6.6	Multi-dimensional network energy efficiency metrics

The use case, requirements and solutions for multi-dimensional network energy efficiency are described in clause 5.6. It is recommended to add the use case and requirements in TS 28.310 [12] and take the solutions as baseline for normative work.

It is recommended to define multi-dimensional EE metrics as per the following categories:

-	Network data traffic, which measures the number or amount of network traffic provided.

-	For example, the amount of data transferred as the Data Volume used in EE KPI, the number of connections supported, etc.

-	As proposed by potential solution #1, no need to introduce new EE KPIs for this category in normative work.

-	Network quality, which measures the quality of network provided.

-	Such as user throughput, coverage quality, or connection quality, etc.

-	New EE KPIs, as proposed by potential solution #3, need to be introduced for this category in normative work.

-	Network availability, which measures the availability of network.

-		New EE KPIs, as proposed by potential solution #2, need to be introduced for this category in normative work.

End of change