3GPP TSG-SA5 Meeting #158	S5-246564

Orlando, USA, 18 - 22 November 2024

# Source:	Huawei

# Title:	Update on exposure of management services through the CAPIF framework

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.879 v1.1.0: Study on OAM for service management and exposure to external consumers

# 3	Rationale

This contribution proposes to update the use case exposure of management services through the CAPIF framework in TR 28.879 [1].

# 4	Detailed proposal

This document proposes the following changes in TR 28.879 [1].

1st Change

# 5	Use Cases

## 5.1	Exposure of management services

### 5.1.0	Exposure of management services through the CAPIF framework

#### 5.1.0.1	Description

To expose management services through the CAPIF framework, the 3GPP management system should define the functional entity to provide the API provider domain functions (i.e., API Exposing Function (AEF), API management function (AMF), API publishing function (APF)) as defined by the CAPIF framework in clause 6.3 of TS 23.222[5].

For the present document, the functional entity is called the exposure governance management function (EGMF). The EGMF can consume CCF APIs (i.e., CAPIF-3, CAPIF-4 and CAPIF-5 interfaces) and allow API invokers to access management services through service APIs (using CAPIF-2/2e interface).

Figure 5.1.0-1: EGMF implementing the API provider domain functions as defined by the CAPIF framework.

The EGMF implements the CAPIF API provider domain functions, and the following can apply for this functional entity:

-

-	The EGMF is a management function (MnF) as defined in TS 28.533 [2] with the role of management service exposure governance (i.e. abstraction, simplification, filtering, etc.) because:

-	The EGMF requires implementing CAPIF interfaces, which are not defined in the 3GPP management system.

-

-	According to the CAPIF architecture, CAPIF-2 and CAPIF-2e consist of framework aspects and service specific aspects. The service specific aspects are out of scope of CAPIF. The EGMF implements the service specific aspect compliant with the CAPIF architecture. 3GPP SA5 MnS service interface can implement the service specific aspects of CAPIF-2 and CAPIF-2e, and can provide the service APIs exposed by the EGMF (AEF) to the AF (API invoker).

-	The EGMF is not an MnS producer as defined in TS 28.533 [2].

-	The possible deployment options of the EGMF will be agreed at the normative phase.

-	The EGMF should be able to expose one or more management services.

-	Whether the EGMF needs to implement the required mappings between SA5 and SA6 data types will be discussed in normative phase.

#### 5.1.0.2	Potential requirements

PREQ-FS_MExpo-EGMF-01: The 3GPP management system should support the capability to expose management services to an external MnS consumer through the CAPIF framework.

PREQ-FS_MExpo-EGMF-02: The 3GPP management system should support the capability to implement the CAPIF API provider domain functions.

End of change