# 3GPP TSG-SA5 Meeting #158	S5-246786

# Orlando, USA, 18 - 22 November 2024

# Source:	China Mobile

# Title:	TR 28.880 Add solution for renewable energy enabling NF re-selection

# Document for:	Approval

Agenda Item:	6.19.20

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP draft TR 28.880: “Management and orchestration; Study on energy efficiency and energy saving aspects of 5G networks and services v1.1.0”.

3	Rationale

This contribution proposes to add solution for renewable energy enabling 5GC NF re-selection for TR 28.880.

4	Detailed proposal

First Change

5.9	Use case #9: Renewable energy enabling 5GC NF re-selection

5.9.1	Description

To cut down on emissions and increase network efficiency, operators have an interest in powering their network using renewable energy sources. It is also important for operators to understand and track the proportion of energy consumed in their networks that is sourced from renewable sources, which can be utilized by the operators to decide to deploy 5GC NFs which can be powered by renewable energy, such as optimizing the energy utilization of UPFs, which powered by either renewable or non-renewable energy sources.

By incorporating UPFs that utilize renewable energy sources, the 3GPP management system can manage and adjust the working mode of the UPFs (e.g. turn on or turn off), which are powered by either renewable or non-renewable energy sources. This can allow the 5GC to utilize renewable energy for optimization.

5.9.2	Potential requirements

REQ-5GC renewable_energy-01: The 3GPP management system should be able to report the renewable energy consumption information of a 5GC NFs to an authorized MnS consumer.

5.9.3	Potential solutions

As described in TS 28.310 [2], the 3GPP management system can manage and adjust the working mode of the UPFs (e.g. turn on or turn off). When one of these UPFs is in energy saving state it may need candidate network elements or network functions to pick up the load. For example, during off-peak traffic periods, one or more edge UPFs in energySaving state should not cause undue load on the other UPFs, all remaining traffic on those edge UPFs is expected to be redirected to other UPFs before the edge UPFs move into energySaving state.

This solution can be based on the existing energy-saving adjustment modes of the UPF, incorporating the consideration of whether the UPF is powered by renewable energy. Therefore, by incorporating NFs that utilize renewable energy sources and support REQ-5GC renewable_energy-01, an new attribute named “RenewableType” needs to be introduced in UPF IOC to indicate weather UPFs are powered by renewable energy and which types of renewable energy sources the UPF utilizes. This indicator can be used for managing the configuration of the UPF and for adjusting service traffic between  network functions and/or network elements.

5.9.4	Evaluation of potential solutions

The solution described in clause 5.9.3 provides enhancements for the IOC of UPFFunction to allow the MnS consumer to express indications of whether UPFs are powered by renewable energy and which types of renewable energy sources the UPF utilizes. Based on this information, the 3GPP management system can manage and adjust the working modes of the UPFs, which powered by either renewable or non-renewable energy sources. Therefore, the solution described in clause 5.9.3 is a feasible solution.

End of Changes