3GPP TSG-SA5 Meeting #158	S5-247098d2

Orlando, USA, 18 - 22 November 2024

- Source:	Huawei, Ericsson, ZTE, China Mobile, AsiaInfo, China Unicom, Deutsche Telekom, China Telecom Corporation, Telecom italia, NEC

- Title:	Rel-19 pCR TR 28.858 enhance overview with support the NG-RAN and 5GC

- Document for:	Approval

Agenda Item:	6.19.1

- 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

- 2		References

[1]		TR 28.858 " Study on Artificial Intelligence / Machine Learning (AI/ML)" management Phase 2.

[2]		TR 38.743 Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[3]		TS 28.105 Artificial Intelligence / Machine Learning (AI/ML) management

- 3	Rationale

For RAN, In TS 28.105 [3], the Managing NG-RAN AI/ML-based use case defined in clause 6.5.4.2 including Managing NG-RAN AI/ML-based distributed Network Energy Saving, Managing NG-RAN AI/ML-based distributed Mobility Optimization and Managing NG-RAN AI/ML-based distributed Load Balancing. These three use cases have independent requirements.

TR 38.743 [2] defines the use case of AI/ML based Coverage and Capacity Optimization and AI/ML based Network Slicing, which states that:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU

For CN, TS 28.105 [1] define the AI/ML functionalities management scenarios are described to support managing where the ML model training and AI/ML inference are both located in CN in clause 4a.2 as following, and it states that for NWDAF, the ML training function can be located in the MTLF of the NWDAF or the management system, the AI/ML inference function is in the AnLF.

- According to these definitions in TS 28.541, it can be seen that the MnS consumer can manage and configure the NWDAF function and AnLF function related with ML model and inference. We need to reflect this use case in the TS 28.105 to make it clear.

Therefore, we propose to enhance the overview with Managing NG-RAN AI/ML-based use case for RAN.

- 4	Detailed proposal

Start of First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.105: Artificial Intelligence / Machine Learning (AI/ML) management".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[5]	3GPP TR 38.843; Study on Artificial Intelligence (AI)/Machine Learning (ML) 
for NR air interface.

[6]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[x]	3GPP TR 38.743: " Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN ".

[y]	3GPP TS 38.300: " NR and NG-RAN Overall Description;".

[z]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support; network data analytics services".

Next change

# 4	Concepts and overview

## 4.1	Overview

AI/ML techniques have gained significant interests and advancements across various industries, including telecommunications, where they are now being applied to enhance mobile networks. While AI/ML technologies have matured considerably, certain aspects continue to evolve, and new techniques frequently emerge to complement existing methods.

AI/ML techniques can be characterized from several perspectives:

- Learning Methods: AI/ML employs various learning methods such as supervised, semi-supervised, unsupervised, and reinforcement learning. Each method is suited to specific inference categories (e.g., prediction) and requires distinct types of training data.

- Learning Complexity: AI/ML techniques vary in complexity, ranging from basic Machine Learning to more advanced e.g., Deep Learning with neural networks.

- Learning Architecture: Depending on the topology and location of learning tasks, AI/ML can be categorized into Generative AI, centralized, distributed, and federated learning.

- Learning Continuity: AI/ML can be implemented as online learning, offline learning or continual learning, depending on whether the learning process is static or ongoing.

In the 5GS, AI/ML capabilities are used across various domains, including management and orchestration, 5GC and NG-RAN. The performance of AI/ML inference function within 5GS depends on how well-trained ML models for accurate and efficient inference.

To effectively deploy AI/ML capabilities in 5GS, it is crucial to manage the ML models and AI/ML inference functions according to the specific characteristics and requirements of the use cases they support.

The study investigates enhancements of AI/ML management capabilities by building on the work from Rel-18 and exploring new management aspects across various 3GPP working groups. Additional advanced use cases are investigated, covering AI/ML management and operational capabilities to support different types of AI/ML technologies required for AI/ML in the 5G system (e.g. Federated Learning, Reinforcement Learning, online/offline learning, Generative AI and Distributed Learning):

Some primary aspects of AI/ML sustainability are studied, including the evaluation of energy and resource consumption and efficiency impacts associated with AI/ML features across all operational phases (training, emulation, deployment, inference).

The present TR also investigates management aspects (LCM CM and PM) of additional AI/ML functionalities defined by 3GPP SA WGs and RAN WGs.

## 4.X	Management of AI/ML capabilities for RAN and 5GC

### 4.X.1	Management of ML model training and AI/ML inference function for RAN

#### 4.X.1.1	Managing NG-RAN AI/ML based Coverage and Capacity Optimization

In TS 28.105 [2], the existing use cases (AI/ML inference capability configuration management in clause 6.5.4) are described to support managing NG-RAN AI/ML-based related use case defined in TS 38.300 [y].

TR 38.743 [x] defines the use case of AI/ML based distributed Coverage and Capacity Optimization. The MnS producer for AI/ML inference management needs to provide a capability for configuration of the AI/ML based distributed Coverage and Capacity Optimization inference function.

An NG-RAN AI/ML-based distributed Coverage and Capacity Optimization capability may use one or more ML models or AI/ML Inference Functions to derive CCO recommendations.

The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML Iinference Ffunctions related to an AI/ML-based Coverage and Capacity Optimization function. The activation and deactivation actions for AI/ML Iinference Ffunctions related to an AI/ML-based Coverage and Capacity Optimization conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

#### 4.X.1.2	Managing NG-RAN AI/ML-based Network Slicing

In TS 28.105 [2], the existing use cases (AI/ML inference capability configuration management in clause 6.5.4) are described to support managing NG-RAN AI/ML-based related use case defined in TS 38.300 [y].

TR 38.743 [x] defines the use case of AI/ML based Network Slicing. The MnS producer for AI/ML inference management needs to provide a capability for configuration of the AI/ML based Network Slicing inference function.

An NG-RAN AI/ML-based distributed Network Slicing function may use one or more ML models or AI/ML Iinference Ffunctions to derive slice optimization recommendations.

The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML Iinference Ffunction related to an AI/ML-based Network Slicing function. The activation and deactivation actions for AI/ML Iinference Ffunction related to an AI/ML-based Network Slicing conducted by the MnS producer may also be triggered by defined policies provided by the consumer.

### 4.X.2	Management of ML model training and AI/ML inference function for 5GC

In TS 28.105 [2], the existing AI/ML functionalities management scenarios (relation with managed AI/ML features) (clause 4a.2) AI/ML functionalities management scenarios have captured the case where the ML model training and AI/ML inference functions are both located in 5GC.

In case of managing ML model training for NWDAF, an NWDAF may use one or more ML models to derive analytics results, the MnS consumer needs to manage the ML model training (e.g. federated learning) supported by the ML training function (i.e., MTLF) in the NWDAF.

In case of managing AI/ML inference for NWDAF, for monitoring whether AI/ML capability is enabled or not, the MnS consumer can check the value of activation status  of an AnLF insidewithin NWDAF in case of AI/ML capability is supported by AnLF.

End of First change