# 3GPP TSG SA WG5 Meeting #158	S5-246994

# Orlando, Florida, USA 18 - 22 November 2024

# Source:	Huawei

# Title:	New charging solution for DC application usage by volume

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to approve the follow update in TR 28.851.

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR proposes new solution for topic 2 on IMS DC download and usage charging in TR 28.851 [1].

The first solution proposes to charge the application usage by the volume consumed durging the whole IMS DC session, which may include one or more application DC.

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851.

First change

5.2.4.y	Solution #y: DC application usage charging by data volume per IMS session

5.2.4.y.1	General

This solution #y resolves the key issues #1a and #1b for REQ_NG_RTC_CH-DCAPP-02. The SMF charging specified in TS 32.255 [10] can support the volume based charging for IMS services, as specified in clause 5.1.18, on a per IMS DC session basis.

5.2.4.y.2	Description

The caller and callee information are already supported in the PDU session charging information, which makes it possible to correlate the PDU session with IMS service. The volume is counted thoughout the whole IMS DC session, which may include more than one application DC.

End of changes