3GPP TSG-SA5 Meeting #158	S5-247149

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.130 | CR | 0036 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.130 Add requirements for Management of Indirect of Network Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed in S5-246289, WT-2.1 Add descriptions to support management of Indirect Network Sharing is agreed. According to TS 22.261, Indirect Network Sharing is defined as follow: "a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator’s core network is routed through the Hosting NG-RAN Operator’s core network ". Therefore, Indirect Network Sharing is proposed to share NG-RAN indirectly through core network. Regarding 5G MOCN, NG-RAN is shared directly by the connection to each POP’s core network. According to the definition of S-RAN in TS 32.130, shared NG-RAN in the Indirect Network Sharing scenario might be identified as S-RAN. As a result, each POP in the Indirect Network Sharing scenario could have requirements to manage S-RAN. The hosting operator, as MOP in the Indirect Network Sharing scenario, should have capabilities to allow authorized consumers to manage S-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add requirements for Management of Indirect Network Sharing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 3GPP management system will lack the capabilities to manage the Indirect Network Sharing scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.1	Requirements for the OAM&P of shared RAN

REQ-NS_GEN-CON-1		The 3GPP management system of the MOP shall support a capability allowing its authorized consumer  to manage S-RAN of any of the following radio access technologies:

i/ GERAN;

ii/ UTRAN;

iii/ E-UTRAN;

iV/ NG-RAN.

REQ-NS_GEN-CON-2		The 3GPP management system of the MOP shall support a capability allowing its authorized consumer to manage S-RAN according to any of the following scenarios:

i/ Multi-Operator Core Network

ii/ Gateway Core Network

iii/ Indirect Network Sharing.

REQ-NS_GEN-CON-3		The  3GPP management system of the MOP shall support a capability allowing its authorized consumer to know which POPs the shared resources belong to.

REQ-NS_GEN-CON-4		The  3GPP management system of the MOP shall support a capability allowing its authorized consumer to configure which POPs share each cell.

REQ-NS_GEN-CON-5		Any POP shall be able to activate a signaling-based trace / MDT session on its subscribers, whether they are attached to the POP own RAN or to the S-RAN, provided:

a/ this is compliant with the RAN sharing contract; and

b/ user consent for participation in MDT is respected.

REQ-NS_GEN-CON-6		The MOP shall be able, on behalf of any POP, to activate an area-based trace / MDT session, on the portion of the S-RAN that the POP shares and on the POP subscribers only, from the 3GPP management system of MOP, provided

a/ only the POP related subscriber data are collected;

b/ this is compliant with the RAN sharing contract; and

c/ user consent for participation in MDT is respected.

REQ-NS_GEN-CON-7		The MOP shall be able, on behalf of multiple POPs, to activate an area-based trace / MDT session, on the portion of the S-RAN that each POP shares and on each POP subscribers only, from the 3GPP management system of MOP, provided:

a/ each POP has access only to its subscriber data (i.e. not to other POPs' subscriber data);

b/ this is compliant with the RAN sharing contract; and

c/ user consent for participation in MDT is respected.

End of Changes