3GPP TSG-SA WG6 Meeting #64	S6-255225

Orlando (FL), USA, 18th – 22th November 2024	(revision of S6-244290/S6-244620/ S6-244695)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0341 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the 1st EN in 9.3.11.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | (1) There is one EN in cluase 9.3.11.2: Editor's note: Detailed handling for “loss of connectity” in LM server is FFS. For instance, whether to address the case when reference UE and target UE(s) both lose connectivity (e.g., using UE-only operation for SL/Ranging). In case of both Reference UE and target UE lose the connectivity, there is no way to obtain the real-time location information and determines the target UE location status. However, the missing target UE location are still interested by the vertical applications. So, the off-network UE can still utilize the UE-only Operation SL/Ranging mechanism to obtain the location information, cache the location result, and report the result to the LM server when connectivity is back. Then the LM server can still has complete location status of the target UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (1) Add new procedures to clause 9.3.21 about provisioning off-network positioin policy to the UE and reporting cached off-network location result from the UE; (2) update the 9.3.11.2 to capture the case when both reference UE and the target UE lose connectivity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The target UE location status is missing during the reference UE and target UE lose the connectivity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {9.3.2.aa, 9.3.2.bb, 9.3.2.cc}, 9.3.11.2, new {9.3.21.x, 9.3.21.x.1, 9.3.21.x.2} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In Rev3, the figure 9.3.21.x.1-1 and 9.3.21.x.2-1 is modified to align the procedure descriptions. Some editorial changes are made. |  |  |  |  |  |  |  |  |

# /******************************* First Changes *****************************/

9.3.2.aa	Off-network location positioning configuration request

Table 9.3.2.aa-1 describes the information flow from the LM server to the LM client for location area monitoring subscription request.

Table 9.3.2.aa-1: Off-network location positioning configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| VAL service ID | O | Identity of the VAL service for which the location is requested (e.g.Geofencing). |
| Target UE identity(s) | M | Indicate the Identity of the VAL user or identity of the VAL UE whose history location is requested |
| Requested location type | M | Indicates the location information type (e.g., absolute location or relative location) |
| Location positioning triggers | M | Indicates the event (e.g., to initiate the off-network positioning |
| History location report triggers | M | Indicates the event (e.g., Uu connectivity is back) to report the history location result |
| Location QoS | O | Definition of the location Quality of Service for which the location information is requested |
| Requested positioning method | O | Indicates the positioning method for which the location information is requested, e.g., SL/Ranging. |
| Time duration | O | Indicates the time period when the off-network positioning and history location report is expected. |

9.3.2.bb	Off-network location positioning configuration response

Table 9.3.2.bb-1 describes the information flow from the LM client to the LM server for Off-network location positioning configuration response.

Table 9.3.2.bb-1: Off-network location positioning configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |
| Subscription identity | O (see NOTE) | If subscription is successful, identifies the subscription |
| NOTE: This IE shall be present if subscription status is set to success. |  |  |

9.3.2.cc	Off-network history location result report

Table 9.3.2.16-1 describes the information flow from the LM client to the LM server for Off-network history location result report.

Table 9.3.2.cc-1: Off-network history location result report

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription |
| UE location history information | M | Indicates the history location of the requested VAL UE/VAL user. |
| Timestamp | O | Timestamp of the UE location report |

# /******************************* Second Changes *****************************/

9.3.11.2	Monitoring Location Deviation procedure

Figure 9.3.11.2-1 describes the procedure for monitoring the VAL UE's location in a given area of interest.

Pre-condition:

-	The LM server authorized to consume the 3GPP core network service (Monitoring events provided by NEF as specified in 3GPP TS 23.502 [11]).

Figure 9.3.11.2-1: Monitoring VAL UE's location at a given location

1.	The VAL server sends Monitor Location Subscription Request to LM server including target VAL UE Identifier(s), predetermined or dynamic area of interest information, notification interval and notification URI where the VAL server intends to receive the notifications from LM server regarding target VAL UE's presence in a given area, interested notification information, dynamic geofencing conditions.

-	"Area of interest" is the location information, which the VAL server wishes to monitor the target VAL UE's location adherence. This parameter can include an area of interest information and other relevant parameters. In the case of dynamic area of interest, it includes a reference VAL UE Identifier and its proximity range.

-	"Notify_Interval" represents the periodic interval in which the LM server needs to notify target VAL UE's location information to the VAL server. When the target VAL UE moves away from the "Area of interest", then the LM server ignores the "Notify_Interval" and sends the location notification to the VAL server immediately.

-	"dynamic geofencing conditions" includes a temporal condition(s) (e.g. time of day and day of week), connectivity condition(s) (e.g. operator information, roaming status, access type) and/or a spatial condition(s) (e.g. location area(s)) for the reference VAL UE. For instance, the monitoring is valid when a reference UE is served by operator A without using satellite in location area X from 7:00am to 9:00am on working days.

2.	If dynamic area of interest information and dynamic geofencing conditions are received, the LM server requests 3GPP CN (e.g. NEF service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [11]) to monitor the reference UE location change, access type change and roaming status change, in order to determine spatial dynamicity in area of interest related to the target UE(s). If spatial condition is received, the LM server also requests 3GPP CN (i.e. NEF service for area of interest monitoring as specified in 3GPP TS 23.502 [11]) to know whether the reference UE is in the VAL server's predetermined area (e.g. PLMN x). The LM server will process the monitoring considering the received dynamic geofencing conditions for the reference UE.

3.	If dynamic area of interest information is received, the LM server requests 3GPP CN (i.e. NEF service for loss of connectivity as specified in 3GPP TS 23.502 [11]) for both target UE(s) and reference UE.

4a.	LM server processes the Area of interest information in the request, and then subscribes to target UE location monitoring as specified in 3GPP TS 23.502 [11] with appropriate parameters mapping. Based on the subscription, the LM server receives the target VAL UE location information periodically from the 3GPP core network. The LM server also invokes NEF service for Area of Interest monitoring as specified in clause 5.3.3 of 3GPP TS 23.256 [52] to know whether the target VAL UE is in the VAL server's predetermined or dynamic area of interest.

4b.	LM server shall use the Location information procedures as specified in clause 9.3.7 and clause 9.3.10, to periodically obtain the target VAL UE location information. Based on the geographic information from the VAL server, the LM server may determine to additionally include the positioning methods in SEAL LMS procedures to obtain location information.

4c.	LMS may periodically obtain the target VAL UE location information from the 3rd party location management server based on the geographic information from the VAL server.

4e.	LM server may configure the off-network position and report policy to the target VAL UE and/or reference VAL UE as described in clause 9.3.21.x.1 when they still have connectivity. This is to initiate the UE-only Operation SL/Ranging as described in 3GPP TS 23.586 [57], cache the location,  report the location result when the connectivity is back. Time duration is included if the Dynamic geofencing conditions is specified in step 1.

NOTE 1:	How the LMS obtains the target UE location from the 3rd party location management server is out of scope of this specification.

NOTE 2:	Step 4a to 4c are performed for each target VAL UE if multiple target VAL UE Identifiers are provided by the VAL server.

5.	LM server, after successful subscription according to steps 2~4, sends Monitor Location Subscription response, indicating that the LM server accepts VAL server's request and will monitor the location of the target VAL UE(s) to verify if the target VAL UE(s) is in the area of interest.

6a and 6b. LM server processes the location information received from SEAL Location Information procedures, the 3GPP core network and/or the 3rd party location management server, and validates the information. If the location information is matching, then the LM server shall check if the VAL UE's current location is within the predetermined or dynamic area of interest received in step 1.

NOTE 3:	The location information received from the 3GPP core network and from LM client does not necessarily be exactly the same, the LM server can aggregate the location information if there is no big deviation.

6c.	LM server verifies its determined target VAL UE location status (i.e. presence or absence with regard to the predetermined area of interest) with the retrieved 3GPP core network result from the NEF monitoring service for Area of Interest. The LM server will continue with step 7, step 8 and step 9 as applicable.

7.	If the location information received from Location management client, the 3GPP core network and/or the 3rd party location management server do not match or the NEF reported Area of Interest monitoring result does not match with the determined target VAL UE location status (present/absence) in LM server, then the LM server shall consider the target VAL UE as outside from its specified area of interest and shall notify ("Notify Mismatch Location" message) the VAL server of the same, including target VAL UE ID and the location information from LM server, the 3GPP core network and/or the 3rd party location management server in the notification message.

8.	If the target VAL UE's current location is from Location management client, the 3GPP core network and/or the 3rd party location management server matches, and either not in the predetermined area of interest received from VAL server in Monitor Location Subscription Request message or not within the proximity range of the reference UE, and such out of area of interest result is also aligned with the NEF reported area of interest monitoring result, then the LM server considers the target VAL UE as outside from its specified area of interest and shall notify the VAL server that the target VAL UE's current location is outside of area of interest and target VAL UE ID in "Notify Absence" message.

9.	When the target VAL UE's current location is either in the predetermined area of interest or in the proximity range of the reference UE, and such in area of interest result is also aligned with the NEF reported area of interest monitoring result, then the LM server shall notify ("Notify Presence" message) the VAL server periodically, according to the "Notify_Interval" value in "Monitor Location Subscription Request" message, indicating the VAL server that the target VAL UE is within the area of interest, along with target VAL UE's current location information.

10.	If the LM server is notified by the 3GPP CN about loss of connectivity for the reference UE or target UE(s), then the LM server notifies ("Notify Unknown" message with detailed reason) the VAL server indicating that the LM server does not know whether the target UE is "Presence" or “Absence", therefore the dynamic geofencing monitoring for the impacted target UE(s) is suspended.

NOTE 4:	Upon "Notify Unknown", the VAL server can remind end user to check status for the reference/target UE, which is out of 3GPP scope.

Editor's note:	Detailed handling for “loss of connectity” in LM server is FFS. For instance, whether to address the case when reference UE and target UE(s) both lose connectivity (e.g., using UE-only operation for SL/Ranging).

Editor's note:	It is FFS whether to use SL/Ranging mechanism to further figure out the target UE(s) are present or absent in the proximity range of the reference UE when the reference UE (or target UE(s)) lost the connectivity with CN but the target UE(s) (or the reference UE) can still be reached via ProSe.

NOTE 5:	Notifications in step 7-10 can be subject to explicit notification indication from the VAL server, for instance, the VAL server can be interested to know only "Notify Absence" and "Notify Unknown".

11.	If both the target VAL UE and/or reference VAL UE lost the connectivity, the off-network target VAL UE and/or reference VAL UE initiate the UE-only Operation SL/Ranging as described in 3GPP TS 23.586 [57], and cache the location result as configured in step 4e.

12. If the connectivity is back, the target VAL UE and/or reference VAL UE check whether it is still within the valid period per the temporal conditions if configured. If yes, the the target UE and/or reference VAL UE report the cached history location to the LM server as described in clause 9.3.21.x.1. The LM server may use the cached history location e.g., to notify the VAL server about the history target VAL UE location status during the loss of connectivity.

# /******************************* 3rd Changes *****************************/

9.3.21.x	Off-network history location result retrieve

9.3.21.x.1	Off-network location positioning configuration

Figure 9.3.21.x.1-1 illustrates the high level procedure of provisioning the off-network location positioning and report configuration from LM server to the LM client.  This procedure is also used to update the configuration.

Figure 9.3.21.x.1-1: Off-network location positioning configuration

1.	The LM server sends an Off-network location positioning and report configuration request to the location management client to configure the off-network location positioning and report policy to be used later. service in advance. The request includes events/conditions for triggering the location history report, requested location type (i.e., absolute location or relative location), location QoS, the required positioning mechanism (i.e., UE-only Operation SL/Ranging), time duration for the off-networking positioning and report.

NOTE:	Step 1 can be also performed when the LM server requires updating the configuration corresponding to location management client.

2.	The LM client checks whether the VAL server is authorized to configure off-network location positioning and report policy. If the request is approved, the LM client stores the configuration received in step 1.

3.	The LM server sends the Off-network location positioning and report configuration response to the LM server.

9.3.21.x.2	Off-network history location result report

Figure 9.3.21.x.2-1 illustrates the high level procedure of reporting the cached history location result to the LM server.

Pre-condition:

- The location management client is configured with the off-networking location positioning and report policy.

Figure 9.3.21.x.2-1: Off-network history location result report

1.	The Off-network location positioning trigger is met, e.g., the referece VAL UE which hosts the LM client detects it is in off-network and the target VAL UE identified by the target UE identity is also in off-network.

2.	The LM client checks whether it is within the valid period as indicated by the time duration if configured. If yes, the LM client initiates the UE-only Operation SL/Ranging as described in 3GPP TS 23.586 [57], obtains its own location or the target VAL UE location, and caches the location result with time stamp. This step may be performed periodically as configured.

3.	When the connectivity is back and it is within the valid period as indicated by the time duration, the LM client sends the Off-network history location result to the LM server.