3GPP TSG-SA WG6 Meeting #64	S6-245229

Orlando, USA, 18th – 122nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0048 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to application performance analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ADAES |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specific content of the analytics output in Table 8.2.4.7-1 for application performance analytics is unclear. The definition of clear outputs is needed to complete the analytics documentation and avoid implementation problems. This contribution proposes to update the outputs of the application performance analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the IEs in the table for analytics outputs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete output definition for application performance analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.2.4.7	Analytics Notification

Table 8.2.4.7-1 describes information elements for the Analytics Notification from the ADAE server to the VAL server / Consumer.

Table 8.2.4.7-1: Analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “VAL server performance analytics” for procedure in 8.2.2, or “VAL session performance analytics” for procedure in 8.2.3. |
| Analytics Output | M | The analytics outputs, which can be predictive or statistical parameter., which can be: |
| > VAL server performance analytics | O (see NOTE 2) | Statistics or predictions of the VAL server performance. |
| >> VAL server performance indicators | O (see NOTE 2) | Observed/predicted performance and/or functional indicators of the VAL server, such as server RTT, jitter, throughput, PER, server load, number of connections to the server, number or distribution of application client requests issued to the server, etc. |
| >> VAL server performance change | O (see NOTE 2) | Predicted or expected VAL server performance change. |
| >> VAL server performance sustainability | O (see NOTE 2) | Predicted or expected VAL server sustainability. |
| > VAL session performance analytics | O (see NOTE 3) | Statistics or predictions of the VAL session performance. |
| >> VAL session performance indicators | O (see NOTE 3) | Observed/predicted performance and/or functional indicators of the application session, such as session RTT, jitter, throughput, PER, etc. |
| >> VAL session performance change | O (see NOTE 3) | Predicted or expected VAL session performance change. |
| >> VAL session performance sustainability | O (see NOTE 3) | Predicted or expected VAL session sustainability. |
| Confidence level | O (see NOTE 1) | The achieved confidence level. |
| Time horizon | O (see NOTE 1) | The time horizon for predictive analytics. |
| > Start time | O | The start time point of predictive validity. If omitted, the default value is the current time. |
| > End time | M | The end time point of predictive validity. |
| NOTE 1: These information elements shall be provided for the predictive analytics. NOTE 2: These information elements shall be provided if the analytics ID is “VAL server performance analytics”. NOTE 3: These information elements shall be provided if the analytics ID is “VAL session performance analytics”. |  |  |

* * * End of Changes * * * *