3GPP TSG-SA WG6 Meeting #64	S6-245230

Orlando, USA, 18th – 122nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0049 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to edge load analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ADAES |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specific content of the analytics output in Table 8.8.3.7-1 and Table 8.8.3.9-1 for edge load analytics is unclear. The definition of clear outputs is needed to complete the analytics documentation and avoid implementation problems. This contribution proposes to update the outputs of the edge load analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the IEs in the tables for analytics outputs. - Corrected the title and description of Table 8.8.3.9-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete output definition for edge load analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.7, 8.8.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.3.7	Edge analytics Notification

Table 8.8.3.7-1 describes information elements for the Edge analytics Notification from the ADAE server to the VAL server / Consumer.

Table 8.8.3.7-1: Edge analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| Analytics Output | M | The analytics outputs, which can be predictive or statistical parameter. |
| > Applicable time period | O | The time period that the analytics applies to. |
| > Edge platform analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EDN or the associated edge platform, such as the number of active VAL users and/or sessions in the EDN, percentage of resource available in the EDN or the associated edge platform (compute, memory, storage). |
| > EES analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EES, such as the number of EASs being hosted by the EES, percentage of resource available at the EES or the associated edge platform (compute, memory, storage), connection bandwidth per EEC or EAS, response time to service EEC or EAS requests, length of time EES is available to service EEC or EAS requests. |
| > EAS analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EAS, such as the number of ACs served by the EAS, percentage of resource available at the EAS for ACs (compute, memory, storage), connection bandwidth per AC, response time to service AC requests, length of time EAS is available to service AC requests. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| NOTE: At least one of these information elements shall be provided. |  |  |

* * * Next Change * * * *

#### 8.8.3.9	Get analytics data response

Table 8.8.3.9-1 describes information elements for the Get analytics dataresponse from the ADAE server to the consumer.

Table 8.8.3.9-1: Get analytics dataresponse

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics data request (positive or negative acknowledgement). |
| Analytics ID | O | The identifier of the analytics event. |
| Analytics Output | O | The analytics outputs, which can be predictive or statistical parameter. |
| > Applicable time period | O | The time period that the analytics applies to. |
| > Edge platform analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EDN or the associated edge platform, such as the number of active VAL users and/or sessions in the EDN, percentage of resource available in the EDN or the associated edge platform (compute, memory, storage). |
| > EES analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EES, such as the number of EASs being hosted by the EES, percentage of resource available at the EES or the associated edge platform (compute, memory, storage), connection bandwidth per EEC or EAS, response time to service EEC or EAS requests, length of time EES is available to service EEC or EAS requests. |
| > EAS analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EAS, such as the number of ACs served by the EAS, percentage of resource available at the EAS for ACs (compute, memory, storage), connection bandwidth per AC, response time to service AC requests, length of time EAS is available to service AC requests. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| NOTE: At least one of these information elements shall be provided. |  |  |

* * * End of Changes * * * *