3GPP TSG-SA WG6 Meeting #64	S6-245256

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Ericsson

Title:	Solution on UE-deployed API invoker accessing resources not owned by that UE

Spec:	3GPP TR 23.700-22 v1.1.0

Agenda item:	8.7

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This contribution provides a proposal for solution for KI#6 on UE-deployed API invoker accessing resources not owned by that UE.

2. Reason for Change

As described for KI#6, the scope of an API invoker on a UE in Resource owner-aware northbound API access (RNAA) is limited to accessing its own resources only.

However, there are cases in which it is needed to support for API invoker(s) which are deployed on a UE accessing resources of other UEs, e.g., consider the clause 6.2.2 in 5GAA - C-V2X Use Cases and Service Level Requirements Volume I, dealing with Vehicle Health Monitoring in fleet management, in which an Application Client on UE 1 could request access to fetch location and/or vehicle health issues to another user (UE 2).

3. Proposal

It is proposed to agree the new key issue solution for 3GPP TR 23.700-22 V1.1.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |
| Sol #15 | X |  |  |  |  |  |  |
| Sol #16 | X |  |  |  |  |  |  |
| Sol #17 |  |  |  |  | X |  |  |
| Sol #18 | X |  |  |  |  |  |  |
| Sol #19 |  |  |  |  |  | X |  |
| Sol #20 | X |  |  |  |  |  |  |
| Sol #21 | X |  |  |  |  |  |  |
| Sol #22 |  |  |  |  | X |  |  |
| Sol#x |  |  |  |  |  | X |  |

* * * Next Change * * * *

## 6.y	Solution #x: UE-deployed API invoker accessing resources not owned by that UE

### 6.y.1	Solution description

#### 6.y.1.1	General

As per KI#6 description, according to clause 4.17.1 in 3GPP TS 23.222 [2], the scope of an API invoker on a UE in Resource owner-aware northbound API access (RNAA) is limited to accessing its own resources only. This is also acknowledged by SA3 in clause 6.5.3 in 3GPP TS 33.122 [3], in which "only a UE accessing its own resources is considered if the API invoker is on a UE".

However, there are cases in which it is needed to support for API invoker(s) which are deployed on a UE accessing resources of other UEs, e.g., consider the clause 6.2.2 in 5GAA - C-V2X Use Cases and Service Level Requirements Volume I, dealing with Vehicle Health Monitoring in fleet management, in which an Application Client on UE 1 could request access to fetch location and/or vehicle health issues to another user (UE 2).

This solution enhances the CCF Authorization Function to enable UE1-deployed API invoker to access resources owned by other UEs, by binding RO authorization (to access a Service API for a scope and a purpose) to a list of UEs.

#### 6.y.1.2	UE-deployed API invoker accessing resources not owned by that UE

Figure 6.y.1.2-1 presents the procedure for enabling a UE-hosted API invoker accessing network-hosted resources owned by other UEs.

Pre-condition:

- CCF Authorization Function has connectivity and can interact with ROF via CAPIF-8.

- The API invoker in UE2 knows the identifier for UE1.

Figure 6.y.1.2-1: UE-deployed API invoker accessing other UEs’ resources

- The ROF for UE1 stores Resource Owner authorization information for UE1 in CCF. The CCF keeps per Resource Owner identifier, the Resource Owner granted access to a service API for a purpose and scope for a list of UEs (e.g. UE2, UE3, etc.).

NOTE 1:	The detail and procedure for management (e.g. provisioning/revocation) of Resource Owner authorization to the CCF will be specified by SA3.

- The API invoker in UE2 sends an Obtain Service API Authorization request to the CCF for obtaining permission to access the service API by including the API invoker identity information and any information required for authentication of the API invoker. The request shall include the identity for UE1, whose resources are trying to be accessed, and optionally the information about the Purpose and Scope to be performed on the targeted resources (service API), and the identity for UE2.

NOTE 2:	The detail of UE ID(s) being used in the RO authorization information and authorization flow is to be specified in SA3.

- CCF validates and authorizes the API invoker in UE2 as specified in 3GPP TS 33.122 [3].

- CCF retrieves Resource Owner authorization information for UE1 and validates whether UE2 is authorized to access UE1 resources for the specified purpose and scope.

- If Resource Owner authorization needs to be captured because the CCF does not contain Resource Owner authorization for UE1 or the retrieved Resource Owner authorization does not include permissions for UE2, the CCF may trigger the capture of the UE1 authorization information.

NOTE 3:	The details for step 5 will be specified by SA3.

- If the CCF granted to UE2 the Resource Owner authorization for accessing resources of UE1, the authorization information to access the service API is sent to the API invoker in the Obtain service API authorization response (e.g. access granted for API invoker in UE2 to the resources of UE1).

- The service API invocation takes place including the authorization information received in the previous step.

### 6.y.2	Architecture Impacts

This solution is based on the existing architecture of CAPIF as described in 3GPP TS 23.222 [2] and relates to the CAPIF-8 reference point.

### 6.y.3	Corresponding APIs

Whether new or enhanced APIs are required to support this solution is in scope of SA3.

### 6.y.4	Solution evaluation

This solution addresses the following open issue in KI#6:

1.	Whether (and how) RNAA can support the scenario where API invoker(s) which are deployed on the UE can access resources (hosted in the network) of other resource owners (users) (e.g., application client on UE is fetching location of another UE or setting QoS for PDU sessions of another UE)

This solution provides a feasible solution for capturing Resource Owner authorizaiton information from the CCF, which has been previously provided by the ROF of the user, indicating service API access authorization for a certain scope and purpose for a list of UEs.

* * * Next Change * * * *

## 9.7	Evaluation of key issue#6

Clause 5.6 captures the key issue on UE-deployed API invoker accessing resources not owned by that UE. The following open issues are listed:

1.	Whether (and how) RNAA can support the scenario where API invoker(s) which are deployed on the UE can access resources (hosted in the network) of other resource owners (users) (e.g., application client on UE is fetching location of another UE or setting QoS for PDU sessions of another UE)

Solution#19 specified in clause 6.20 provides a proposal to address open issue#1 by enabling API invoker on a UE to access resources of the other UEs in the group which the UE belongs to.

Solution#X specified in clause 6.Y provides a proposal to address open issue#1 by enabling API invoker on a UE to access resources of the other UEs based on the binding of Resource Owner authorization (to access a Service API for a scope and a purpose) to a list of UEs, not necessarily belonging to a group of UEs. The existing mechanism specified in clause 8.31 of 3GPP TS 23.222 [2] is to be supported, extended with the binding of the identification of the UE accessing the resource and the identification of the UE owning the resource.

* * * Next Change * * * *

## 10.8	Conclusions of key issue #6

Solution#19 can be considered as candidate for normative work to address how API invoker on UE can access resources of group of UEs.

Solution#X can be considered as candidate for normative work to address how API invoker on UE can access resources for other UEs, without the limitation of all the UEs belonging to the same group. The existing mechanism specified in clause 8.31 of 3GPP TS 23.222 [2] is to be supported and extended with the binding of the identification of the UE accessing the resource and the identification of the UE owning the resource.

Coordination with SA3 is required to enable the corresponding security solutions.

* * * End of Changes * * *